3GPP TSG-RAN2 Meeting #103bis 	Tdoc 1814775

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | CR0466 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCCH monitoring for when DRX is not configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to section 5.7, the UE shall continuously monitor PDCCH if DRX is not configured, but this does not take measurement gaps into account which prevents the UE from monitoring PDCCH continuously. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A sentence in section 5.7 is modifed to add the condition of measurement gaps to PDCCH monitoring for when DRX is not configured as described in section 5.14 Impact analysis: Impacted functionality: DRX If the UE implements the change but the NW does not, there may be misalignment in what instances the NW expects a UE not configured with DRX to monitor PDCCH. If the NW implements the change but the UE does not, in cases for where measurement gaps are configured and no UE DRX configuration is present, the UE behaviour and requrement w.r.t PDCCH monitoring is unclear and may lead to unnecessary battery drain or measurement performance impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE PDCCH monitoring at the absensce of a DRX configuration is unclear in situations when measurement gaps are configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously taking into account any possible occurrence of measurement gaps as specified in 5.14.

-- subclause parts omitted --

END OF THE CHANGE

UNMODIFIED SECTION BELOW INCLUDED FOR REFERENCE

## 5.14	Handling of measurement gaps

During a measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 as specified in subclause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running:

2>	monitor the PDCCH as specified in subclauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH.