3GPP TSG-RAN WG2 Meeting #103bis	Tdoc R2-1814778

Chengdu, PR China, 8th – 12th October 2018

Agenda Item:	10.3.1.1

Source:	Ericsson

Title:	Removal of description of RRC parameters from MAC

Document for:	Discussion, Decision

# 1	Introduction

In the current MAC specification, 38.321 [1], a number of RRC parameters and their descriptions are listed in various sections. This introduces an unnecessary redundancy and also introduce a risk of having non-aligned descriptions in the RRC specification, 38.331 [2] and [1]. How to solve this is discussed in this contribution and its accompanying CR shows an example of how this could be handled.

# 2	Discussion

In [1], the RRC parameters are typically described in the beginning of a procedure. As an example, the start if the Random Access procedure initialization is shown below.

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS shall be set to a value calculated by multiplying rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE by powerControlOffset as specified in TS 38.214 [6];

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters; “

These parameters are described also in [2]. In the current version of 38.331, some of the parameters, e.g. the description of rsrp-ThresholdCSI-RS, is missing. For the other of the six first MAC parameters, the corresponding RRC descriptions are as follows:

The description of prach-ConfigurationIndex is found in RACH-ConfigGeneric and is given by

prach-ConfigurationIndex

PRACH configuration index. For prach-ConfigurationIndex configured under beamFailureRecovery-Config, the prach-ConfigurationIndex can only correspond to the short preamble format. Corresponds to L1 parameter 'PRACHConfigurationIndex' (see 38.211, section 6.3.3.2).

The description of preambleReceivedTargetPower is found in RACH-ConfigGeneric and is given by

preambleReceivedTargetPower

The target power level at the network receiver side (see 38.213, section 7.4, 38.321, section 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...).

Some parameters are described for different IEs such as rsrp-ThresholdSSB, is described in BeamFailureRecoveryConfig when the Random Access procedure is used for Beam Failure Recovery and in RACH-ConfigCommon for other cases. The descriptions are:

rsrp-ThresholdSSB

L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free Random Access to recover from beam failure. (see 38.213, section 6)

rsrp-ThresholdSSB

UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see 38.213)

The description of rsrp-ThresholdSSB-SUL is found in RACH-ConfigCommon and is given by

rsrp-ThresholdSSB-SUL

The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321, section 5.1.1). The value applies to all the BWPs.

The description of candidateBeamRSList is found in BeamFailureRecoveryConfig and is given by

candidateBeamRSList

A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters. The network configures these reference signals to be within the linked DL BWP (i.e., within the DL BWP with the same bwp-Id) of the UL BWP in which the BeamFailureRecoveryConfig is provided.

As can be seen, the descriptions are similar but not identical and provide partly different information. The question is how much of this information that needs to be captured in [1] since the detailed description including the possible configurable values are found in [2]. Keeping two sets of descriptions in different specifications will no doubt be a source of confusion and errors when the specifications are updated in coming releases. It would therefore be wise to limit the parameter description to only one specification.

Having two sets of descriptions in different specifications will be a source of errors when the specifications are updated in coming releases.

In[1], the procedure description describes how these parameters are used. Furthermore, the RRC parameters are easily recognizable due to the italic font and can be looked up in [2] for detailed description and possible configurable values. We can also note that the internal UE variables are listed but not described in [1] and their use is perfectly clear from the procedure descriptions and the same would apply for the RRC parameters.

The use of the RRC parameters are described in the procedure descriptions.

Based on the above analysis, we believe that it would be sufficient and lead to less inconsistencies between the MAC and RRC specification if the RRC parameters are only described in one of the specifications. Since the RRC specification will describe configurable values for the parameters, we believe that also the descriptions should remain in 38.331. We therefore propose.

Remove the RRC parameter descriptions from 38.321.

There is a corresponding CR in [3].

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Having two sets of descriptions in different specifications will be a source of errors when the specifications are updated in coming releases.

Observation 2	The use of the RRC parameters are described in the procedure descriptions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the RRC parameter descriptions from 38.321.

# 4	References

3GPP TS 38.321 V15.3.0 (2018-09)

3GPP TS 38.331 V15.3.0 (2018-09)

R2-1814779, “Correction for Removal of RRC parameter descriptions”, Ericsson, RAN2#103bis