3GPP TSG-RAN WG2#103bis	Tdoc R2-1814792

# Chengdu, China, 8th – 12th October 2018											 Revision of R2-1811612

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0232 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDCCH monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The action PDCCH monitoring is not very well captured. The UE should monitor PDCCH according to the configuration from RRC, but this is not well described in MAC nor in L1 specification. Additionally, there are some exceptions to this procedure for UEs not supporting full duplex operation which are not covered in the specification. When the UE not caple of full-duplex is configured with flexible TDD operation the rules for priortizing between uplink grants, PUCCH transmissions, SRS transmissions, PDCCH occasions and PRACH transmission are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The general procedure for PDCCH monitoring is corrected. Further mode the excpetions for UEs not supporting full duplex operation are described. The UE not caple of full-duplex skips the PDCCH monitoring occasion if the PDCCH monitoring occasion overlaps with PUSCH transmission, PUCCH transmission, SRS transmission or PRACH transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour for PDCCH monitoring is ambiguous and the behaviour in case a PDCCH monitoring occasion overlaps with a PUSCH transmission, a PUCCH transmission, or SRS transmission is not clear. Impact analysis: Impacted functionality: PDCCH monitoring Interoperability analysis: If the network implements the CR and the UE does not the network may schedule the UE such that PDCCH monitoring occasions, PUCCH, PUSCH, SRS and PRACH transmissions overlap. If the UE implements the CR and the network does not the UE may skip certain PDCCH monitoring occasions which the network expects the UE to read. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5.X	Monitoring of PDCCH

The MAC entity shall monitor the PDCCH occasions according to configuration from upper layers, taking into account any DRX configuration as specified in section 5.7 and any possible occurrence of measurement gaps as specified in 5.14.

For a UE not capable full-duplex operation, the MAC entity shall:

1>	if the MAC entity receives an uplink grant to indicate an uplink transmission on a serving cell and if a PDCCH occasion on the same serving cell overlaps with this uplink transmission in time; or

1>	if the MAC entity has an uplink transmission using a configured grant on a serving cell and if a PDCCH occasion on the same serving cell overlaps with this uplink transmission in time; or

1>	if a PUCCH transmission and a PDCCH occasion on the same serving cell overlap in time; or

1>	if an SRS transmission and a PDCCH occasion on the same serving cell overlap in time; or

1>	if a PRACH transmission and a PDCCH occasion on the same serving cell overlap in time:

2>	skip the PDCCH occasion.