3GPP TSG-RAN WG2#103bis	Tdoc R2-1814806

# Chengdu, PR China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0475 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Data Inactivity timer in MAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The DataInactivityTimer is used to monitor data and control signalling activity and to let the MAC layer inform RRC when sufficient inactivity has been detected. The timer has been introduced in 38.331 but the corresponding introduction has not been done in 38.321. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.x: The corresponding text describing DataInactivityTimer in LTE is reused for NR. Impact analysis: Impacted functionality: Data inactivity monitoring Interoperability analysis: If the network implements the CR and the UE does not the network will assume the UE goes to idle after expiry of the DataInactivityTimer. If the UE implements the CR and the network does not the network will assume the UE is available, even though the UE is not (due to expiry of the DataInactivityTimer). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DataInactivityTimer is not introduced in MAC and can therefore not be used. There are inconsistencies between MAC and RRC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.X	Data inactivity monitoring

The MAC entity may be configured by RRC with a Data inactivity monitoring functionality, when in RRC_CONNECTED. RRC controls Data inactivity operation by configuring the timer DataInactivityTimer.

When DataInactivityTimer is configured, the MAC entity shall:

1>	if the MAC entity receives a MAC SDU for DTCH logical channel, DCCH logical channel, or CCCH logical channel; or

1>	if the MAC entity transmits a MAC SDU for DTCH logical channel, or DCCH logical channel:

2>	start or restart DataInactivityTimer.

1>	if the DataInactivityTimer expires:

2> indicate the expiry of the DataInactivityTimer to upper layers.