3GPP TSG-RAN WG2 #103bis	Tdoc R2-1814813

Chengdu, China, 8th – 12th October 2018

Agenda Item:	11.7.2

Source:	Ericsson

Title:	UE Time Synchronization

Document for:	Discussion

# Introduction

In RAN#81 the revised study item on NR Industrial Internet of Things (NR-IIoT, [1]) has been approved. The objectives of the study include some form of Time Sensitive Networking (TSN) related enhancements including the aspect of delivering accurate reference timing information to a UE through a 5G network, which is discussed in the following.

# Discussion

An NR-IIoT feature of interest is that of providing UE based applications (or applications residing in IIoT devices connected to a UE via ethernet ports) with reference time information derived from a source clock in a network external to a 5G network. The working assumption is that a UE needs to acquire the reference time in the form of a global clock or a local clock (e.g. GPS time or International Atomic Time or a local clock value only applicable to a specific factory, thereby allowing applications residing in diverse physical locations to coordinate the points in time when certain actions are to be performed).

Precision Time Protocol (PTP) is a protocol defined by IEEE 1588-2008 to allow for synchronizing clocks throughout a computer network and can be used to realize the goal of providing a UE with reference time information. On a local area network PTP can be used to realize clock accuracy in the sub-microsecond range, making it suitable for use by NR-IIoT type applications requiring such precision.

There is a need for UEs to be provided with reference time information that is based on a global or local clock e.g. GPS time or International Atomic Time or a local clock value only applicable to a specific factory.

On a local area network PTP can be used to realize clock accuracy in the sub-microsecond range, making it suitable for use by NR-IIoT type applications requiring such precision.

Some candidate methods by which reference time information can be provided to a UE are as follows:

Method 1: PTP/Ethernet based UE Reference Time Establishment at UE. In this method the 5G network supports the transmission of PTP messages (containing reference time information) to a gNB and then over the radio interface to a UE using ethernet PDUs.

Method 2: PTP/Ethernet based Reference Time Establishment at gNB. In this method the 5G network supports the transmission of PTP messages (containing reference time information) to a gNB using ethernet PDUs where a 5G specific method is then used to convey the reference time to a UE (e.g. Method 5 below).

Method 3: PTP/IP based Reference Time Establishment at UE. In this method the 5G network supports the transmission of PTP messages (containing reference time information) to a gNB and then over the radio interface to a UE using IP packets.

Method 4: PTP/IP based Reference Time Establishment at gNB. In this method the 5G network supports the transmission of PTP messages (containing reference time information) to a gNB using IP packets where a 5G specific method is then used to convey the reference time to a UE (e.g. Method 5 below).

Method 5: SIB/RRC Based Reference Time Establishment at UE. In this method the gNB in a 5G network supports the transmission of reference time information to a UE using RRC messages or SIB messages. The means by which the gNB acquires the reference time could be as described for methods 1, 2, 3 and 4 above or could be 5G specific (e.g. gNB has direct access to a GPS receiver).

## Method 1 – PTP/Ethernet based Reference Time Establishment at UE

IEEE 1588 specifies a PTP Boundary clock function wherein PTP capable nodes exchange PTP packets thereby allowing a reference time value to be relayed from one node to another (i.e. each node is able to receive and generate PTP packets). This method considers the case where a gNB has acquired reference time information from a source clock (e.g. GrandMaster clock in a TSN network) using PTP/Ethernet packets and then uses PTP message exchanges to establish reference time information at a UE using ethernet PDUs:

- The gNB exchanges PTP packets with a UE using ethernet PDUs thereby allowing the UE to acquire reference time information and serve as a Boundary Clock node i.e. the UE may perform further PTP message exchanges with IoT devices (using ethernet PDUs) to thereby provide them with reference time information.

- The value of the round-trip delay between two PTP capable nodes is established using PTP message exchanges and requires support for hardware time stamping with sufficiently fine resolution/granularity.

- Sending reference time related PTP messages between two given nodes assumes the UL and DL signalling path delays between them is the same. This imposes very strict requirements if such PTP messages are sent over the radio interface.  Note that this is different from the issue of realizing alignment between SFNs on the UL and DL of radio interface of a 5G network for which sub-microsecond alignment can be expected.

- This method therefore imposes the complexity of hardware time stamping at the gNB and UE in support of sending PTP messages over the radio interface to convey reference time information.

- Using PTP/Ethernet message exchanges between a gNB and a UE for establishing a reference time at the UE is undesirable in light of corresponding hardware complexity concerns. Method 1 is not considered to be desirable.

Using PTP/Ethernet message exchanges between a gNB and a UE for establishing a reference time at the UE is undesirable in light of corresponding hardware complexity concerns. Method 1 is not considered to be desirable.

## Method 2 – PTP/Ethernet based Reference Time Establishment at gNB.

This method considers the case where a gNB has acquired reference time information from a source clock (e.g. GrandMaster clock in a TSN network) using PTP/ethernet packets and then uses a 5G specific mechanism to convey the reference time to a UE.

- A UE that acquires reference time information from the gNB using 5G specific methods may then serve as a Boundary Clock node i.e. the UE may perform further PTP message exchanges with IoT devices (using ethernet PDUs) to thereby provide them with reference time information.

- This method therefore avoids the complexity of hardware time stamping at the gNB and UE required by method 1.

- Using a 5G specific mechanism for establishing a reference time at the UE can be similar to that described for LTE Rel-15 and avoids the complexity of hardware time stamping.

Using a 5G specific mechanism for establishing a reference time at the UE can be similar to that described for LTE Rel-15 and avoids the complexity of hardware time stamping.

The means by which a gNB acquires reference time information from a source clock is not in the scope of the study item described in [1] and is therefore not studied in RAN2.

## Method 3 – PTP/IP based Reference Time Establishment at UE

This method considers the case where a gNB has acquired reference time information from a source clock (e.g. GrandMaster clock in a TSN network) using PTP/IP packets and then uses PTP message exchanges to establish reference time information at a UE using IP packets. Using PTP/IP message exchanges between a gNB and the node managing the source clock is undesirable in light of the potential for asymmetry regarding uplink and downlink IP packet delays between these nodes.  As such method 3 is not further considered.

## Method 4 - PTP/IP based Reference Time Establishment at gNB

This method considers the case where a gNB has acquired reference time information from a source clock (e.g. GrandMaster clock in a TSN network) using PTP/IP packets and then uses a 5G specific mechanism to convey the reference time to a UE.  Using PTP/IP message exchanges between a gNB and the node managing the source clock is undesirable in light of the potential for asymmetry regarding uplink and downlink IP packet delays between these nodes.  As such method 4 is not further considered.

Using PTP/IP message exchanges between a gNB and the node managing the source clock is undesirable in light of the potential for asymmetry regarding uplink and downlink IP packet delays between these nodes.  Methods 3 and 4 are not considered to be desirable.

## Method 5 - SIB/RRC Based Reference Time Establishment at UE

This method assumes the gNB has access to a reference time which it forwards to a UE using an RRC-dedicated or SIB message specific to the 5G radio interface (i.e. PTP messages are not used for this method). This method raises the issue of how to ensure the accuracy of reference time information received by the UE still satisfies the target accuracy level.

- The inaccuracy with which a UE acquires synchronization to SFNs transmitted by a gNB combined with the inaccuracy with which a gNB acquires synchronization to SFNs transmitted by a UE must be limited to a range suitable for factory automation scenarios (e.g. ± .25µs).

- Asymmetry in delays experienced for the UL and DL of the Uu interface must be limited to a range suitable for factory automation scenarios (e.g. ± .25µs range) since reference time establishment procedures applicable to this method assume symmetric delay (i.e. a UE assumes the reference time it receives from a gNB is to be advanced by ½ of the applicable Timing Advance value).

- This method is seen to be very similar to that adopted for LTE Rel-15 wherein the reference time projected to apply when the end of SFNz occurs at the ARP (antenna reference point) is the reference time conveyed at the UE using SIB/RRC signaling. As such, a gNB implementation must be able to accurately determine the time that will elapse between when it acquires reference time information and when the end of SFNz occurs at the ARP.

- Combining these possible sources of inaccuracy will determine if the accuracy of the UE reference time is still acceptable for the most demanding time sensitive factory applications.

- Considering typical multipath delays expected for a factory setting and corresponding TA resolution capabilities for small cell sizes it is expected that the combined inaccuracies introduced in conveying reference time from a gNB to a UE using this method will be less than 1µs.

Considering typical multipath delays expected for a factory setting and corresponding TA resolution capabilities for small cell sizes it is expected that the combined inaccuracies introduced in conveying reference time from a gNB to a UE using this method will be less than 1µs.

Considering all 5 methods above it can be noted that once a UE has received a reference time value it can act as a local clock source and therefore further distribute the time reference value to as many UEs as required using PTP messages sent using ethernet PDUs to ethernet connected IoT devices.

# Conclusion

The following observations have been made:

Observation 1: There is a need for UEs to be provided with reference time information that is based on a global clock (e.g. GPS time or International Atomic Time or a clock value only applicable to a specific factory.

Observation 2: On a local area network PTP can be used to realize clock accuracy in the sub-microsecond range, making it suitable for use by NR-IIoT type applications requiring such precision.

Observation 3: Using PTP/Ethernet message exchanges between a gNB and a UE for establishing a reference time at the UE is undesirable in light of corresponding hardware complexity concerns. Method 1 is not considered to be desirable.

Observation 4: Using a 5G specific mechanism for establishing a reference time at the UE can be similar to that described for LTE Rel-15 and avoids the complexity of hardware time stamping.

Observation 5: The means by which a gNB acquires reference time information from a source clock is not in the scope of the study item described in [1] and is therefore not studied in RAN2.

Observation 6: Using PTP/IP message exchanges between a gNB and the node managing the source clock is undesirable in light of the potential for asymmetry regarding uplink and downlink IP packet delays between these nodes.  Methods 3 and 4 are not considered to be desirable.

Observation 7: Considering typical multipath delays expected for a factory setting and corresponding TA resolution capabilities for small cell sizes it is expected that the combined inaccuracies introduced in conveying reference time from a gNB to a UE using Method 5 will be less than 1µs.

Observation 8: Considering all 5 methods above it can be noted that once a UE has received a reference time value it can act as a local clock source and therefore further distribute the time reference value to as many UEs as required using PTP messages sent using ethernet PDUs to ethernet connected IoT devices.

The simplest method for establishing a reference time value at the UE that also satisfies reference time accuracy targets is seen to be that of using Method 2 for acquiring reference time by a gNB and using Method 5 to convey the reference time value from the gNB to the UE.

Proposal 1	The simplest method for establishing a reference time value at the UE that also satisfies reference time accuracy targets is seen to be that of (a) a gNB first acquiring reference time information (out of scope of RAN2) and (b) using a RRC-dedicated or SIB message specific to the 5G radio interface to convey the reference time information from a gNB to a UE.

# References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018