# 3GPP TSG-RAN WG2 Meeting #103bis														R2-1814833

# Chengdu, China, 8 - 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0385 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC_INACTIVE state handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction for combined RNA and TAU procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction for combined RNA and TAU procedure. The change is applicable to NR standalone (SA) only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB cannot monitor length of RRC inactive connection |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.4, 5.3.13.8, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	restore the PDCP state and reset COUNT value for SRB2 and all DRBs;

2>	restore the cellGroupConfig from the stored UE AS context;

2>	indicate to lower layers that stored UE AS context is used;

1>	discard the fullI-RNTI, shortI-RNTI and the stored UE AS context, except ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

Editor’s Note: FFS Whether it is supported to configure secondaryCellGroup at Resume.

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

Editor’s Note: FFS Whether there needs to be a second radioBearerConfig.

1>	resume SRB2 and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1> stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>  if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2> if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1>	if the variable pendingRnaUpdate is set to 'TRUE':

2>  set the variable pendingRnaUpdate to 'FALSE';

2>  set the resumeCause value set to ‘rna-Update’;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

END OF MODIFIED SUBCLAUSE

#### 5.3.13.8	RNA update

Upon entering RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if upon cell reselection the UE enters an RNA not belonging to the configured ran-NotificationAreaInfo:

2>	if upper layers request resumption of an RRC connection;

3>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set in accordance with the information received from upper layers;

3>	set the variable pendingRnaUpdate to 'TRUE'

2>	else:

3>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set to ‘rna-Update’;

Editor’s Note: FFS How to handle simultaneous NAS triggered events and AS triggered events (except TAU and RNAU, which has been explicitly agreed).

1> if barring is alleviated for Access Category [the standardised RAN specific access category], as specified in 5.3.14.4:

Editor’s Note: Which value to use for the standardised RAN specific access category needs to be confirmed by SA1.

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to 'TRUE':

3> set the variable pendingRnaUpdate to 'FALSE';

3>	initiate RRC connection resume procedure in 5.3.13.2 with cause value set to ‘rna-Update’.

NEW MODIFIED SUBCLAUSE

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList         		OPTIONAL,

resumeCause               				ResumeCause         					OPTIONAL,

lateNonCriticalExtension                OCTET STRING                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                              OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

END OF CHANGES