3GPP TSG-RAN WG2 Meeting #103bis			 R2-1815173

Chengdu, China, 8th - 12th Oct 2018

Agenda Item:	11.1.2 (FS_NR_IAB)

Source: 	LG Electronics Inc.

Title:         	Further consideration on downlink data congestion handling

Document for: 	Discussion and Decision

Introduction

The downlink data congestion problem and related high level solutions were intensively discussed in the email discussion and the related TP was made as in [1] which contains the following sentences.

“The congested IAB node may provide feedback information to the parent IAB node or the IAB donor. Based on this feedback, the parent IAB node or IAB donor may perform flow control and alleviate downlink data congestion.”

This contribution analyses more details of downlink data congestion handling.

Discussion

The figure 1 as an example is given to explain a little more detail operation of downlink data congestion handling. If the link between IAB node 1 and IAB node 2 has a problem and this cause downlink data congestion at the IAB node 2, the IAB node 3 should perform flow control and alleviate downlink data congestion at the IAB node 2.

Figure 1. Example figure to describe downlink data congestion handling.

In this condition, IAB node 2 can transmit all downlink traffics to UE2 without any problems, but all traffics toward the IAB node 1 cannot be transmitted. This means that only IAB node 1 involves in downlink data congestion at the IAB node 2. However, if only IAB node 2 ID with buffer load is provided to the IAB node 3 to indicate the IAB node 2 where data congestion has occurred, the IAB node 3 cannot figure out which IAB node or UE generate downlink data congestion and finally should decrease or may block all downlink traffics to IAB node 2. This must be undesirable downlink congestion handling because even if the IAB node 2 can forward all downlink data traffics to UE 2, the IAB node 3 may block or reduce all downlink traffics toward UE2 unnecessarily.

Thus, if downlink data congestion occurs at the IAB node 2, the feedback information from the IAB node 2 should include the IAB node 1’s ID to make the IAB node 3 reduce only downlink traffics toward the IAB node 1 from the IAB node 2. Of course, if link between UE 2 and IAB node 2 has problem, in this case, feedback information should include UE 2’s ID.

Proposal 1. UE ID or IAB node ID which causes downlink data congestion should be indicated to the parent IAB node by the feedback information.

L2 user plane structure for architecture group 1 was discussed and the TP was agreed as in [2]. There are three DL L2 structure of IAB node with UE bearer to BH RLC-channel mapping:

- Option 1: IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels for end-to-end RLC ARQ

- Option 2: IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels, multiplexing of RLC-channels to logical channels, and end-to-end RLC ARQ

- Option 3: IAB node for many-to-one mapping of UE-bearers to BH RLC-channels

The downlink data congestion handling needs to be considered based on the above three DL L2 structure. In option 1, if the IAB node 3 in figure 1 receives the feedback information from IAB node 2 which contains IAB node 1’s ID, the MAC entity of the IAB node 3 can throttle downlink traffics only toward IAB node 1 because the MAC entity would be able to figure out which logical channels are toward the IAB node 1. RLC entity would be inappropriate due to only segmentation allowed at the RLC layer in this option.

In option 2, all BH RLC channels are multiplexed to a logical channel by the adaptation layer, i.e., it seems to be above MAC option for adaptation layer placement. If the IAB node 3 in figure 1 receives the feedback information from IAB node 2 which contains IAB node 1’s ID, only adaptation layer of the IAB node 3 can figure out which logical channels are toward the IAB node 1.

In option 3, one BH RLC channel may buffer traffics toward both the IAB node 1 and UE 2 in figure 1 because it is many-to-one mapping and same QoS profiles would be multiplexed into the same BH RLC channel at the IAB node 3. Given in same condition, if the RLC or MAC entity performs downlink flow control in this option, RLC or MAC entity cannot block or reduce downlink traffics only toward the IAB node 1. Thus, if the IAB node 3 needs to reduce or block all downlink traffics only toward the IAB node 1, the adaptation layer should perform downlink flow control.

Based on the above explanation, MAC layer should perform downlink flow control in option 1, but adaptation layer should perform downlink flow control in option 2 and 3.

Proposal 2. In DL L2-structure of IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels for end-to-end RLC ARQ, MAC layer should perform downlink flow control.

Proposal 3. In DL L2-structure of IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels, multiplexing of RLC-channels to logical channels, and end-to-end RLC ARQ, adaptation layer should perform downlink flow control.

Proposal 4. In DL L2-structure of IAB node for many-to-one mapping of UE-bearers to BH RLC-channels, adaptation layer should perform downlink flow control.

Proposal

In this contribution, we analysed more detail operation for downlink data congestion handling and proposed below proposals:

Proposal 1. UE ID or IAB node ID which causes downlink data congestion should be indicated to the parent IAB node by the feedback information.

Proposal 2. In DL L2-structure of IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels for end-to-end RLC ARQ, MAC layer should perform downlink flow control.

Proposal 3. In DL L2-structure of IAB-node for 1:1 mapping of UE-bearers to BH RLC-channels, multiplexing of RLC-channels to logical channels, and end-to-end RLC ARQ, adaptation layer should perform downlink flow control.

Proposal 4. In DL L2-structure of IAB node for many-to-one mapping of UE-bearers to BH RLC-channels, adaptation layer should perform downlink flow control.

- References

[1] R2-1813023	TP for IAB Flow Control and Congestion Handling	LG Electronics Inc.

[2] R2-1813454	L2 structure for IAB arch group 1	Qualcomm Inc.