3GPP TSG-RAN WG2#103bis	R2-1815428

Chengdu, China, Oct. 8th – 12th 2018

Agenda Item:	10.4.1.4.2

Source:	NTTDOCOMO

Title:	Measurement gap coordination for NE-DC and NR-DC

Document for:	Discussion, Decision

Introduction

It has been discussed in email discussion in 103#52 MR-DC measurement that how the measurement gap/gap sharing configuration is applied to NE-DC and NR-DC. Since there is no conclusion drawn from the discussion, the following further discussions were summarized by rapporteur.

- Discuss which option is applied to NE-DC measurement gap/gap sharing configuration:

-	Option 1: Measurement gap(s)/gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

-	Option 3: Per-UE gap/gap sharing configuration or per FR2 gap/gap sharing configuration is configured by MN, while per FR1 gap/gap sharing configuration can be configured by MN or SN.

- Discuss which option is applied to NR-DC measurement gap/gap sharing configuration:

-	Option 1: Measurement gap(s) /gap sharing configuration are configured by MN, including per-UE gap, per FR1 gap and per FR2 gap.

-	Option 2: Per-UE gap/gap sharing configuration is configured by MN, per FR1 gap/gap sharing configuration or per FR2 gap/gap sharing configuration can be configured by MN or SN.

Discussion

Measurement gap sharing in NE-DC

For gap/gap sharing configuration in NE-DC, it is easy to agree that per-UE gap and FR2 gap/gap sharing configuration is configured by MN, the controversial point among option1 and option3 is whether per FR1 gap/gap sharing is only configured by MN or optionally configured by MN/SN. For Option3, since SCG can only work with FR1 frequency, by analogizing with EN-DC, it is flexible for SN to decide the FR1 gap configuration. However, differently from EN-DC that only SN operates on FR2 frequency, FR1 frequency will be operated with both MN and SN, thus if FR1 gap can be configured by MN or SN, there exists possibility that MN and SN configure FR1 gap simultaneously and conflict will occur. Therefore, it is necessary to allow one node (MN/SN) to determine the gap configuration.

Observation1:   If MN and SN configure FR1 gap simultaneously, conflict will occur.

Measurement gap sharing in NR-DC

For gap/gap sharing configuration in NR-DC, the controversial point among option1 and option2 is whether per FR1/FR2 gap/gap sharing is only configured by MN or optionally configured by MN/SN. For Option2, there exists merit of flexibility that by knowing the per FR gap/gap sharing configuration from MN, the SN can directly configure gap pattern to the UE via SRB3 without sending gap request to the MN. However, similar with NE-DC, it is necessary to allow one node (MN/SN) to be responsible for determining the gap configuration, otherwise MN and SN possibly configure the FR1/FR2 gap simultaneously and conflict will occur.

Observation2:   Similar with NE-DC, it is necessary to ask one node (MN/SN) to be responsible for determining the gap configuration, otherwise conflict will occur.

Proposal 1	For NE-DC and NR-DC, RAN2 agreee to take the same measurement gap coordnation method.

Proposal 2       For both NE-DC and NR-DC, MN decides all gap patterns for measurement gap coordination

Conclusion

Based on the discussion in section 2 we propose the following:

Observation1:    If MN and SN configure FR1 gap simultaneously, conflict will occur.

Observation2:    Similar with NE-DC, it is necessary to ask one node (MN/SN) to be responsible for determining the gap configuration, otherwise conflict will occur.

Proposal 1	For NE-DC and NR-DC, RAN2 agree to take the same measurement gap coordnation method

Proposal 2       For both NE-DC and NR-DC, MN decides all gap configuraitons for measurement gap coordination

References

Email discussion 103#52 –MR-DC measurement