3GPP TSG-RAN WG2 Meeting #103bis	R2-1815510

Chengdu, China, 8th – 12th October 2018

Agenda Item:

Source: 	ITL

Title: 	Considerations on resource pool in NR Sidelink for V2X

Document for:	Discussion and decision

Introduction

In RAN #80 plenary meeting, NR-V2X SI was agreed [1]. This SI includes one important feature that is NR sidelink for V2X. This contribution would like to discuss resource allocation issues among objectives in the SI.

Discussion

Numerology in resource pool

First of all, the definition of resource pool in numerology point of view should be discussed since NR has multiple numerologies to define time / frequency resources. Simply, we can consider a couple of ways as below:

Option 1: Single numerology in single resource pool (same as LTE)

Option 2: Multiple numerologies in single resource pool

Regarding on option 2, the signalling architecture for resource pool can be simple comparing with option 1. But actual signalling overhead may be increased due to additional information to indicate each numerology.

If we go with option 1, specification issues can be minimized for resource pool definition. Moreover, there is no problem to support multiple numerologies with resource pool list.

To control Sidelink resources for IDLE and inactive UE, separated resource pool in numerology domain would be simpler to change resource pool information, QoS mapping and etc.

Observation 1: There is no strong motivation and use case to introduce multiple numerologies in single TX or RX resource pool.

Regarding above observation, we propose as below.

Proposal 1: Only one numerology should be used to define TX or RX resource pool.

QoS in resource pool

In [2], there is clear description how to define QoS for NR Sidelink for V2X.

## 6.19 Solution #19: QoS Support for eV2X communication over PC5 interface ### 6.19.1 Functional Description #### 6.19.1.1 General description This solution addresses key issue#4 (clause 5.4) Support of PC5 QoS framework enhancement for eV2X. The QoS requirements for eV2X are different from that of the EPS V2X, and the previous defined PPPP/ PPPR in TS 23.285 [5] are considered not to satisfy the needs. Specifically, there are much more QoS parameters to consider for the eV2X services. This solution proposes to use 5QI for eV2X communication over PC5 interface. This allows a unified QoS model for eV2X services over different links. #### 6.19.1.2 Solution description The new service requirements were captured in TS 22.186 [4]. The new performances KPIs were specified with the following parameters: - Payload (Bytes); - Transmission rate (Message/Sec); - Maximum end-to-end latency (ms); - Reliability (%); - Data rate (Mbps); - Minimum required communication range (meters). Note that the same set of service requirements apply to both PC5 based V2X communication and Uu based V2X communication. As analysed in solution #2 (clause 6.2), these QoS characteristics could be well represented with 5QI defined in TS 23.501 [7]. It is therefore possible to have a unified QoS model for PC5 and Uu, i.e. also use 5QIs for V2X communication over PC5, such that the application layer can have a consistent way of indicating QoS requirements regardless of the link used. This does not prevent the AS layer from implementing different mechanisms over PC5 and Uu to achieve the QoS requirements. Considering the 5GS V2X capable UEs, there are three different types of traffic: broadcast, multicast, and unicast. For unicast type of traffic, it is clear that the same QoS Model as that of Uu can be utilized, i.e. each of the unicast link could be treated as a bearer, and QoS flows could be associated with it. All the QoS characteristics defined in 5QI and the additional parameter of data rate could apply. In addition, the Minimum required communication range could be treated as an additional parameter specifically for PC5 use. Similar consideration applies to multicast traffic, as it can be treated as a special case of unicast, i.e. with multiple defined receivers of the traffic. For broadcast traffic, there is no bearer concept. Therefore, each of the message may have different characteristics according to the application requirements. The 5QI should then be used in the similar manner as that of the PPPP/PPPR, i.e. to be tagged with each of the packet. 5QI is able to represent all the characteristics needed for the PC5 broadcast operation, e.g. latency, priority, reliability, etc. A group of V2X broadcast specific 5QIs (i.e. VQIs) could be defined for PC5 use. NOTE: The 5QI used for PC5 may be different from that used for Uu even for the same V2X service, e.g. the PDB for the PC5 can be longer than that for the Uu as it is a direct link.

Observation 2: QoS parameters for NR Sidelink for V2X should be reuse the 5QI in NR for unicast and multicast. For broadcast, VQI should be defined based on 5Qis in NR

Additional QoS values or changes may be discussed in SA2.

The motivation of Mapping QoS and resource pool is to set priorities of data which can be transmitted via each resource pool. Therefore it is obvious that 5QI for unicast and multicast and VQI (or PPPP/PPPR) for broadcast used for resource pool configurations. Details configuration rules will be discussed in WI phase.

Proposal 2: Mapping QoS and resource pool in LTE with new QoS parameters can be baseline for NR (5QI for unicast and multicast / VQI for broadcast)

BWP in resource pool

In NR, BWP was introduced to manage frequency resources on wide system bandwidth for efficiency. It is natural to introduce BWP in NR Sidelink. However, it is one of open issue how to set multiple BWP configuration on a TX or RX resource pool.

If we assume that multiple BWP configuration for a resource pool, flexible resource selection (TX) can be achieved by network. But complicated sensing (TX) / monitoring (RX) operation in UE side would be burden. Moreover, introduction of additional RRC or PHY signaling should be discussed to support multiple BWP. It may cause a big specification impact.

So, we propose:

Proposal 3: It should be considered to use single BWP configuration for each resource pool in NR Sidelink for V2X.

Conclusion

In this contribution, we discussed the initial resource allocation issues on NR Sidelink for V2X. Based on the discussion, we suggest the following observations and proposals.

Observation 1: There is no strong motivation and use case to introduce multiple numerologies in single TX or RX resource pool.

Observation 2: QoS parameters for NR Sidelink for V2X should be reuse the 5QI in NR for unicast and multicast. For broadcast, VQI should be defined based on 5Qis in NR

Proposal 1: Only one numerology should be used to define TX or RX resource pool.

Proposal 2: Mapping QoS and resource pool in LTE can be reused for NR with new QoS parameters (5QI for unicast and multicast / VQI for broadcast)

Proposal 3: It should be considered to use single BWP configuration for each resource pool in NR Sidelink for V2X.

Reference

RP-1801480, New SID on NR V2X

TR 23.786