3GPP TSG-RAN2 Meeting #103bis 	R2-1815566

Chengdu, China, 2018-10-08 to 2018-10-12                              (Revision of R2-1814623)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0374 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Correction to clarify configuration aspects for handover between NR and E-UTRA (38.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the successful completion procedures of mobility from NR, procedures upon going to RRC_IDLE are triggered, which will release current radio resources. However, in case of handover to ng-eNB, delta signaling can be used and thus, the release of radio resources from the source is not needed. In addition, new NR configurations may be added during the inter-RAT handover (if target is NGEN-DC) and these configurations shouldn’t be released either. Another aspect that should be clarified is delta signaling between ng-eNB and NR, which is already supported with radioBearerConfig IE |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.3.11 UE actions upon going to RRC Idle - Added a note to clarify that only configuratuions pertaining to source connection should be released/discarded. 5.4.3.3 Reception of MobilityFromNR by the UE - Add a note similar to the one in clause 5.4.2.3 Reception of the RRCReconfiguration by the UE (use inde the mobility to NR), to clarify that also from NR to ng-eNB, delta is enabled with radioBearerConfig IE. q Impact analysis Impacted functionality: Inter-RAT handover from NR to E-UTRA/5GC. Inter-operability: If the network is implemented according to this CR while the UE is not, the UE would delete all new configurations received during inter-RAT handover. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR inter-RAT handover would have duplicate behavior and ambiguity, and handover to ng-eNB would not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11 UE actions upon going to RRC Idle 5.4.3.3 Reception of MobilityFromNR by the UE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

# 5.3.11	UE actions upon going to RRC_IDLE

UE shall:

1>	reset MAC;

1>	stop all timers that are running except T320 and T325;

1>	discard any stored AS context, fullI-RNTI, shortI-RNTI-Value, ran-PagingCycle and ran-NotificationAreaInfo;

1>	discard the AS security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	enter RRC_IDLE and perform procedures as specified in TS 38.304 [21], except if going to RRC_IDLE was triggered by reception of the MobilityFromNRCommand message or by selecting an inter-RAT cell while T311 was running.

NOTE: If going to RRC_IDLE was triggered by the reception of the MobilityFromNRCommand message, the release of the radio resources does not refer to those associated with RBs (re-)configured by the RRCConnectionReconfiguration message received in the MobilityFromNRCommand message.

END OF FIRST CHANGE

START OF SECOND CHANGE

# 5.4.3.3	Reception of the MobilityFromNR by the UE

The UE shall:

1>	if the targetRAT-Type is set to eutra:

2>	consider inter-RAT mobility as initiated towards E-UTRA;

- access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT.

NOTE:     In the case that the target RAT is an E-UTRA cell connected to 5GC, the delta configuration can be carried in RRCReconfiguration, e.g. PDCP-config IE and SDAP-config IE can be absent, which means the PDCP entity and SDAP entity established with source cell of the DRBs is maintained and admitted by target cell during the handover. After the inter-RAT handover is completed the PDCP and SDAP entity is maintained for the target cell of the target RAT. As a result, in-sequence and lossless handover can be achieved during intra-system inter-RAT handover.

END OF SECOND CHANGE