3GPP TSG-RAN WG2 Meeting #103bis	R2-1816055

Chengdu, China, 08 - 12 October 2018

Title:	Further reply LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon

Response to:	R1-1809958

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Dawid Koziol

E-mail Address:	dawid.koziol@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has further discussed the issue of ssb-PositionsInBurst mismatch and noticed that parameters ssb-PositionsInBurst provided in ServingCellConfigCommon and ssb-PositionsInBurst provided in ServingCellConfigCommonSIB are treated as two different parameters in RAN1 specifications. RAN2 would like to clarify that the corresponding parameters provided in ServingCellConfigCommon and ServingCellConfigCommonSIB are treated with equal priority and if the network would provide a value in ServingCellConfigCommon that differs from the value broadcast in ServingCellConfigCommonSIB, the UE would override the former with the latter. That would be not a proper network configuration, so, as such, it is not possible to configure the ssb-PositionsInBurst in ServingCellConfigCommon differently from ssb-PositionsInBurst in ServingCellConfigCommonSIB neither in this nor future releases of the RRC specifications.

RAN2 understands that RAN1 intended to have another SSB bitmap defined to be used for UE-specific PDSCH rate matching around SSBs, which would override a bitmap provided by ssb-PositionsInBurst for this purpose. RAN2 can introduce a new parameter for such purpose in a backwards compatible way in UE’s dedicated RRC signalling (e.g. ssb-PositionsToRateMatchAround). This parameter would not be a dedicated version of the currently present ssb-PositionsInBurst but rather a separate parameter. Hence, the UE may use them independently for different purposes (to be described in the RAN1 specifications).

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above information when drafting their specifications. RAN1 is further requested to provide feedback on the introduction of the new parameter as described above.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#104 		12 - 16 Nov 2018    		Spokane

3GPPRAN2#105		25 Feb - 01 Mar 2018		Athens