3GPP TSG-RAN WG2 Meeting #103bis         	                                 R2-1816064

Chengdu, China, October 8th – 12th 2018

Title:	LS on the search space of cross-carrier scheduling

Response to:	R1-1810009/R2-1813525

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Contact Person:

Name:	Yue Ma

E-mail Address:	mayue@vivo.com

1. Overall Description

RAN2 would like to thank RAN1 for the LS on cross-carrier scheduling configuration. RAN2 discussed the issue and have the strong preference to adopt an approach which is without ASN.1 change. RAN2 has the following two potential directions, both of which can be done without ASN.1 change:

- Solution without ASN.1 change suggested by RAN1:

- RAN2 understanding is that a UE uses the same search space ID for linkage of scheduled cell and scheduling cell. The IE nrofCandidates configured in the SearchSpace with the same search space ID in the scheduled cell is used by the scheduling cell for cross carrier scheduling.

- RAN2 understanding is that all parameters (including CORESET id) of SearchSpace other than nrofCandidates in any SearchSpace of the scheduled cell can be ignored.

- Solution without ASN.1 change discussed in RAN2:

- A search space of the scheduled cell is always configured in the PDCCH-Config for each BWP of the scheduled cell. From the crossCarrierSchedulingConfig the UE knows the scheduling cell in which it has to search for the grants and assignment.

- The SearchSpace configured in the PDCCH-Config of the scheduled cell points to the CORESET ID of the CORESET that is configured in the scheduling cell.

There are basically two understandings of solution2 in RAN2:

- UE follows all other parameters other than nrofCandidates in a search space configured in scheduled cell, i.e., in scheduling cell UE applies the whole search space configuration of the scheduled cell.

- UE follows the nrofCandidates in search space configured in scheduled cell, and uses the CORESET ID to find the associated CORESET in the scheduling cell, all other IEs are ignored, similar to solution1

In short summary, the main difference of the two above solutions

- Solution1: UE uses the same ID for linkage of the search spaces configured in scheduled and scheduling cell, and ignores the CORESET ID configured in the scheduled cell

- Solution2: UE uses the CORESET ID configured in the scheduled cell, to point to the same CORESET ID configured in the scheduling cell, while the two above understandings are to be discussed.

For both two solutions above, RAN2 have some general issues to be confirmed:

- For the configuration of search space, the related IEs PDCCH-Config, controlResourceSet and search space are to be configured in scheduled cell, besides nrofCandidates it is beneficial for RAN2 to know which parameters inside the configuration are useful, e.g. tci-PresentInDCI

- Since SearchSpaces and CORESETs are configured “inside a BWP” which is configured inside a Serving Cell, SearchSpaces belong unambiguously to a BWP of the scheduled cell, and SearchSpaces with same ID or CORESETs belong unambiguously to a BWP of the scheduling cell. Therefore, a UE will acquire DCI for a cross-carrier scheduled cell if the BWP of the SearchSpace/CORESET on the scheduling cell is active. But it is questionable if the search space configured on a BWP of the scheduled cell is to be used by scheduling cell, whether this BWP configured with the search space should always be activated in the scheduled cell.

2. Actions:

ACTION:

RAN2 respectfully asks RAN1 to evaluate the above two main directions and select a preferable one. RAN1 is also kindly asked to:

- Confirm the issues listed above

- If solution2 is preferable, RAN2 would like to ask which understanding for solution2 above is acceptable for RAN1.

- Feedback any other related issues identified in RAN1

3. Date of Next RAN2 Meetings:

RAN2#104		12-16 November 2018			Spokane, Washington, US

RAN2#105		25 February-1 March 2019		Athens, GR