3GPP TSG-RAN2 Meeting #103bis 	R2-1814078

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0250 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on Cell Identifier for MAC-I calculation at resume and reestablishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently it is unclear which cell identifier should be used in the resumeMAC-I and shortMAC-I calculation. Additionally, the cell identifier is not globally unique leading to the risk that the same resumeMAC-I and shortMAC-I would be generated in different target cells. Impact analysis: If the UE implements the change but the NW does not, the resume and reestablishment procedure would likely fail leading to recovery via IDLE state If the NW implements the change but the UE does not, the resume and reestablishment procedure would likely fail leading to recovery via IDLE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE should use the cell identifier associated with the first PLMN on the PLMN list. Adding PLMN identifier as input to the resumeMAC-I and shortMAC-I calculation in order to avoid the issue with non-unique cell identifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The resume and re-establish procedure would not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | #### VarResumeMAC-Input - VarShortMAC-Input |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### –	VarResumeMAC-Input

The UE variable VarResumeMAC-Input specifies the input used to generate the resumeMAC-I during RRC Connection Resume procedure.

VarResumeMAC-Input variable

-- ASN1START

-- TAG-VAR-RESUMEMACINPUT-START

VarResumeMAC-Input  ::=     SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellIdentity                      CellIdentity,

source-c-RNTI                           RNTI-Value,

plmn-Identity							PLMN-Identity

}

-- TAG-VAR-RESUMEMACINPUT-STOP

-- ASN1STOP

Editor’s Note: FFS Additional input to VarResumeMAC-Input (replay attacks mitigation).

| VarResumeMAC-Input field descriptions |  |
| --- | --- |
| targetCellIdentity Set to CellIdentity associated with the first PLMN-IdentityInfo in the PLMN-IdentityInfoList in SIB1 of the target cell. |  |
| source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to suspension of the RRC connection. |  |
| sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to suspension of the RRC connection. |  |
| plmn-Identity Set to the first plmn-Identity in the first PLMN-IdentityInfo in the PLMN-IdentityInfoList in SIB1 of the target cell. |

#### –	VarShortMAC-Input

The UE variable VarShortMAC-Input specifies the input used to generate the shortMAC-I during RRC Connection Reestablishment procedure.

VarShortMAC-Input variable

-- ASN1START

-- TAG-VAR-SHORTMACINPUT-START

VarShortMAC-Input   ::=                 SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellIdentity                      CellIdentity,

source-c-RNTI                           RNTI-Value,

plmn-Identity							PLMN-Identity

}

-- TAG-VAR- SHORTMACINPUT-STOP

-- ASN1STOP

| VarShortMAC-Input field descriptions |  |
| --- | --- |
| targetCellIdentity Set to CellIdentity associated with the first PLMN-IdentityInfo in the PLMN-IdentityInfoList in SIB1 of the target cell. |  |
| source-c-RNTI Set to C-RNTI that the UE had in the PCell it was connected to prior to the reestablishment. |  |
| sourcePhysCellId Set to the physical cell identity of the PCell the UE was connected to prior to the RRC connection. |  |
| plmn-Identity Set to the first plmn-Identity in the first PLMN-IdentityInfo in the PLMN-IdentityInfoList in SIB1 of the target cell. |

#### –