3GPP TSG-RAN WG2#103bis	Tdoc R2-1814768

# Chengdu, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0464 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LCH-to-cell restrictions when duplication is deactivated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The logical channel restriction allowedServingCells is applied for both duplication activated and deactivated state. This avoids the additional complexity and exceptional handling introduced in lifting the restrictions. Retransmissions of the in RLC PDUs for which duplication is deactivated may interfere by using the same resources as the remaining RLC entity of the duplication bearer, potentially introducing delay/jitter due to the multiplexing requred. PDCP duplication deactivation is not necessarily coupled with RLC failure or failure of the cell the LCH is restricted to leading to any PDU to remain stuck in RLC. Furthermore, PDCP discards from the RLC all new not yet transmitted duplicates, i.e. no outstanding new PDUs need to be transmitted on RLC for which duplication is deactivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 5.4.3.1.2: Removes the condition that restriction allowedServingCells does not apply to logical channels associated with a DRB configured with PDCP duplication for which PDCP duplication is deactivated. Impact analysis: Impacted functionality: Logical channel prioritization/restriction will be impacted, i.e. implementing this CR allows configuration and maintaining LCH restriction after CA/DC duplication deactivation. Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability problem. - If the UE is implemented according to the CR and the network is not, there is no inter-operablitiy problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Logical channel cell restriction will be applied differently in the MAC entity depending on CA or DC duplication is configured and deactivated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

End of changes