# 3GPP TSG-RAN2 Meeting #103bis	R2-1814975

# Chengdu, China, 2018-10-08 to 2018-10-12

Title:	[DRAFT] Reply LS on cross-carrier scheduling

Response to:	LS on cross-carrier scheduling (R1-1810009)

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Ericsson [To be RAN WG2]

To:	RAN1

Cc:	-

Contact Person:

#### Name:	Henning Wiemann

Tel. Number:	+49-2407-575-0

E-mail Address:	Henning [dot] Wiemann [at] Ericsson [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-1814974

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on cross-carrier scheduling. RAN2 discussed the issue and agreed the attached CR. In short, RAN2 intends to configure SearchSpaces and CORESETs for cross-carrier scheduling as follows:

- A search space is always configured in the PDCCH-Config of the scheduled cell.

- From the crossCarrierSchedulingConfig the UE knows the scheduling cell in which it has to search for the grants and assignment, i.e., the associated CORESET.

- The CORESET referred to from within the SearchSpace is configured in the scheduling cell.

Since SearchSpaces and CORESETs are configured “inside a BWP” which is configured inside a Serving Cell, SearchSpaces belong unambiguously to a BWP of the scheduled cell and CORESETs belong unambiguously to a BWP of the scheduling cell. Therefore, a UE will only acquire DCI for a cross-carrier scheduled cell if the BWP of the SearchSpace on the scheduled cell and the BWP of the CORESET on the scheduling cell are active. The network has to ensure this.

RAN2 assumes that the outlined configuration allows the UE to determine the number of PDCCH candidates for each scheduled cell.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to review the change/clarification proposed above and in the attached CR and to inform RAN2 if RAN1 identifies problems with that approach.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #104 	2018-11-12 to 2018-11-16	Spokane, US.

TSG-RAN WG2 Meeting #105	2019-02-25 to 2019-03-01	Athens, Greece