3GPP TSG-RAN WG2 Meeting #103bis	R2-1815264

Chengdu, China, 8th - 12th October 2018

Title:	Reply LS on measurement gap applicability

Response to:	R4-1807971

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG2

To:	RAN WG4

Cc:

Contact Person:

Name:	Xiaoran ZHANG

E-mail Address:	zhangxiaoran@chinamobile.com

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on measurement gap applicability (R4-1807971). At RAN2#103bis meeting, RAN2 discussed the capability signalling of supporting or not supporting measurement gap (pattern #2 and #3) in NR standalone operation for LTE measurement, and the conclusions are:

- Existing UE capability signalling of supportedGapPattern in TS 38.331 can indicate supporting or not supporting measurement gap (pattern#2 and #3) requested by RAN4. No new signaling is needed.

==================================TS 38.331===================================

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::= 					SEQUENCE {

measAndMobParametersCommon				MeasAndMobParametersCommon			OPTIONAL,

measAndMobParametersXDD-Diff				MeasAndMobParametersXDD-Diff		OPTIONAL,

measAndMobParametersFRX-Diff				MeasAndMobParametersFRX-Diff		OPTIONAL

}

MeasAndMobParametersCommon ::= 			SEQUENCE {

supportedGapPattern					BIT STRING (SIZE (22))			OPTIONAL,

ssb-RLM								ENUMERATED {supported}			OPTIONAL,

ssb-AndCSI-RS-RLM					ENUMERATED {supported}			OPTIONAL,

...,

[[

eventB-MeasAndReport		ENUMERATED {supported}		OPTIONAL,

handoverFDD-TDD			ENUMERATED {supported}		OPTIONAL,

eutra-CGI-Reporting		ENUMERATED {supported}		OPTIONAL,

nr-CGI-Reporting			ENUMERATED {supported}		OPTIONAL

]]

}

- RAN2 also found that the definition of supportedGapPattern in TS 38.306 is not aligned with RAN4 LS and RAN4 feature list. So RAN2 agreed to revise the definition of UE capability supportedGapPattern as below:

| Definitions for parameters | Per | M | FDD-TDD diff | FR1 FR2 diff |
| --- | --- | --- | --- | --- |
| supportedGapPattern Indicates short measurement gaps (pattern#2 and #3 in TS 38.133 [5]) optionally supported by the UE in NR standalone operation for LTE measurements. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.311 [9] and so on. | UE | No | No | No |

2. Actions:

To RAN2 WG:

RAN2 kindly asks RAN4 to take the above information into account, and confirm RAN2’s understanding on the UE capability signalling and definition of supportedGapPattern.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting#104 	November 12 – 16, 2018		Spokane, US

TSG-RAN2 Meeting #105	Feb 25 – Mar 01, 2019		Athens, GR