3GPP TSG-RAN WG2 Meeting#103-bis	        			R2-1815410

Chengdu, China 08-12 October 2018

Source:	ZTE Corporation, Sanechips

Title:	Consideration on the RRC re-establishment across E-PLMN

Agenda Item:	10.4.1.3.5

Document for:	Discussion & Decision

Introduction

The intention of the contribution is to share some consideration on the RRC re-establishment across E-PLMN.

Discussion

In the previous meeting, the LS from CT1 [1] was received and the LS clarified that "When the UE in NAS connected mode with RRC Inactive reselects to an equivalent PLMN and performs a resume procedure, the UE provides the selected PLMN ID in the resume complete message if network sharing is supported by the gNB.". According to the LS, the IE selectedPLMN-Identity is introduced in the RRCResumeComplete message. However, since the LS only mention the resume procedure, the RRC re-establishment procedure has not been touched. Considering RRC re-establishment is quite similar as the resume in some aspect, it would be nice to clarify that whether the RRC re-establishment across E-PLMN is allowed or not.

Based on current RRC spec, the UE should perform cell selection in accordance with the cell selection process as specified in TS 38.304 to find a suitable NR cell. The definition of suitable cell in 38.304 can be found as follow:

--------------------------------------------------- From 38.304 f00 -----------------------------------------------------

suitable cell:

A cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either:

-	the selected PLMN, or:

-	a PLMN of the Equivalent PLMN list.

-	The cell selection criteria are fulfilled, see subclause 5.2.3.2;

-	A cell is served by the selected/registered PLMN and not barred.

According to the latest information provided by NAS:

-	The cell is not barred, see subclause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" [12], which belongs to a PLMN that fulfils the first bullet above.

--------------------------------------------------- From 38.304 f00 -----------------------------------------------------

Based on the description above, besides the cell belonging to the selected PLMN, it can be observed that the cell belonging to a PLMN of the Equivalent PLMN list can be considered as suitable cell as well. Also considering the change of PLMN within Equivalent PLMN list will not trigger a PLMN selection procedure, we think the RRC re-establishment across E-PLMN should be allowed.

Proposal 1: Confirm that the RRC re-establishment across Equivalent PLMN or between selected PLMN and Equivalent PLMN is allowed.

With the proposal 1, another issue need to be discussed is whether the PLMN ID is needed in the RRCReestablishmentComplete message. In LTE, it is clearly specified that “The connection re-establishment succeeds only if the concerned cell is prepared i.e. has a valid UE context.”. However, in NR, such sentence is removed and instead of that we added the description “The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to section 5.3.3.4. ”, which enable the resume similar procedure with context fetch procedure.

Observation 1: In LTE, the re-establishment can only be made on the prepared cell (i.e. has a valid UE context). However, in NR, the re-establishment is allowed to be made on any cell if the network is able to find and verify a valid UE context.

With this different, the RRC reestablishment procedure is quite similar as the resume procedure. Based on the similar reason for the introduction of selectedPLMN-Identity in the RRCResumeComplete, we think the selectedPLMN-Identity should be introduced in RRCReestablishmentComplete as well.

Proposal 2: The selectedPLMN-Identity should be introduced in RRCReestablishmentComplete.

Conclusion

Based on the analysis given above, we give our proposal as follow:

Proposal 1: Confirm that the RRC re-establishment across Equivalent PLMN or between selected PLMN and Equivalent PLMN is allowed.

Observation 1: In LTE, the re-establishment can only be made on the prepared cell (i.e. has a valid UE context). However, in NR, the re-establishment is allowed to be made on any cell if the network is able to find and verify a valid UE context.

Proposal 2: The selectedPLMN-Identity should be introduced in RRCReestablishmentComplete.

- The corresponding CR can be found in [4]

References

- R2-1811004	Reply LS on inclusion of NSSAI information during Inactive to connected mode transition (C1-184474; contact: Qualcomm)	CT1	LS in

- 38.331

- 38.304

- R2-1815411 CR to 38.331 on the RRC re-establishment across E-PLMN