3GPP TSG-RAN WG2 #103bis	R2-1815541

# Chengdu, China, 8th - 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The handling of AS and NAS events triggered simultaneously |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In previous RAN2 meeting, RAN2 has made the following agreements about RNA update and TAU. For this specific scenario RAN2 has agreed to take the combining RNAU and TAU method and use the cause value associated with the TAU. However, we have not talked about other possible scenario of RNA update and NAS triggered events happening simultaneously. This CR implements changes resulting from our contribution(R2-1814729), with the proposals: RNAU and NAS triggered events happening simultaneously - To use access barring parameters and the cause value associated with NAS triggered events for the case of RNAU and NAS triggered events happening simultaneously. RAN paging and NAS triggered events happening simultaneously To use access barring parameters and cause value associated with RAN paging for the case of RAN paging and NAS triggered events (except for CN paging) happening simultaneously, i.e. the combining procedures are not subject to UAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR implements changes resulting from our contribution(R2-1814729), with the proposals: RNAU and NAS triggered events happening simultaneously - To use access barring parameters and the cause value associated with NAS triggered events for the case of RNAU and NAS triggered events happening simultaneously. RAN paging and NAS triggered events happening simultaneously To use access barring parameters and cause value associated with RAN paging for the case of RAN paging and NAS triggered events (except for CN paging) happening simultaneously, i.e. the combining procedures are not subject to UAC. The first case has already been covered by the TAU and RNAU happening simultaneously case, we only need to implement the handling of RAN paging and NAS triggered events happening simultaneously case. - So a sentence “The UE initiates the procedure for NG-RAN paging when upper layers and AS (when responding to NG-RAN paging) requests the resume of a suspended RRC connection.” was added to address the case to make sure RAN paging overrides NAS triggered events. Impact Analysis Impacted functionality: NR for standalone operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling on simultaneously triggered NAS&AS events is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to NG-RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

The UE initiates the procedure for NG-RAN paging when upper layers and AS (when responding to NG-RAN paging) requests the resume of a suspended RRC connection simultaneously.

Upon initiation of the procedure, the UE shall:

Editor's Note: FFS Whether SCG configuration should be released or whether that should be treated as any other configuration (i.e. with delta signalling).

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting the resumption of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the resumption of the RRC connection is triggered due to an RNA update:

2>	if an emergency service is ongoing:

NOTE:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

2>	else:

3>	select [the standardised RAN specific access category] as the Access Category;

Editor's note: Which value to use for the standardised RAN specific access category needs to be confirmed by SA1.

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	release the current dedicated Serving Cell configuration and apply the specified values in corresponding specification except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.x1;

1>	release delayBudgetReportingConfig, if configured and stop timer T3xx, if running;

1>	apply the CCCH configuration as specified in 9.1.1.x2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	stop timer T380, if running;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

Editor's Note: FFS Requirements on up to date system information acquisition before connection resumption.

End Text Proposal Change