3GPP TSG-RAN2 Meeting #103bis 	R2- 1815599

Chengdu, China, 2018-10-08 to 2018-10-12 	(Revision of R2-1814900)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0394 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on AS-NAS interaction of RRC Reject |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing text is not very clear and contain some redundant text. The condition on the first level is based on the message that was rejected. However, RRCResumeRequest can be requested either by upper layers or RRC. So with the current ordering and identation, one needs to add in the second level a condition regarding whether upper layers or RRC requested the resume initiation. RRCSetupRequest can only be responded with RRCReject if this is a request from upper layers, hence, that condition should be one level below a request from upper layers. RRCResumeRequest or RRCResumeRequest1 can be responded by RRCReject eve if this is requested by upper layers or RRC. However, upper layers are only informed if the request comes from upper layers. Hence, this should also be one level below i.e. only when request is from upper layers. For the rejected request triggered by RRC, that should be one step higher so actions when the rejected request is from upper layers and RRC request are quite independent. Impact analysis: There is no impact on the UE if network does not implement that. There is no impact on the NW if the UE does not implement that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conditions for UE actions upon reception of RRCReject are split based on the case where rejected request was from upper layers or RRC. As the actions are mainly defined based on that criterion, the actions become quite clear in each case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE actions are ambiguous as resume can be triggered either by upper layers or RRC (in case of RNA updates). |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.3.15	RRC connection reject

#### 5.3.15.1	Initiation

The UE initiates the procedure upon the reception of RRCReject when the UE tries to establish or resume an RRC connection.

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	reset MAC and release the MAC configuration;

1>	start timer T302, with the timer value set to the waitTime;

1>	set the variable pendingRnaUpdate to 'FALSE';

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories ‘0’and '2';

2>	if RRCReject is received in response to an RRCSetupRequest:

3>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

2>	if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	inform upper layers about the failure to resume the RRC connection;

1>	if RRCReject is received in response to a request from RRC:

2>	set the variable pendingRnaUpdate to 'TRUE';

2>	discard the security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key;

2>	suspend SRB1, upon which the procedure ends;

Editor’s Note: FFS Handling of timer T380 upon Reject e.g. stop, re-start, etc.

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.