3GPP TSG-RAN2 104                                                                                          R2-1816307

Spokane, USA, 12th - 16th Nov 2018

Title:	Reply LS on network synchronization for handover purposes

Response to:	R1-1812074

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Samsung (To be RAN WG2 )

To:	RAN WG1

Cc:

Contact Person:

#### Name: Anil Agiwal

E-mail Address:	anilag@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on network synchronization for handover purposes. RAN2 discussed the issue and following was agreed:

- Upon receiving RRCReconfiguration including reconfigurationWithSync, UE is not required to always acquire MIB from target SpCell.

1a: UE acquires MIB from target SpCell if source SpCell’s frame timing cannot be used in target SpCell.

1a-1:    UE acquires MIB from target SpCell before the random access if frame timing is needed to determine the RACH occasions.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above agreements in to account in their specification work.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN2 Meeting #105		25th February – 1st March 2019		Athens, Greece

TSG-RAN2 Meeting #105-bis		8th April  – 12th April 2019		China