3GPP TSG-RAN WG2 meeting#104 	R2-1816356

# Spokane, USA, 12th – 16th November 2018 								     Revision of R2-1813858

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0423 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the scaling between CSI-RS and SSB for BFR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 38.213-f30, both SS-RSRP and CSI-RSRP measurements on resources for BFR candidate beam set are compared against the same single threshold, rsrp-ThresholdSSB (Qin,LR in 38.213-f30). Since there is only one threshold, and the physical layer text does not assume any scaling on that threshold, the scaling procedure of the CSI-RS threshold in MAC (with powerControlOffset) is incorrect. Changing this procedure also implies that powerControlOffset is redundant in MAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the scaling of rsrp-ThresholdCSI-RS based on the parameter powerControlOffset in Section 5.1.1 of the MAC specification Impact analysis Impacted functionality: Random Access procedure initialization. Inter-operability: - If the network is implemented according to the CR and the UE is not, there is an interoperability issue in that the NW assumes a correct UE behavior when it selects the candidate beams for BFR based on CSI-RS measurements, although the threshold used for that will be erroneously scaled by MAC. - If the UE is implemented according to the CR and the network is not, there is no interoperability problem since the issue is fixed in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE applies the SSB to CSI-RS scaling twice: on the measurement itself in PHY and on the threshold in MAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS and corresponding Random Access Preamble and/or PRACH occasion. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	powerRampingStep: the power-ramping factor;

End of change