| 3GPP TSG RAN WG2 Meeting #104 | R2-1816436 |
| --- | --- |
| Spokane, USA, 12th – 16th November 2018 |  |

Agenda item:	12.2.3

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title: 	UE-Group Wake-Up Signal

Document for:	Discussion and decision

# Introduction

At the RAN plenary #80, a new Work Item (WI) on “Rel-16 enhancements for NB-IoT” was approved [1]. The WID states that one objective is to specify the following improvement for machine-type communications for NB-IoT FDD:

Improved DL transmission efficiency and/or UE power consumption:

- Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

This Topic was already discussed in RAN1#94 and RAN1#94bis with considerable agreements for MTC and NB-IoT WUS [2]. [3] [4].

At RAN2 #94bis the following agreements have been made:

The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping

In this contribution we focus on the topic of UE-group WUS for NB-IoT and MTC.

# Discussion

Rel-15 introduced a wake-up signal to trigger UEs that will be paged in the next PO. A Rel-15 WUS wakes up every UE for that PO, even UEs that will not be paged in this PO.

By further dividing the the UEs into Rel-16 groups, the probability of false wake-ups can be decreased and false alarm rate of the paged UEs can be further decreased. The Rel-16 WUS shall enable a sub-grouping within a PO to reduce the false alarm rate and reduce the unnecessary wake-up of UEs that will not be paged.

## Number of groups in Rel-16 WUS

Until now there is no agreement on the supported number of groups for Rel-16 WUS. In RAN1 companies proposed numbers between 1 and 4 (8?), some including a common group, other not. Regarding the false-alarm rate, a higher number of groups is considered beneficial. To reduce the complexity of the WUS sequence, a 2-step approach consisting of two Rel-16 WUS with CDM WUS sequence for each step allows sub-grouping and fine granular wake-up functionality is proposed with each step enabling a total of 4 groups/sub-groups.

The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

## Criteria for grouping

During RAN2 #103bis the agreement was made that at least UE_ID based grouping is supported. A second option for engrouping is considered, but not agreed yet. An additional criteria for grouping could be a service ID that reflects i.e. the requirements for updates or energy restriction.

The UE-grouping is based on the UE ID. A service ID might be used for a second level of differentiation.

Coverage is not considered as a grouping feature.

## Coexistance of Rel-16 WUS with Rel-15 legacy WUS

The agreements from RAN1#94bis state that Rel-16 capable UEs shall also be capable of Rel-15 WUS. Whether Rel-15 WUS is a fallback for Rel-16 WUS in case Rel-16 WUS is not enabled in the cell or whether Rel-16 UEs shall also receive Rel-15 WUS even if Rel-16 WUS is active is not defined yet.

This opens the possibility to use Rel-15 WUS as a wake-up beacon for Rel-16 grouping WUS feature. The impact of Rel-15 WUS as a beacon for Rel-16 UEs should be considered carefully.

Having Rel-15 WUS as a super-group in addition to Rel-16 devices would ease the backward compatibility but may impact other Rel-15 devices due to a higher number of wake-ups.

The Rel-15 behavior of Rel-16 UEs should be further studied.

## Sub-grouping for Rel-16 WUS

As mentioned in 2.1.1, a higher number of subgroups reduces the probability of unnecessary wake-ups. The number of groups is still FFS in RAN1. Considering the complexity of the WUS and the advantages of a reasonable number of groups, a 2-step approach with sub-groups is a well-suited solution.

The Rel-16 WUS can therefore introduce a 2-step addressing scheme for sub-grouping that consists of

- A: Rel-15(or 15/16) WUS + Rel-16 CDM WUS for sub-groups or

- B: Rel-16 CDM WUS + 2nd Rel-16 CDM WUS for sub-sub-groups

Each WUS needs to support only a small number of (sub-)groups and therefore reduces the necessary complexity of the WUS receiver.

Rel-16 WUS uses a 2-step addressing for sub-groups.

The Rel-15 WUS can either be ignored (Option B) or can be used as an initial beacon (Option A). In both cases, subgrouping for Rel-16 UEs is possible. Option B does not use the Rel-15 WUS but uses a dedicated Rel-16 CDM WUS sequence as a first beacon and a second one for sub-grouping. This option allows for a fine granular addressing of UE-groups and can even be used to differentiate between “groupcast” and “unicast” if found beneficial.

Figure 1 – Rel-15 WUS with REL-16 WUS subgrouping – Option A

Figure 1 shows the concept of subgrouping for Option A. The use of Rel-15 WUS as a “super-group” allows for backward compatibility. The Rel-15 devices and Rel-16 devices may be considered already as “subgroup x”. The effect on legacy devices should be considered and is FFS. Figure 2 illustrates the Rel-16 UE behavior of Rel-15 WUS which is NOT used as a super-group-beacon. For activation of Rel-16 UEs, no Rel-15 WUS is needed and Rel-15 UEs do not wake up.

Figure 2 – Rel-16 WUS subgrouping w/o Rel-15 WUS – Option B

In addition to a Rel-15 beacon, a 2-step Rel-16 WUS with CDM WUS sequence for each step allows sub-subgrouping and fine granular wake-up functionality. The grouping of 1st and 2nd step CDM WUS is done via UE_ID, but may also include another differentiation criteria.

The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

# Conclusions

In this contribution the following observations and proposals have been made:

Proposal 1:	The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

Proposal 2:	The UE-grouping is based on the UE ID. A service ID might be used for a second level of differentiation.

Proposal 3:	Coverage is not considered as a grouping feature.

Proposal 4:	The Rel-15 behavior of Rel-16 UEs should be further studied.

Proposal 5:	Rel-16 WUS uses a 2-step addressing for sub-groups.

Proposal 6:	The Rel-16 part of the WUS supports a minimum of 4 different sequences (=subgroups) per step.

# References

| [1] | „RP-181674: "WID: Additional enhancements for NB-IoT"“. |
| --- | --- |
| [2] | „R1-1810051: Final Report of 3GPP TSG RAN WG1 #94 v1.0.0,“ Gothenburg. |
| [3] | „R1-1811658: Summary of 6.2.1.1 UE group MWUS“. |
| [4] | „R1-1811664: Feature summary of UE-group wake-up signal for NB-IoT“. |