3GPP TSG RAN WG2 #104	R2-1816564

Spokane, USA, 12th – 16th November 2018

Agenda Item:	11.1.3

Source:	Ericsson

Title:	Minimizing CN functionalities for IAB network

Document for:	Discussion

# Introduction

It is expected that a main driver for IAB is to be able to deploy 5G NR coverage also at sites where it is too costly or otherwise not feasible to deploy dedicated transport. As such deploying IAB could be a local decision as part of the RAN deployment in a certain area. It could even be so that an IAB node could be “alone” in an area surrounded by NR base stations which have dedicated transport.

We think it is important for the feasibility of IAB deployment that the impacts and requirements on the overall network should be minimized for a single deployment of an IAB node. As such we think it would be beneficial if it is possible to deploy an IAB node without any impacts to existing CN (regardless if it is EPC or 5GC).

IAB deployment could be based on local RAN deployment decisions based on local transport situation and as such it is beneficial if this can be done without any interworking with the Core Network(s).

This paper is analysing what would be needed to support a “CN less” deployment of IAB nodes. It is proposed to capture a text proposal to 38.874.

# Discussion

## Overview

Currently when a UE attaches to 3GPP network it relies on certain functionality from the CN, including at least the following:

- Attach signalling, Initial Authentication Key Agreement, NAS security

  - Handling of NAS/AS security capabilities

- User plane related aspects:

  - PDP Context/PDU session signalling, incl. QoS, packet filtering, IP allocation

  - UP anchoring

- Idle state mode mobility

- UP anchoring

The figure below shows the CN/RAN functional split from 38.300 and 36.300:

Figure 4.2-1: Functional Split between NG-RAN and 5GC

Figure 4.1-1: Functional Split between E-UTRAN and EPC

In our view not all of these functions are necessary for supporting IAB nodes (which are network nodes), and those that are needed could potentially be handled in a different way for IAB nodes.

If it would be possible to handle all of these functions in the RAN it would significantly reduce the effort to deploy new IAB nodes since it could be handled internally in the RAN. In the following section we go through these functions one by one.

If it would be possible to handle all the functions needed to support IAB nodes in the RAN it would significantly reduce the effort to deploy IAB nodes.

## User plane functions

The user plane functions in CN would be needed if there is a need to assign an IP address in the CN e.g. to let the IAB network get access to OAM system. In our view this is not needed, instead it would be possible to assign the UE an IP address in the RAN as part of Donor DU function. In this case the anchor point for the IAB node towards the operators own internal network would be located in the RAN (e.g. Donor gNB). The advantages with this are that there are no CN UP functions needed at all to support IAB nodes which significantly limits the needed CN functionality to support IAB nodes.  The disadvantage with this is mainly related to that it would be more complex to support mobile IAB nodes when the IP anchoring is in the RAN. Mobile IAB node is however not in the scope for Rel-16.

Additionally, it is expected that the QoS handling on the BH interface will be handled by the RAN, meaning that there is no need for all the QoS functionality support in the CN for IAB nodes.

By supporting IP UP connectivity in the RAN towards the IAB node there is no need to rely on CN UP functionality including QoS for supporting IAB nodes.

Solutions should be explored where IAB nodes can be supported without any CN UP functions.

## Idle mode mobility

In our view we do not think there is a need to support IDLE mode mobility for IAB nodes. We expect that the IAB nodes can remaining in MM Connected state all the time after initial IAB node attach. Any IAB node path changes can be performed in connected mode within the same registration area. For this reason, there is no need to support CN Idle mode handling e.g. Tracking Area Updates, Paging.

Initial cell selection and re-selection (e.g. at RLF) still need to be supported by IAB nodes.

Besides initial cell selection and re-selection (e.g. at RLF), IAB nodes does not need to support other RRC_IDLE functionality (e.g. paging, UE registration area updates) currently performed by the CN.

Solution should be explored where IAB nodes can be supported without any CN IDLE mode mobility functionality.

## Attach signalling, initial AKA, NAS security etc.

Currently UEs use NAS signalling to authenticate themselves to the network as well as well as ensuring that the network is a valid network. It should however be noted that gNBs have other ways today for authenticating themselves to the network including:

- Ways to authenticate themselves to the OAM system.

- Ways to setup Network Domain Security

These ways rely on Certificates provisioned in the gNBs and the network. Potentially these certifications could also be used protect AS signalling. One option is relying on only NDS to protect all the IAB node signalling. The advantage of this is that also Attach signalling, initial AKA, NAS security could be avoided.

Example solution:

- UE performs RRC connection setup to the network and indicates that the UE is an IAB node

- The network (CU-CP) setups up IP connectivity to the IAB node but places the IAB node outside the NDS secure domain since no RRC security mode command is performed.

- The IAB node performs authentication using certificates to setup NDS security

- IAB node only use NDS protected signalling for any critical signalling

It might be possible for IAB nodes to rely only on NDS for protecting control and user plane signalling towards the network. In this case there is no need to support CN based attach signalling, authentication and NAS security.

Solution should be explored for only using NDS to protect IAB node signalling avoiding the need for CN to authenticated or handle IAB nodes.

# Conclusions

In earlier sections we made the following observations:

Observation 1	IAB deployment could be based on local RAN deployment decisions based on local transport situation and as such it is beneficial if this can be done without any interworking with the Core Network(s).

Observation 2	If it would be possible to handle all the functions needed to support IAB nodes in the RAN it would significantly reduce the effort to deploy IAB nodes.

Observation 3	By supporting IP UP connectivity in the RAN towards the IAB node there is no need to rely on CN UP functionality including QoS for supporting IAB nodes.

Observation 4	Besides initial cell selection and re-selection (e.g. at RLF), IAB nodes does not need to support other RRC_IDLE functionality (e.g. paging, UE registration area updates) currently performed by the CN.

Observation 5	It might be possible for IAB nodes to rely only on NDS for protecting control and user plane signalling towards the network. In this case there is no need to support CN based attach signalling, authentication and NAS security.

Based on the discussion in earlier sections we propose the following:

Proposal 1	Solutions should be explored where IAB nodes can be supported without any CN UP functions.

Proposal 2	Solution should be explored where IAB nodes can be supported without any CN IDLE mode mobility functionality.

Proposal 3	Solution should be explored for only using NDS to protect IAB node signalling avoiding the need for CN to authenticated or handle IAB nodes.

It is proposed to accept the following text proposal to 38.874.

# Text proposal to 38.874

### 5.2.4	Core-network impact

IAB-related features such as IAB-node integration and topology adaptation may impact core-network specifications. It is desirable to minimize the impact to core-network specifications related to IAB.

Also, dependent on design, IAB features may create additional core-network signaling load. The amount of signaling load may vary among the various designs discussed in the study. Core-network signaling load is therefore considered an important KPI for the comparison of IAB designs.

Solutions should be explored in the work item for supporting IAB nodes only within the RAN. I.e. where all higher layer functionalities are terminated in the RAN.

Requirements:

1:	The IAB design shall strive to minimize the impact to core network specifications.

2:	The study should consider the impact to the core network signalling load as an important KPI.