3GPP TSG-RAN WG2 Meeting #104	R2-1816568

Spokane, USA, 12-16 November 2018

Agenda Item:	11.2.1.3

Source:	Charter Communications Inc., Qualcomm, ZTE, Nokia, Nokia Shanghai Bell, InterDigital

Title:	TP for Channel Access Priority Class in NR-U

Document for: Discussion and Agreement

# Introduction

This contribution aims to shed some light on the discussion of Channel Access Priority Class (CAPC), and proposes TP for inclusion in TR 38.889.

# Discussion

The following RAN1 agreement regarding channel access is captured in TR 38.889 [1].

##### 7.2.1.3.1	Channel access procedures

If absence of Wi-Fi cannot be guaranteed (e.g. by regulation) in the band (sub-7 GHz) where NR-U is operating, the baseline assumption is, the NR-U operating bandwidth is an integer multiple of 20MHz.

Channel access mechanisms need to comply with regulations and may therefore need to be adapted for particular frequency ranges.

For channel access mechanism, LTE-LAA LBT mechanism is adopted as baseline for 5GHz band and adopted as the starting point of the design for 6GHz band. At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

For 5GHz band, a no-LBT option is beneficial for NR-U, such as for supporting fast A/N feedback, and is permitted per regulation. Restrictions/conditions on when no-LBT option can be used will be further identified, e.g., in consideration of fair coexistence. No-LBT option can be applied to 6GHz band if allowed by regulation. Restrictions/conditions on when no-LBT option can be used will be further identified, if fair coexistence criterion is defined for 6GHz band.

For CWS adjustment procedure in NR-U, in addition to aspects considered in LTE LAA, NR-U may additionally consider at least the following aspects: CBG based HARQ-ACK operation, NR scheduling and HARQ-feedback delays and processing times, wideband (>20 MHz) operation including BWPs, Configured grant operation.

The channel access mechanism for LTE LAA is defined in clause 4 of TS 37.213 [2]. Based on which, the corresponding mapping of standardized QCIs to Channel Access Priority Class is defined in clause 5.7.1 of TS 36.300 [3].

### 5.7.1	Channel Access Priority Classes

Four Channel Access Priority Classes are defined in [6] which can be used when performing uplink and downlink transmissions in LAA carriers. Table 5.7.1-1 shows which Channel Access Priority Class should be used by traffic belonging to the different standardized QCIs. A non-standardized QCI (i.e. Operator specific QCI) should use suitable Channel Access Priority Class based on the below table, i.e. the Channel Access Priority Class used for a non-standardized QCI should be the Channel Access Priority Class of the standardized QCIs which best matches the traffic class of the non-standardized QCI.

For uplink, the eNB selects the Channel Access Priority Class by taking into account the lowest priority QCI in a Logical Channel Group.

Table 5.7.1-1: Mapping between Channel Access Priority Classes and QCI

| - Channel Access Priority Class () | - QCI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 69, 70 |
| 2 | 2, 7 |
| 3 | 4, 6, 8, 9 |
| 4 | - |

For LTE LAA UL scheduled transmissions, the network indicates the CAPC in the UL grant e.g. based on information conveyed in the BSR.

For LTE LAA autonomous UL (AUL) transmissions, the CAPC is configured per logical channel, with MAC CE having the highest priority. This is also described in clause 5.7 of TS 36.300 [3].

For type 1 uplink channel access on AUL, E-UTRAN signals the Channel Access Priority Class for each logical channel and UE shall select the lowest Channel Access Priority Class (i.e, with a higher number in the Table 5.7.1-1) of the logical channel(s) with MAC SDU multiplexed into the MAC PDU. The MAC CEs except padding BSR use the highest Channel Access Priority Class (i.e, the lowest number in the Table 5.7.1-1).

The CAPCs mapping to parameters which control how Clear Channel Assessment is performed in indicated per Table 4.1.1-1 and Table 4.2.1-1 of TS 37.213 [2] for LTE LAA DL and UL respectively.

Since RAN1 agreement uses LTE LAA channel access as the baseline, it is proposed that for the purpose of the SID, the resulting mechanisms in LTE LAA (e.g. standardized QCICAPC mapping for DL and UL, CAPC used for scheduled UL configured per logical channel, CAPC used for AUL configured per logical channel) can be used as the starting point. During the NR-U WID phase, if RAN1 defines additional CAPCs, or any channel access enhancements (from LTE LAA baseline) e.g. due to introduction of PRACH, or support for FBE, then RAN2 can correspondingly complete the associated higher layer aspects.

Furthermore NR-U is expected to support both DC and SA. This would imply transmission of RRC control signalling over unlicensed carrier. This is in contrast to LTE LAA where CAPC was only applicable to DRB carrying user data. Therefore, CAPC for RRC control signaling (e.g. SRB0, 1, 2, 3 which do not correspond to standardized 5QIs) need to be defined. It is expected that CAPC of such RRC control signaling will have the highest access priority.

Proposal 1: For channel access and transmissions in NR-U the mechanisms and associated signaling adopted by LTE LAA (e.g. standardized QCI to access priority mapping for DL and UL, how access priority per logical channel is determined for scheduled UL and AUL transmissions etc) are used as the baseline. Any changes due to new physical layer design and channel access mechanisms for NR-U (e.g. introduction of PRACH, support of FBE) can also be introduced.

Proposal 2: In addition, access priority for control signaling (transmissions over SRBs) over unlicensed carriers should be introduced for stand-alone and DC NR-U. In this case, it is assumed that control signaling will have the highest access priority.

# Conclusion

The following proposals are provided in this contribution.

Proposal 1: For channel access and transmissions in NR-U the mechanisms and associated signaling adopted by LTE LAA (e.g. standardized QCI to access priority mapping for DL and UL, how access priority per logical channel is determined for scheduled UL and AUL transmissions etc) are used as the baseline. Any changes due to new physical layer design and channel access mechanisms for NR-U (e.g. introduction of PRACH, support of FBE) can also be introduced.

Proposal 2: In addition, access priority for control signaling (transmissions over SRBs) over unlicensed carriers should be introduced for stand-alone and DC NR-U. In this case, it is assumed that control signaling will have the highest access priority.

Corresponding text proposal is provided in Section 5.

# References

- TS 38.889 v0.1.1, “Study on NR-based Access to Unlicensed Spectrum”

- TS 37.213 v15.1.0, “Physical layer procedures for shared spectrum channel access”

- TS 36.300 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- R2-1814292, “On Channel Access Priority Classes in NR-U”, Nokia

- R2-1813675, “CAPC for data transmission in NR-U”, MediaTek

# Text Proposal

##### 7.2.2.2.2	MAC (except RACH)

##### 7.2.2.2.3	Other

##### 7.2.2.2.X	Channel Access Priority

For channel access and transmissions in NR-U the mechanisms and associated signaling adopted by LTE LAA (e.g. standardized QCI to access priority mapping for DL and UL, how access priority per logical channel is determined for scheduled UL and AUL transmissions etc) are used as the baseline. Any changes due to new physical layer design and channel access mechanisms for NR-U (e.g. introduction of PRACH, support of FBE) can also be introduced.

In addition, access priority for control signaling (transmissions over SRBs) over unlicensed carriers should be introduced for stand-alone and DC NR-U. In this case, it is assumed that control signaling will have the highest access priority.