3GPP TSG-RAN WG2 #104	R2-1816643

# Spokane, USA, 12th – 16th November 2018 			Resubmission of R2-1814347

Agenda Item:	12.1.12

Source:	Ericsson

Title:	Temporary SI densification

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss a temporary system information (SI) densification technique for LTE-M and NB-IoT to reduce system acquisition time. The technique has not yet been discussed in RAN2.

This paper is a resubmission of contribution R2-1814347.

# 2 Discussion

## Temporary SI densification

Further techniques to allow reduced time to acquire SI may be considered in RAN2. For example, when a large number of UEs are expected to acquire SI, such as when SI is modified, it would be useful to help the UEs to acquire the SI quicker. This could be done by increasing the number of repetitions for a short period of time when SI is modified. With denser repetitions the UEs could acquire the SI when they are assumed to do so in a reduced acquisition time with no large system overhead. Since SI doesn’t need to be received very often, unless it is changed, the number of repetitions otherwise (when SI is not modified) could even be reduced. This could potentially even result in a reduction of the total system overhead. In case there would be legacy UEs not able to acquire denser repetitions in the system reading the same SI, they would be unaffected and both, legacy and Rel-16 UEs could coexist and receive the SI repetitions, as the denser repetitions can be interleaved with common SI transmissions.

We see a few possibilities on how to implement temporary SI densification. One option would be to define an additional set of scheduling information of SI to be applied during the dense periods. Second option would be to scale the SI scheduling by a factor, which is provided as a parameter in SIB1-BR/-NB / MIB. The second option could allow less overhead in system information. In addition to the new parameters, signalling would need to be defined from network to the UEs on when to apply the denser scheduling configuration. This could be implemented by defining a new bit for example in a MIB or a Paging message. The indicator for the beginning of denser SI repetitions could also be implicitly signalled, for example, with the indication of a system information update, which was agreed in RAN2#99bis, and the UE would know to apply the denser scheduling from the beginning of next modification period boundary. This would reduce the need for additional bits required for signalling.

Temporary SI densification for LTE-M/NB-IoT UEs is supported.

An illustration of temporary SI densification is shown in the figure below.

Figure 1 Temporary SI densification

In the Figure 1, on the time-axis, the periodic SI transmissions are shown as black bars, as configured in MIB/SIB1. Both, legacy and Rel-16 UEs are aware of this periodic scheduling and can read the SI. Upon a SI update, the temporary SI densification is applied, and the additional repetitions of SI transmissions are shown as red bars. Rel-16 UEs supporting temporary SI densification are aware of the densification of SI when the SI update occurs and are able to receive the additional repetitions in addition to the common scheduling configuration of the SI. As discussed earlier, the notification to apply the denser repetitions of SI may be implicitly assumed by Rel-16 UEs with the SI update notification sent prior the update period, during the notification period. The denser repetition period may, for example, last for one modification period after the SI update, after which the scheduling returns to normal. The common scheduling configuration still applies during the SI update period, as shown with the black bars, and therefore legacy UEs can still read the SI according to the common SI scheduling configuration.

As illustrated in the figure above, with periodic SI densification more repetitions of SI can be transmitted to the Rel-16 UEs during a certain period, thus decreasing the SI acquisition time and/or reliability of correct acquisition. This comes with the cost on system resources, as naturally more resources are needed when more repetitions are transmitted. Also, this solution takes into account the legacy UEs, which are not aware of the SI densification, by still applying the common SI scheduling configuration even during the update period.

A text proposal example for eMTC, on how to implement temporary SI densification can be seen below. The text proposal shows two new SIB1 parameters:

- tempSIDensCoeff  which multiplies the common SI scheduling by a factor,

- tempSIDensPeriod which configures the period of time that the temporary SI densification lasts.

In addition to densifying SI message repetitions, it could be desirable to densify SIB1-BR repetitions for the same reasons. As seen in the table below (3GPP 36.213), the value of schedulingInfoSIB1-BR-r13 included in MIB maps to a certain number of repetitions for SIB1-BR (carried in PDSCH).

The parameter tempSIDensCoeff could therefore also be applied to SIB1-BR repetitions, by multiplying the common repetition number of SIB1-BR configured by schedulingInfoSIB1-BR-r13 by the factor indicated by the parameter tempSIDensCoeff. Another option for SIB1-BR repetition densification could be to define another parameter, which controls only the repetitions of SIB1-BR during the dense period.

Table 7.1.6-1: Number of repetitions for PDSCH carrying SystemInformationBlockType1-BR for BL/CE UE.

| Value of schedulingInfoSIB1-BR-r13 | Number of PDSCH repetitions |
| --- | --- |
| 0 | N/A |
| 1 | 4 |
| 2 | 8 |
| 3 | 16 |
| 4 | 4 |
| 5 | 8 |
| 6 | 16 |
| 7 | 4 |
| 8 | 8 |
| 9 | 16 |
| 10 | 4 |
| 11 | 8 |
| 12 | 16 |
| 13 | 4 |
| 14 | 8 |
| 15 | 16 |
| 16 | 4 |
| 17 | 8 |
| 18 | 16 |
| 19-31 | Reserved |

RAN2 to adopt the text proposal including tempSIDensCoeff.

# 3	Text Proposal

<Start of modified section>

### 5.2.3a	Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE

When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT);

2>	determine the integer value x = (n – 1)*w, where w is the si-WindowLength-BR (or si-WindowLength in NB-IoT);

2>	if the UE is a NB-IoT UE:

3>	the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset, where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset);

2>	else:

3>	the SI-window starts at the subframe #0 in the radio frame for which SFN mod T = FLOOR(x/10), where T is the si-Periodicity of the concerned SI message;

1>	if the UE is a NB-IoT UE:

2> if the procedure is triggered by a system information change notification and SystemInformationBlockType1-v16xx-IEs is included in SystemInformationBlocktype1-NB

3> receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window  and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern divided by the value given by tempSIDensCoeff and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB and SystemInformationBlockType1-NB. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern. If the UE continues to receive over a time period longer than indicated by tempSIDensPeriod, the UE then shall fall back to receiving and accumulating SI message transmissions starting from the radio frames as provided in si-RepetitionPattern;

2>	else:

3> receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB and SystemInformationBlockType1-NB. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

1>	else:

2>	if the procedure is triggered by a system information change notification and SystemInformationBlockType1-v16xx-IEs is included in SystemInformationBlocktype1

3> receive and accumulate SI message transmissions on DL-SCH on narrowband provided by si-Narrowband, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength-BR, only in radio frames as provided in si-RepetitionPattern divided by the value given by tempSIDensCoeff and subframes as provided in fdd-DownlinkOrTddSubframeBitmapBR in bandwidthReducedAccessRelatedInfo, or until successful decoding of the accumulated SI message transmissions. If the UE continues to receive over a time period longer than indicated by tempSIDensPeriod, the UE then shall fall back to receiving and accumulating SI message transmissions starting from the radio frames as provided in si-RepetitionPattern;

2> else:

3> receive and accumulate SI message transmissions on DL-SCH on narrowband provided by si-Narrowband, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength-BR, only in radio frames as provided in si-RepetitionPattern and subframes as provided in fdd-DownlinkOrTddSubframeBitmapBR in bandwidthReducedAccessRelatedInfo, or until successful decoding of the accumulated SI message transmissions;

1>	if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message;

<Next change>

#### –	SystemInformationBlockType1

SystemInformationBlockType1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. SystemInformationBlockType1-BR uses the same structure as SystemInformationBlockType1.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH and BR-BCCH

Direction: E-UTRAN to UE

SystemInformationBlockType1 message

-- ASN1START

SystemInformationBlockType1-BR-r13 ::=	SystemInformationBlockType1

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SystemInformationBlockType1-v890-IEs::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING SystemInformationBlockType1-v8h0-IEs)			OPTIONAL,

nonCriticalExtension				SystemInformationBlockType1-v920-IEs	OPTIONAL

}

-- Late non critical extensions

SystemInformationBlockType1-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList					MultiBandInfoList		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v9e0-IEs	OPTIONAL

}

SystemInformationBlockType1-v9e0-IEs ::= SEQUENCE {

freqBandIndicator-v9e0				FreqBandIndicator-v9e0		OPTIONAL,	-- Cond FBI-max

multiBandInfoList-v9e0				MultiBandInfoList-v9e0		OPTIONAL,	-- Cond mFBI-max

nonCriticalExtension				SystemInformationBlockType1-v10j0-IEs	OPTIONAL

}

SystemInformationBlockType1-v10j0-IEs ::= SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL,	-- Need OR

nonCriticalExtension				SystemInformationBlockType1-v10l0-IEs					OPTIONAL

}

SystemInformationBlockType1-v10l0-IEs ::= SEQUENCE {

freqBandInfo-v10l0					NS-PmaxList-v10l0			OPTIONAL,	-- Need OR

multiBandInfoList-v10l0				MultiBandInfoList-v10l0		OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- Regular non critical extensions

SystemInformationBlockType1-v920-IEs ::=	SEQUENCE {

ims-EmergencySupport-r9				ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfo-v920				CellSelectionInfo-v920		OPTIONAL,	-- Cond RSRQ

nonCriticalExtension				SystemInformationBlockType1-v1130-IEs	OPTIONAL

}

SystemInformationBlockType1-v1130-IEs ::=	SEQUENCE {

tdd-Config-v1130				TDD-Config-v1130			OPTIONAL,	-- Cond TDD-OR

cellSelectionInfo-v1130			CellSelectionInfo-v1130		OPTIONAL,	-- Cond WB-RSRQ

nonCriticalExtension			SystemInformationBlockType1-v1250-IEs	OPTIONAL

}

SystemInformationBlockType1-v1250-IEs ::=	SEQUENCE {

cellAccessRelatedInfo-v1250					SEQUENCE {

category0Allowed-r12						ENUMERATED {true}		OPTIONAL	-- Need OP

},

cellSelectionInfo-v1250					CellSelectionInfo-v1250		OPTIONAL,	-- Cond RSRQ2

freqBandIndicatorPriority-r12			ENUMERATED {true}			OPTIONAL,	-- Cond mFBI

nonCriticalExtension			SystemInformationBlockType1-v1310-IEs	OPTIONAL

}

SystemInformationBlockType1-v1310-IEs ::=	SEQUENCE {

hyperSFN-r13								BIT STRING (SIZE (10))		OPTIONAL,	-- Need OR

eDRX-Allowed-r13							ENUMERATED {true}			OPTIONAL,	-- Need OR

cellSelectionInfoCE-r13					CellSelectionInfoCE-r13	OPTIONAL,	-- Need OP

bandwidthReducedAccessRelatedInfo-r13	SEQUENCE {

si-WindowLength-BR-r13					ENUMERATED {

ms20, ms40, ms60, ms80, ms120,

ms160, ms200, spare},

si-RepetitionPattern-r13				ENUMERATED {everyRF, every2ndRF, every4thRF,

every8thRF},

schedulingInfoList-BR-r13				SchedulingInfoList-BR-r13	OPTIONAL,	-- Cond SI-BR

fdd-DownlinkOrTddSubframeBitmapBR-r13	CHOICE {

subframePattern10-r13					BIT STRING (SIZE (10)),

subframePattern40-r13					BIT STRING (SIZE (40))

}																	OPTIONAL, 	-- Need OP

fdd-UplinkSubframeBitmapBR-r13			BIT STRING (SIZE (10)) 		OPTIONAL, 	-- Need OP

startSymbolBR-r13						INTEGER (1..4),

si-HoppingConfigCommon-r13				ENUMERATED {on,off},

si-ValidityTime-r13						ENUMERATED {true}	OPTIONAL,			-- Need OP

systemInfoValueTagList-r13				SystemInfoValueTagList-r13	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond BW-reduced

nonCriticalExtension						SystemInformationBlockType1-v1320-IEs	OPTIONAL

}

SystemInformationBlockType1-v1320-IEs ::=	SEQUENCE {

freqHoppingParametersDL-r13				SEQUENCE {

mpdcch-pdsch-HoppingNB-r13				ENUMERATED {nb2, nb4}		OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13					ENUMERATED {int1, int5, int10, int20}

}																	OPTIONAL,	-- Need OR

interval-DLHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13					ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13					ENUMERATED { int5, int10, int20, int40}

}																	OPTIONAL,	-- Need OR

mpdcch-pdsch-HoppingOffset-r13			INTEGER (1..maxAvailNarrowBands-r13)	OPTIONAL	-- Need OR

}																OPTIONAL,	-- Cond Hopping

nonCriticalExtension						SystemInformationBlockType1-v1350-IEs					OPTIONAL

}

SystemInformationBlockType1-v1350-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-r13				CellSelectionInfoCE1-r13	OPTIONAL,	-- Need OP

nonCriticalExtension					SystemInformationBlockType1-v1360-IEs				OPTIONAL

}

SystemInformationBlockType1-v1360-IEs ::=	SEQUENCE {

cellSelectionInfoCE1-v1360				CellSelectionInfoCE1-v1360	OPTIONAL, 	-- Cond QrxlevminCE1

nonCriticalExtension						SystemInformationBlockType1-v1430-IEs		OPTIONAL

}

SystemInformationBlockType1-v1430-IEs ::=	SEQUENCE {

eCallOverIMS-Support-r14				ENUMERATED {true}			OPTIONAL,	-- Need OR

tdd-Config-v1430						TDD-Config-v1430			OPTIONAL,	-- Cond TDD-OR

cellAccessRelatedInfoList-r14			SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF

CellAccessRelatedInfo-r14	OPTIONAL,	-- Need OR

nonCriticalExtension					SystemInformationBlockType1-v1450-IEs				OPTIONAL

}

SystemInformationBlockType1-v1450-IEs ::=	SEQUENCE {

tdd-Config-v1450						TDD-Config-v1450		OPTIONAL,	-- Cond TDD-OR

nonCriticalExtension					SystemInformationBlockType1-v16xx-IEs					OPTIONAL

}

SystemInformationBlockType1-v16xx-IEs ::=	SEQUENCE {

tempSIDensCoeff							ENUMERATED {n1, n2, n3, n4}	OPTIONAL,

tempSIDensPeriod						ENUMERATED {n1, n2, n3, n4}	OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

PLMN-IdentityList ::=					SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellReservedForOperatorUse				ENUMERATED {reserved, notReserved}

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SchedulingInfoList-BR-r13 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-BR-r13

SchedulingInfo-BR-r13 ::=	SEQUENCE {

si-Narrowband-r13				INTEGER (1..maxAvailNarrowBands-r13),

si-TBS-r13						ENUMERATED {b152, b208, b256, b328, b408, b504, b600, b712,

b808, b936}

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310, sibType21-v1430}

SystemInfoValueTagList-r13 ::=		SEQUENCE (SIZE (1..maxSI-Message)) OF SystemInfoValueTagSI-r13

SystemInfoValueTagSI-r13 ::=		INTEGER (0..3)

CellSelectionInfo-v920 ::=			SEQUENCE {

q-QualMin-r9						Q-QualMin-r9,

q-QualMinOffset-r9					INTEGER (1..8)						OPTIONAL	-- Need OP

}

CellSelectionInfo-v1130 ::=			SEQUENCE {

q-QualMinWB-r11						Q-QualMin-r9

}

CellSelectionInfo-v1250 ::=			SEQUENCE {

q-QualMinRSRQ-OnAllSymbols-r12						Q-QualMin-r9

}

CellAccessRelatedInfo-r14 ::=	SEQUENCE {

plmn-IdentityList-r14				PLMN-IdentityList,

trackingAreaCode-r14				TrackingAreaCode,

cellIdentity-r14					CellIdentity

}

-- ASN1STOP

| SystemInformationBlockType1 field descriptions |
| --- |
| bandwithReducedAccessRelatedInfo Access related information for BL UEs and UEs in CE. NOTE 3. |
| category0Allowed The presence of this field indicates category 0 UEs are allowed to access the cell. |
| cellAccessRelatedInfoList This field contains a list allowing signalling of access related information per PLMN. One PLMN can be included in only one entry of this list. NOTE 4. |
| cellBarred barred means the cell is barred, as defined in TS 36.304 [4]. |
| cellIdentity Indicates the cell identity. NOTE 2. |
| cellReservedForOperatorUse As defined in TS 36.304 [4]. |
| cellSelectionInfoCE Cell selection information for BL UEs and UEs in CE. If absent, coverage enhancement S criteria is not applicable. NOTE 3. |
| cellSelectionInfoCE1 Cell selection information for BL UEs and UEs in CE supporting CE Mode B. E-UTRAN includes this IE only if cellSelectionInfoCE is present in SystemInformationBlockType1-BR. NOTE 3. |  |
| csg-Identity Identity of the Closed Subscriber Group the cell belongs to. |
| csg-Indication If set to TRUE the UE is only allowed to access the cell if it is a CSG member cell, if selected during manual CSG selection or to obtain limited service, see TS 36.304 [4]. |
| eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2. |
| eDRX-Allowed The presence of this field indicates if idle mode extended DRX is allowed in the cell. The UE shall stop using extended DRX in idle mode if eDRX-Allowed is not present. |
| fdd-DownlinkOrTddSubframeBitmapBR The set of valid subframes for FDD downlink or TDD transmissions, see TS 36.213 [23]. If this field is present, SystemInformationBlockType1-BR-r13 is transmitted in RRCConnectionReconfiguration, and if RRCConnectionReconfiguration does not include systemInformationBlockType2Dedicated, UE may assume the valid subframes in fdd-DownlinkOrTddSubframeBitmapBR are not indicated as MBSFN subframes. If this field is not present, the set of valid subframes is the set of non-MBSFN subframes as indicated by mbsfn-SubframeConfigList. If neither this field nor mbsfn-SubframeConfigList is present, all subframes are considered as valid subframes for FDD downlink transmission, all DL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD DL transmission, and all UL subframes according to the uplink-downlink configuration (see TS 36.211 [21]) are considered as valid subframes for TDD UL transmission. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| fdd-UplinkSubframeBitmapBR The set of valid subframes for FDD uplink transmissions for BL UEs, see TS 36.213 [23]. If the field is not present, then UE considers all uplink subframes as valid subframes for FDD uplink transmissions. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10. Value 0 in the bitmap indicates that the corresponding subframe is invalid for transmission. Value 1 in the bitmap indicates that the corresponding subframe is valid for transmission. |
| freqBandIndicatorPriority If the field is present and supported by the UE, the UE shall prioritize the frequency bands in the multiBandInfoList field in decreasing priority order. Only if the UE does not support any of the frequency band in multiBandInfoList, the UE shall use the value in freqBandIndicator field. Otherwise, the UE applies frequency band according to the rules defined in multiBandInfoList. NOTE 2. |
| freqBandInfo A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency band in freqBandIndicator. If E-UTRAN includes freqBandInfo-v10l0 it includes the same number of entries, and listed in the same order, as in freqBandInfo-r10. |
| freqHoppingParametersDL Downlink frequency hopping parameters for BR versions of SI messages, MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH. If not present, the UE is not configured downlink frequency hopping. |
| hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. |
| ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode. NOTE 2. |
| intraFreqReselection Used to control cell reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 36.304 [4]. NOTE 2. |
| multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42, table 5.5-1] that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE. |
| multiBandInfoList-v10j0 A list of additionalPmax and additionalSpectrumEmission values, as defined in TS 36.101 [42, table 6.2.4-1] for UEs neither in CE nor BL UEs and TS 36.101 [42, table 6.2.4E-1] for UEs in CE or BL UEs, for the frequency bands in multiBandInfoList (i.e. without suffix) and multiBandInfoList-v9e0. If E-UTRAN includes multiBandInfoList-v10j0, it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). If E-UTRAN includes multiBandInfoList-v10l0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList-v10j0. |
| plmn-IdentityList List of PLMN identities. The first listed PLMN-Identity is the primary PLMN. NOTE 2. |
| p-Max Value applicable for the cell. If absent the UE applies the maximum power according to the UE capability. NOTE 2. |
| q-QualMin Parameter “Qqualmin” in TS 36.304 [4]. If cellSelectionInfo-v920 is not present, the UE applies the (default) value of negative infinity for Qqualmin. NOTE 1. |
| q-QualMinRSRQ-OnAllSymbols If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, perform RSRQ measurement on all OFDM symbols in accordance with TS 36.214 [48]. NOTE 1. |
| q-QualMinOffset Parameter “Qqualminoffset” in TS 36.304 [4]. Actual value Qqualminoffset = field value [dB]. If cellSelectionInfo-v920 is not present or the field is not present, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell. |
| q-QualMinWB If this field is present and supported by the UE, the UE shall, when performing RSRQ measurements, use a wider bandwidth in accordance with TS 36.133 [16]. NOTE 1. |
| q-RxLevMinOffset Parameter Qrxlevminoffset in TS 36.304 [4]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell. |
| sib-MappingInfo List of the SIBs mapped to this SystemInformation message. There is no mapping information of SIB2; it is always present in the first SystemInformation message listed in the schedulingInfoList list. |
| si-HoppingConfigCommon Frequency hopping activation/deactivation for BR versions of SI messages and MPDCCH/PDSCH of paging. |
| si-Narrowband This field indicates the index of a narrowband used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.211 [21, 6.4.1] and TS 36.213 [23, 7.1.6]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21]. |
| si-RepetitionPattern Indicates the radio frames within the SI window used for SI message transmission. Value everyRF corresponds to every radio frame, value every2ndRF corresponds to every 2 radio frames, and so on. The first transmission of the SI message is transmitted from the first radio frame of the SI window. |
| si-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on. |
| si-TBS This field indicates the transport block size information used to broadcast the SI message towards BL UEs and UEs in CE, see TS 36.213 [23, Table 7.1.7.2.1-1] for a 6 PRB bandwidth and a QPSK modulation. |
| schedulingInfoList-BR Indicates additional scheduling information of SI messages for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| si-ValidityTime Indicates system information validity timer. If set to TRUE, the timer is set to 3h, otherwise the timer is set to 24h. |
| si-WindowLength, si-WindowLength-BR Common SI scheduling window for all SIs. Unit in milliseconds, where ms1 denotes 1 millisecond, ms2 denotes 2 milliseconds and so on. In case si-WindowLength-BR-r13 is present and the UE is a BL UE or a UE in CE, the UE shall use si-WindowLength-BR-r13 and ignore the original field si-WindowLength (without suffix). UEs other than BL UEs or UEs in CE shall ignore the extension field si-WindowLength-BR-r13. |
| startSymbolBR For BL UEs and UEs in CE, indicates the OFDM starting symbol for any MPDCCH, PDSCH scheduled on the same cell except the PDSCH carrying SystemInformationBlockType1-BR, see TS 36.213 [23]. Values 1, 2, and 3 are applicable for dl-Bandwidth greater than 10 resource blocks. Values 2, 3, and 4 are applicable otherwise. |
| systemInfoValueTagList Indicates SI message specific value tags for BL UEs and UEs in CE. It includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix). |
| systemInfoValueTagSI SI message specific value tag as specified in subclause 5.2.1.3. Common for all SIBs within the SI message other than MIB, SIB1, SIB10, SIB11, SIB12 and SIB14. |
| systemInfoValueTag Common for all SIBs other than MIB, MIB-MBMS, SIB1, SIB1-MBMS, SIB10, SIB11, SIB12 and SIB14. Change of MIB, MIB-MBMS, SIB1 and SIB1-MBMS is detected by acquisition of the corresponding message. |
| tdd-Config Specifies the TDD specific physical channel configurations. NOTE 2. |
| tempSIDensCoeff A factor according to which the scheduling of System Information is densified during temporary SI densification period, such as upon SI modification. During SI densification period, the repetition periodicity configured by si-RepetitionPattern is divided by the factor indicated by this parameter. n1 corresponds to a factor of 1, n2 corresponds to a factor of 2, n3 corresponds to a factor of 3 and n4 corresponds to a factor of 4. |
| tempSIDensPeriod Number of modification periods that temporary SI densification configuration is applied, starting from the modification period where SI is modified. n1 corresponds to 1 modification period, n2 corresponds to 2 modification periods, n3 corresponds to 3 modification periods and n4 corresponds to 4 modification periods. |
| trackingAreaCode A trackingAreaCode that is common for all the PLMNs listed. NOTE2. NOTE 5. |

<End of modified section>

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Temporary SI densification for LTE-M/NB-IoT UEs is supported.

Proposal 2	RAN2 to adopt the text proposal including tempSIDensCoeff.