3GPP TSG RAN WG2 Meeting #104                                                       R2-1816753

Spokane, USA, 12th - 16th November 2018

Agenda item:	9.7.1

Source:		Intel Corporation (rapporteur)

Title:  		Email discussion report on [103bis#43][ LTE/eLTE]] Capture NR agreements (Intel)

Document for:	Discussion and decision

# Introduction

This is the email discussion report on [103bis#43][LTE/5GC] Capture NR agreements (Intel).

[103bis#43][LTE/eLTE] Capture NR agreements (Intel)

Capture the agreements made in NR session which are also applied for eLTE

Intended outcome: 36.331 CR submitted to next meeting

Deadline:  Thursday 2018-11-01

In this email discussion, Rapporteur lists corresponding in principle agreed CRs for NR, and would like to  collect views from companies on whether all related NR agreements are applicable for LTE/5GC , and then provide TS36.331 CR for eLTE (maybe also TS36.304 CR). Rapporteur would suggest to have two phases discussion:

Phase 1: Companies are invited to provide your view on whether each NR agreements are applicable for LTE/5GC or not; Deadline for phase 1: Friday 2018-10-25 Rapporteur may miss some agreements, companies are invited to add if any in section 3.

Phase 2: Companies are invited to provide comments on stage 3 TS36.331 CR; Deadline for Phase 2:  Thursday 2018-11-01

# NR agreements which are related to LTE/5GC

Note: NR agreements which have been captured in LTE/5GC in the meeting, are listed in section 5 Annex.

| # | Agreements/Tdoc number | Companies are invited to provide view on whether agreements are applicable for LTE/5GC or not |
| --- | --- | --- |
| 1 | R2-1816056 CR on fallback to the setup procedure R2-1816052 RIL I556, I557, I558 on RB handling when resuming - discard the suspendConfig if stored - discard the AS security context - discard radio resources and RRC configuration for all RBs except SRB0 TS36.331 5.3.3.4, the handling on INACTIVE shall be updated “1> if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE: ” | [QC] Yes, applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable [Intel] Yes, it is applicable. [Docomo]Yes, it is applicable for eLTE. |
| 2 | R2-1815834 CR to 38.331 on the ambiguity of targetCellIdentity in Resume/Reestablishment MAC-I calculation TS36.331 7.1 Variable for resume/reestablishment in case of E-UTRA/5GC should be updated, e.g. VarShortINACTIVE-MAC-Input. | [MTK] Our understanding is that we have only one cellIdentity in the system information in LTE (even in share NW case). The CR seems not necessary. [QC] eLTE introduced cellID per PLMN. So, this CR is applicable for eLTE as well. [ZTE]For LTE/5GC, cell ID per PLMN is broadcast in system information, this CR is applicable for LTE/5GC. And even for LTE/EPC, the cell ID per PLMN is broadcast. [Huawei, HiSilicon] This clarification seems not needed. In section 5.2.2, upon receiving SIB1, the UE shall: “3> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList-5GC containing the selected PLMN; ”, there is no ambiguity of CellIdentity used in VarShortINACTIVE-MAC-Input. [Ericsson] In LTE/5GC (and in fact also in LTE/EPC since rel-14) the cell identity is per PLMNs so this change is needed. Regarding Huawei’s comment above, it seems they are overlooking the fact that the selected PLMN is not known to the ng-eNB at reception of MSG3. If the cell identity associated with the selected PLMN is used, the target ng-eNB would need to forward all cell identities over Xn to the source ng-eNB for the token verification. This was the reason why we agreed to use the first PLMN for NR rather than the selected PLMN. [LG] We have same understanding with Ericsson, so this agreement is needed in eLTE. [ZTE]Our CR R2-1815554 and R2-1815632 addressing this issue were agreed in principle last meeting. R2-1815554 is for Rel-14 VarShortResumeMAC-Input while R2-1815632 is for VarShortINACTIVE-MAC-Input. Maybe the rapporteur can also merge our agreed CRs in this e-mail discussion. [Nokia] Yes, it is applicable. We think that the first cell identity of the "5GC" PLMN list should be used here. [Intel] Yes, it is applicable. As commented by ZTE, the corresponding CRs have been agreed in parallel session in R2-1815554 and R2-1815632. We do not need to capture it again. [Docomo] Yes, the cell ID broadcast in system information is per PLMN, cell ID is associated with the first PLMN ID shall be clarified. |
| 3 | R2-1815835 Handling on security keys for resume procedure (I774) TS36.331 Handling of keys/NCC shall be updated; | [MTK] Yes, we think that this is applicable to eLTE [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes. [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC. |
| 4 | R2-1814961 Clarifications on RNA update and CN registration (N023) TS36.331 Handling on SIB1 and RNAU shall be updated; | [MTK] Yes, we think that this is applicable to eLTE [QC] Yes , applicable for eLTE as well [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 5 | R2-1815836 CR to 38.331 on pendingRNAUpdate setting TS36.331 Handling on variable pendingRNAUpdate | [MTK] Yes, we think that this is applicable to eLTE. It is better to use same principle to update the pendingRnaUpdate. [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 6 | R2-1815838 Handling of Resume TS36.331, In 5.3.3, add For INACTIVE UE, the UE shall enter IDLE if the UE is unable to comply with (part of) the configuration included in the RRCResume message | [MTK] Yes, we think that this is applicable to eLTE [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 7 | R2-1815952 E573 Configuration of SRB1 during Resume TS36.331, 5.3.3.3a PDCP configuration shall not be restored before Reception of the RRCResume. restore the RRC configuration and security context from the stored UE AS context except physical layer and MAC configuration; | [MTK] Yes, we think that this is applicable to eLTE [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 8 | R2-1815958 Security for RRC connection release TS36.331, For E-UTRA/5GC, UE shall release the RRC connection but ignore the Re-direction, Dedicated frequency reselection priorities, De-prioritisation information, and SuspendConfig fields if included in an RRC release message received before security activation. | [MTK] Yes, we think that this is applicable to eLTE. [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 9 | R2-1815959 RRC connection release triggered by upper layers [H323] TS36.331, For E-UTRA/5GC, reference should be 24.501, in addition, the changes in 5.3.9 is applicable for E-UTRA/5GC | [MTK] Yes, we think that this is applicable to eLTE [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 10 | R2-1815961 CR to 38.304 on Release and Redirect in 2-step procedure TS36.304 shall be updated. 5.2.7 Cell Selection when leaving RRC_CONNECTED state | [MTK] Yes, we think that this is applicable to eLTE [QC] Agree to update [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 11 | R2-1815992 Remain issue for T302 and T390 TS36.331 5.3.3.8, handling on reject is missing, i.e. stop the timer first, and start it if configured; | [MTK] Yes, we think that this is applicable to eLTE. [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 12 | R2-1815993 CR to 38.331 on stopping T390 and UE related actions TS36.331 Handling on T390 can be aligned. | [MTK] Yes, we think that this is applicable to eLTE. [QC] Yes , applicable for eLTE as well [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Yes, it is applicable for LTE-5GC. [Ericsson] Yes, applicable for LTE/5GC. [LG] Yes [Nokia] Yes, it is applicable. [Intel] Yes, it is applicable. [Docomo] Yes, it is applicable for LTE/5GC |
| 13 | R2-1815837 CR for 38.331 for UE AS context Al2 has been postponed to next meeting. Rapporteur assume similar changes are needed for eLTE; | [MTK] In NR, the discussion is postpone. Maybe we could wait the result of NR, than decide what to do in eLTE. [QC] wait for NR discussion to complete [ZTE]Waiting for NR progress. [Huawei, HiSilicon] Agree with Rapp. [Ericsson] Agree with rapporteur. [LG] Agree with Rapp [Nokia] Wait for NR outcome [Intel] Wait for NR. [Docomo] wait for next meeting’s NR outcome |
| 14 | R2-1813624 On UE behaviour upon going out of service, CRs on NR will be provided by Samsung in next meeting. We could align with them if any. | [MTK] Agree to align NR on OOS behavior. [QC] agree to align with NR [ZTE]Yes, applicable for LTE/5GC. [Huawei, HiSilicon] Agree with Rapp. [Ericsson] Agree with rapporteur. [LG] Agree with Rapp [Nokia] Agree to align with NR [Intel] Wait for NR. [Docomo] wait for next meeting’s NR outcome |

Based on inputs from companies:

Following CRs should be captured into TS36.331 for E-UTRA/5GC:

R2-1816056, R2-1815835, R2-1814961,  R2-1815836, R2-1815838, R2-1815952, R2-1815958, R2-1815959, R2-1815992, R2-1815993;

Following CR should be captured into TS36.304 for E-UTRA/5GC:

R2-1815961;

Following CRs should be captured into TS36.331 for E-UTRA/5GC once NR has agreements:

R2-1815837, R2-1813624;

# Any other issues?

Companies are invited to provide if anything is missing.

Anything related to LTE/5GC?

| Company's name | NR agreements/CRs | Remark |
| --- | --- | --- |
| Intel | The changes on HandoverPreparationInformation-v1530-IEs from R2-1813139 is not captured in specification TS36.331 15.3.0. | Capture it back together with other changes. |

In addition, the changes on HandoverPreparationInformation-v1530-IEs from R2-1813139 should be captured in TS36.331.

# Email discussion report

Following CRs should be captured into TS36.331 for E-UTRA/5GC:

R2-1816056, R2-1815835, R2-1814961,  R2-1815836, R2-1815838, R2-1815952, R2-1815958, R2-1815959, R2-1815992, R2-1815993;

Following CR should be captured into TS36.304 for E-UTRA/5GC:

R2-1815961;

Following CRs should be captured into TS36.331 for E-UTRA/5GC once NR has agreements:

R2-1815837, R2-1813624;

In addition, the changes on HandoverPreparationInformation-v1530-IEs from R2-1813139 should be captured in TS36.331.

# Annex

## NR agreements which have been captured in LTE/5GC

| # | NR Agreements/Tdoc number | eLTE Agreements/Tdoc number |
| --- | --- | --- |
| 1 | R2-1815822 clarification of guami-Type | R2-1815859 [E201] CR to 36.331 on handling of mapped GUMMEI at idle mode mobility from 5GS to EPS |
| 2 | R2-1815962 Correction on CN type indication for Redirection from NR to E-UTRA | R2-1815787 Correction of CN type indication for RRC Redirection from E-UTRA/5GC to E-UTRA/5GC or E-UTRAN |
| 3 | R2-1816047 Correction to full configuration | R2-1816008 Correction to DRB release |
| 4 | R2-1813968 Handling on simultaneously triggered NAS/AS events (I770) | R2-1813976 Open issues on E-UTRA connected to 5GC for UAC |
| 5 | R2-1813823 [C204] Handling of timer T380 | R2-1815789 TS36.331 CR on Open issues on E-UTRA connected to 5GC for INACTIVE |
| 6 | R2-1816029 Addition of RAN specific Access Category | R2-1813643 Addition of RAN specific Access Category |
| 7 | R2-1815834 CR to 38.331 on the ambiguity of targetCellIdentity in Resume/Reestablishment MAC-I calculation TS36.331 7.1 Variable for resume/reestablishment in case of E-UTRA/5GC should be updated, e.g. VarShortINACTIVE-MAC-Input. | R2-1815554 and R2-1815632 |