3GPP TSG-RAN WG2 #104	R2-1817250

Spokane, United States, 12th – 16th Nov 2018

Agenda Item:	11.7.3

Source:	OPPO

Title:	Intra-UE Prioritization for IIoT

Document for:	Discussion, Decision

Introduction

In RAN #81, a revised SID on Study on NR Industrial Internet of Things (IoT) is approved. The potential enhancements on UL/DL intra-UE prioritization are discussed and captured in this study item, as shown in the following [1]:

b)	UL/DL intra-UE prioritization/multiplexing, i.e. prioritization (for example dropping, delaying or puncturing lower priority service) between different categories of traffic in the UE, including both data and control channels and considering (RAN2/RAN1):

i)	Different latency and reliability requirements

ii)	Different types of resource allocation for example grant-free and grant-based allocations

Based on the agreement in RAN2#103bis, an email discussion on this topic is approved, which focus on scenarios discussion and the related work division between RAN1 and RAN2.

[103bis#41][NR/IIoT] Intra-UE prioritization (Nokia)

Intended outcome: Email discussion report

Discuss the scenarios and division of work between RAN1 and RAN2. Views from companies to be presented in email discussion.

Deadline:  Thursday 2018-11-02

In this paper, we will further discuss RAN2 related issue on intra-UE prioritization and give our proposals.

Discussion

Ten potential scenarios are raised in email discussion for intra-UE prioritization, and we propose to consider Scenario 2 (i.e. intra-UE UL prioritization for configured and dynamic grants conflict case) and Scenario 5(i.e. intra-UE UL prioritization for control channel and data channel conflict case) in RAN2 scope.

Resource conflict on configured and dynamic grants in uplink

In current specification, the prioritization of dynamic grant is higher than configured grant. With this restriction, if the available uplink data is not suitable for multiplexing into MAC PDU associated to dynamic grant, the QoS of such traffic may not be satisfied. NR URLLC traffic pattern is deterministic, the delay of message delivery is bounded and the message is arrived in the fixed window, therefore, configured grant is a useful feature for NR URLLC. If this restriction is still existed, it is not always possible to protect URLLC transmissions when they collide with eMBB traffic. Thus, to satisfy URLLC KPIs, we propose to reconsider the restriction on dynamic grant prioritization.

When configured grant is for URLLC traffic, it is not always preferred to prioritize dynamic grant than configured grant.

There are two cases for configured grant and dynamic grant overlapping in uplink:

- Case1: Resource overlapping without URLLC traffic available.

In this case, configured grant and dynamic grant is overlapping or partially overlapping in time domain, but there is no available data associated to URLLC in UE buffer. Since there is no available traffic with such stringent QoS as URLLC, waiting for the next occasion of configured grant or assembling into MAC PDU associated to dynamic grant are acceptable for UE. Thus, it seems no need to prioritize configured grant.

- Case2: Resource overlapping with URLLC traffic available

In this case, configured grant and dynamic grant is overlapping or partially overlapping in time domain, and there is available data associated to URLLC in UE buffer. Considering such stringent QoS requirement for URLLC traffic, it is not tolerable for waiting for the next occasion of configured grant or assembling in MAC PDU associated to dynamic grant with a longer PUSCH duration. Thus, it is necessary to prioritize configured grant associated with URLLC traffic. In other hand, if the dynamic grant is for URLLC traffic but configured grant is not for URLLC traffic, follow current rule is preferred.

Prioritize configured grant/dynamic grant is based on whether URLLC traffic is available and which grant is configured for URLLC.

Prioritize configured grant than dynamic grant in case configured grant is for URLLC traffic and URLLC traffic is available in the buffer, otherwise dynamic grant is prioritized as in legacy case.

Resource conflict on control channel and data channel in uplink

In TS 38.321, SR is transmitted only when PUCCH resource for SR transmission is not overlapped with a UL-SCH resource. However, if PUCCH for SR triggered by URLLC traffic is overlapped in time with PUSCH related to traffic with the lower priority, the latency requirement for URLLC may not be satisfied since UE should wait for the next occasion for URLLC resource request.

SR cannot be transmitted when PUCCH resource for the SR transmission occasion overlaps with a UL-SCH resource, even for the case SR is triggered by URLLC and PUSCH is for the traffic with lower priority.

We think there are two possible cases: 1. SR transmission overlaps with UL-SCH on one cell, 2. SR transmission overlaps with UL-SCH on different cell. It is RAN1 scope if PUCCH and PUSCH can be transmitted simultaneously (e.g. case 2) or UCI multiplexing method can satisfy URLLC latency requirement. Otherwise, it is in RAN2 scope whether to prioritize PUCCH for SR triggered by URLLC traffic than PUSCH.

As mentioned above, if PUCCH for SR triggered by URLLC traffic is overlapped with PUSCH for equal/higher priority traffic, waiting for the next PUCCH occasion is acceptable as the QoS requirement is loose. And if PUCCH for SR triggered by URLLC traffic is overlapped with PUSCH for lower priority traffic, waiting for the next PUCCH occasion is not acceptable as the QoS requirement is very stringent. Thus, it is better to prioritize SR which is triggered by URLLC.

May not cancel SR when PUCCH is overlapped with PUSCH if the SR is triggered by URLLC and the PUSCH is for lower priority traffic.

Conclusion

Based on the discussion above, we made the following observations:

Observation 1	When configured grant is for URLLC traffic, it is not always preferred to prioritize dynamic grant than configured grant.

Observation 2	SR cannot be transmitted when PUCCH resource for the SR transmission occasion overlaps with a UL-SCH resource, even for the case SR is triggered by URLLC and PUSCH is for the traffic with lower priority.

And propose the following:

Proposal 1	Prioritize configured grant/dynamic grant is based on whether URLLC traffic is available and which grant is configured for URLLC.

Proposal 2	Prioritize configured grant than dynamic grant in case configured grant is for URLLC traffic and URLLC traffic is available in the buffer, otherwise dynamic grant is prioritized as in legacy case.

Proposal 3	May not cancel SR when PUCCH is overlapped with PUSCH if the SR is triggered by URLLC and the PUSCH is for lower priority traffic.

References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018

TR 22.804, Study on Communication for Automation in Vertical Domains

R2-1816043, LS on TSN requirements evaluation, contact: Nokia