3GPP TSG RAN WG2 #104	 R2-1817400

Spokane, US, 12 – 16 October, 2018    	(Resubmission of R2-1814320)

Agenda Item:	12.3.3 Handover robustness improvements

Source: 	Ericsson

Title:  	Simulation results for conditional handover in LTE

Document for:	Discussion and Decision

# Introduction

In the work item for Even further Mobility Enhancements in E-UTRAN [1], one objective is to improve the robustness at handover. In this contribution simulation results for conditional handover are shown.

# Discussion

## Setup

The conditional handover as presented in [2] is the basis for the simulations. The following call flow was used in the simulations:

This procedure was compared with the legacy procedure executed at handover:

UE

Serving gNB

Target gNB

HO decision

UP data

1. Measurement report

2. HO Request

Accept HO, build RRC config

3. HO Ack (incl. RRC config)

4. HO command

UE accesses the new cell

5. Synchronization and random access

6. HO confirm

7. HO completed

UP  data

The A3 event was used to trigger the conditional handover. Simulations were performed for low, medium and high speed UEs. The threshold for triggering the conditional handover was set lower compared to triggering of legacy handover. The following setups were used:

The legacy LTE procedure is called setup 1 and the conditional handover procedure is called setup 2.

| Simulation parameter | Parameter value |
| --- | --- |
| Deployment | 3gpp-case1.7 sites,3 cells, @30 m high, 80W, Cell Radius 166.666, ISD=500 m |
| eNB Antenna | Antenna 3gpp36873, 8 elements, 1 row, 0.7ƛ separation |
| Carrier Frequency | 2 GHz, 20 MHz carrier BW |
| Propagation model | SCM – {5G-urban macro} |
| User properties | 190 initial users (950 UEs over 5 Seeds) |
| User session | FTP, mean file size 400000 bytes, ftp request size 400 bytes, max number of downloads: 3 |
| Handover Parameters | A3 event, Threshold{-3:4 dB}, Hysteresis {0dB}, TTT{0.04}, Handover issue threshold=-1dB in Setup 2 (conditional handover). |
| Number of Sub-bands | 100 |

- Conditional Handover issuing event threshold set to -1dB.

  - Controls when the conditional Handover command is issued to the UE.

  - Possible for network to add/modify target cell(s).

- Handover execution threshold varied from -3dB to 4dB.

  - Controls when the actual handover is initiated.

## Simulation result for low/medium speed UEs

The first simulation was done for UEs moving at a speed 0,8 m/s. At this low speed the number of failed handovers was very low for both legacy handover and conditional handover so no comparison could be made. The next simulation was done for UEs moving at a speed of 5 m/s. Also at this speed the number of failed handover was quite low for both types of handovers, but some difference could be seen.

Although the number of failed handovers is still small for both types of handovers, the following comparison of handover failure performance at 4dB handover threshold is made:

- ~50% reduction in total number of failed handovers.

- Largest improvement (~85% reduction) in failure due to Handover command not reaching the UE.

- Improvements in RLF performance.

## Simulation result for high speed UEs

The simulations for high speed UEs were done at the speed 30 m/s. At this speed the number of failed handovers is larger. The results are shown below.

At this rate a more reliable comparison can be made and the following comparison of handover failure performance at 4dB handover threshold is made:

- ~70% reduction in total number of failed handovers.

- Largest improvement in failure due to Handover command not reaching to UE.

- Improvements in RLF performance.

## Conclusion

Based on the results above it can be concluded that conditional handover is beneficial.

Observation 1: Conditional handover is beneficial to reduce the number of failed handover.

# Summary

RAN2 is kindly asked to discuss the following observation:

Observation 1: Conditional handover is beneficial to reduce the number of failed handover.

# References

- RP-171489, Work Item on Even further Mobility Enhancements in E-UTRAN, China Telecom, RAN#81, September 2018

- R2-1817399, Conditional handover in LTE, Ericsson