3GPP TSG-RAN WG2 Meeting #104	R2-1817412

Spokane, USA, November 12-16, 2018

Source: 	Document Rapporteur (BlackBerry)

Title:	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC

Agenda Item:	12.1.1 and 12.2.1

Document for:	Information

# Introduction

This document lists RAN2 agreements made for Rel-16 Additional enhancements for NB-IoT (WI code NB_IOTenh3-Core; WID in RP-181674) and Rel-16 Additional MTC enhancements for LTE (WI code LTE_eMTC5-Core, WID in RP-181878), until and including the RAN2#103bis, 08-12 October 2018.

# 1	Additional MTC enhancements for LTE

## 1.1	Organisational

RAN2#103bis agreements: To capture the agreements from all Rel-16 IoT meetings in the same document as agreed in Rel-16 NB-IoT session. The draft can be provided in R2-1815744. [103bis#xx][ eMTC/NB-IoT R16] To capture the agreements from the meeting in a document for information. (Blackberry) Intended outcome: Endorsed report Deadline: Thursday 2018-10-19

## 1.2	Mobile-terminated (MT) early data transmission (EDT)

Treated jointly with 2.2 under this subclause.

RAN2#103bis agreements: RAN2 intends to support MT-initiated EDT for both CP and UP solutions. The intention to use MT-EDT is for user data, i.e. not for NAS signalling.

## UE-group wake-up signal (WUS)

See subclause 2.3 where this topic is treated jointly.

## Transmission in preconfigured resources

See subclause 2.4 where this topic is treated jointly.

## Scheduling multiple DL/UL transport blocks

Treated jointly with 2.5 under this subclause.

RAN2#103bis agreements: Working assumption: For MTCH in SC-PTM, configuration for multiple scheduling is transmitted in MCCH. Backwards compatibility is FFS. SPS is not supported for NB-IoT in Release 16. Enhancements for “SR with SPS for BSR” can be considered.

## 1.6	Quality report in Msg3

RAN2#103bis agreements: Quality report in Msg3 is introduced for EDT. FFS for non-EDT.

## 1.7	MPDCCH performance improvement using CRS

RAN2#103bis agreements: eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling.

## 1.8	Improvements for non-BL UEs

RAN2#103bis agreements: RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode.

## 1.9	Stand-alone deployment

RAN2#103bis agreements: The standalone deployment supports legacy operations for legacy BL UEs or UEs in CE mode. LTE control channel region for DL transmission is supported for Rel-16 BL UEs and UEs in CE mode.

## Mobility Enhancements

RAN2#103bis agreements: None.

## Coexistence with NR

RAN2#103bis agreements: None.

## Other

RAN2#103bis agreements: None.

# Additional enhancements for NB-IoT

## Organisational

RAN2#103bis agreements: To create document agreed after each meeting capturing agreements from all meetings. Copy/paste of chair notes, not intended to be a further discussion of agreements. One week email agreement after each meeting BlackBerry will be rapporteur of the document.

## Mobile-terminated (MT) early data transmission (EDT)

See subclause 1.2 where this topic is treated jointly.

## UE-group wake-up signal (WUS)

Treated jointly with 1.3 under this subclause.

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping

## Transmission in preconfigured resources

Treated jointly with 1.4 under this subclause.

RAN2#103bis agreements: Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed

## 2.5	Scheduling multiple DL/UL transport blocks

See subclause 1.5 where this topic is treated jointly.

## 2.6	Network management tool enhancement

RAN2#103bis agreements: ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) Support RACH report for NB-IoT Will consider whether information in LTE RACH report is extended

## 2.7	Improved multi-carrier operation

RAN2#103bis agreements: None.

## 2.8	Inter-RAT cell selection

RAN2#103bis agreements: RAN2 understands that cell reselection between NB-IoT and any other RAT is not within the scope of this WI NB-IoT network may indicate eMTC/LTE/GERAN assistance information for inter-RAT cell selection eMTC/LTE network may indicate NB-IoT assistance information for inter-RAT cell selection Intention is not to provide cell reselection parameters It is up to UE implementation how and when to perform measurements for cell selection of the target RAT

## 2.9	Coexistence with NR

RAN2#103bis agreements: None.

## 2.10	Other

RAN2#103bis agreements: None.

# References: Reports from breakout sessions on NB-IoT and MTC

| Meeting | Date | Title | Tdoc |
| --- | --- | --- | --- |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report NB-IoT breakout session | R2-1815603 |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report eMTC breakout session | R2-1815604 |