3GPP TSG-RAN WG2#104	R2-1817522

Spokane, USA, 12th – 16th November 2018	(Revision of R2-1814557)

Agenda Item:	10.4.1.3.11

Source:	Ericsson

Title:	UE RRC processing times for NR standalone

Document for:	Discussion, Decision

# Introduction

In RAN2#100, the following agreements were made regarding RRC processing times for EN-DC.

Agreements

1	A joint processing time requirement is defined to LTE reconfiguration message with embedded NR RRC part.

2	The processing time requirement for NR RRC received over SRB3 is defined in 38.331.

3	The processing time requirements only apply to the case where the UE receives one RRC message at a time.

4	The processing time definition and unit of LTE RRC is applied also for LTE RRC with embedded NR RRC.

5	Following aspects should be considered when deciding RRC processing times for LTE RRCConnectionReconfiguration with embedded NR RRC:

- LTE and NR ASN.1 decoding and encoding

- Configuration time including coordination between LTE and NR part of UE

- Encoding of LTE and NR complete message

- Increased complexity due to introduction of new features and RRC IEs in LTE-A and NR

- UE processing capability improvements since R8

6	The following two subcases are defined for RRCConnectionReconfiguration including NR RRC:

- RRC connection re-configuration (NR measurement configuration)

- RRC connection re-configuration (NR SCG establish/mod/release)

The processing times for RRC connection reconfiguration with embedded NR RRC were agreed in RAN2#NR-AH1801 in Vancouver, to be included in 36.331. In this paper, we continue the discussion on processing times values in 38.331 for RRC SA procedures since the aim is to include these delays in 38.331 for the December drop.

# Discussion

In [1], are discussed the NR RRC processing time for standalone NR RRC resume operation. The proposal is that a processing time of 3 ms is needed in order to reach good performance for NR RRC resume. If 3 ms is not reachable in the general case, restrictions on the content of the RRC Resume message may be applied such that the 3ms can be achieved. In this case, a processing time of 6ms could be considered for the general case.

A UE RRC processing time of 3 ms for RRC Resume message should be targeted. If necessary, restrictions may be applied to the content of the RRC resume message to reach the 3 ms. In this case, a processing time of 6 ms could be considered for the general case, whithout restriction on the content.

Now, having a look at Table 1 that is related to the UE processing times for the RRC connection control procedures in TS 36.331 (Section 11.2), it is possible to notice that the UE RRC processing time for LTE is about five time higher than the one proposed for RRC Resume in [1] for NR. For this reason, it would make sense to scale the UE RRC processing times in NR (if applicable) by a similar factor, to reach the 3 ms or 6 ms proposed on [1].

Table 1. UE performance requirements for RRC procedures for UEs in 36.331

| Procedure title: | E-UTRAN -> UE | UE -> E-UTRAN | N | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC connection establishment | RRCConnectionSetup or RRCConnectionResume | RRCConnectionSetupComplete or RRCConnectionResumeComplete | 15 |  |
| RRC connection release | RRCConnectionRelease |  | NA |  |
| RRC connection re-configuration (radio resource configuration) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 |  |
| RRC connection re-configuration (measurement configuration) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 |  |
| RRC connection re-configuration (intra-LTE mobility) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 |  |
| RRC connection reconfiguration (SCell addition/release) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 |  |
| RRC connection reconfiguration (SCG establishment/ release, SCG cell addition/ release) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 |  |
| RRC connection re-configuration (NR measurement configuration) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 |  |
| RRC connection reconfiguration (NR SCG establishment/ /modification/release) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 |  |
| RRC connection re-configuration (intra-LTE mobility with NR SCG establishment/ /modification/release) | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 |  |
| RRC connection re-establishment | RRCConnectionReestablishment | RRCConnectionReestablishmentComplete | 15 |  |
| Initial security activation | SecurityModeCommand | SecurityModeCommandComplete/SecurityModeCommandFailure | 10 |  |
| Initial security activation + RRC connection re-configuration (RB establishment) | SecurityModeCommand, RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 | The two DL messages are transmitted in the same TTI |
| Paging | Paging |  | NA |  |

This is because in the near future a tremendous amount of growth in connectivity, traffic volume and a much broader range of usage scenarios will take place. Some typical trends include explosive growth of data traffic, great increase of connected devices and continuous emergence of new services. Besides the market requirements, it is recognized that technologies for IMT-2020 can be applied in a variety of deployment scenarios and can support a range of environments, service capabilities, and technology options. For this reason, it is mandatory to ensure that NR technology is able to fulfil those expectations and objectives settled by the performance requirements of ITU-R for IMT-2020. Among this, the end-to-end latency and, in particular, the CP latency play a fundamental role.

Therefore, it is proposed to reduce the processing times of other NR RRC messages by the same factor as for the NR RRC Resume message.

It is left for discussion whether adding of possible restriction also to other RRC message types make sense, in order to support lower processing times under certain conditions.

The UE RRC processing time of NR RRC messages is reduced by similar factor as for RRC resume compared to LTE RRC processing times.

FFS whether restrictions are also applied to other RRC message types to support lower processing times under certain conditions.

Adopt the CR provided in [2] as a baseline in TS 38.331.

# Conclusion

In section 2 we made the following observations:

Observation 1	A UE RRC processing time of 3 ms for RRC Resume message should be targeted. If necessary, restrictions may be applied to the content of the RRC resume message to reach the 3 ms. In this case, a processing time of 6 ms could be considered for the general case, whithout restriction on the content.

Based on the discussion in section 2 we propose the following:

Proposal 1	The UE RRC processing time of NR RRC messages is reduced by similar factor as for RRC resume compared to LTE RRC processing times.

Proposal 2	FFS whether restrictions are also applied to other RRC message types to support lower processing times under certain conditions.

Proposal 3	Adopt the CR provided in [2] as a baseline in TS 38.331.

# References

R2-1817524, RRC UE processing time for RRCResume, Ericsson, RAN2#104, November 2018.

R2-1817523, RRC processing times for NR standalone (38.331), RAN2 #104, November 2018.