3GPP TSG-RAN2 Meeting #104 	R2-1817595

Spokane, USA, 12th – 17th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0085 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SN Initiated SN Modification procedure for Measurement Gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN3 #101bis, it has been agreed to correct the SN initiated SN modification procedure to ensure that measurement gap information can be provided from the MN in a nested MN iniated SN modification procedure within the SN iniiated SN modification procedure [R3-186196]. This CR aligns the stage 2 specification to this agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.3.1 SN initiated SN modification with SN involvement - clarified that MN initiated SN modification procedure can be triggered within SN initiated SN modification procedure to configure measuremnet gaps 10.3.2 SN initiated SN modification with SN involvement - clarified that MN initiated SN modification procedure can be triggered within SN initiated SN modification procedure to configure measuremnet gaps Impact analysis Impacted functionality: The CR has no impact on UE If a network node does not implement this, performance will be impacted because the SN will not get the measurement gap information until later when the MN triggers an SN modification later on. That is, during this time period, there could be a mismatch between the measurement gaps that the UE is using and the measurement gap information available at the SN for that UE. The drawback of this is possible throughput degradation or even some data loss (e.g. if the SN scheduled the UE when the UE is performing measurements that require gaps). Interoperability analysis: There is no interoperability impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented, possible mismatch between the measurement gaps that the UE is using and the measurement gap information available at the SN for that UE, leading to throughput degredation and data loss. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 10.3	Secondary Node Modification (MN/SN initiated)

### 10.3.1	EN-DC

<< skipped parts >>

SN initiated SN Modification with MN involvement

Figure 10.3.1-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the release of SCG bearer(s) and the SCG RLC bearer of split bearer(s) (upon which the MN may release the bearer or maintain current bearer type or reconfigure it to an MCG bearer, either MN terminated or SN terminated), and to trigger PSCell change (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). The MN cannot reject the release request of SCG bearer and the SCG RLC bearer of a split bearer. Figure 10.3.1.-2 shows an example signalling flow for an SN initiated SgNB Modification procedure, with MN involvement.

1.	The SN sends the SgNB Modification Required message including a NR RRC configuration message, which may contain bearer context related, other UE context related information and the new SCG radio resource configuration. For bearer release or modification, a corresponding E-RAB list is included in the SgNB Modification Required message. In case of change of security key, the PDCP Change Indication indicates that a S-KgNB update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required.

The SN can decide whether the change of security key is required.

2/3.	If data forwarding and/or SN security key change needs to be applied, or/and measurement gap information needs to be provided to the SN, the MN triggers the MN initiated SN Modification procedure and provides forwarding address and/or a new SN security key information and/or the measurement gap within the SgNB Modification Request message, respectively.

NOTE 2:	If only SN security key is provided in step 2, the MN does not need to wait for the reception of step 3 to initiate the RRC connection reconfiguration procedure.

4.	The MN sends the RRCConnectionReconfiguration message including a NR RRC configuration message to the UE including the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the RRCConnectionReconfigurationComplete message, including an encoded NR RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SgNB Modification Confirm message containing the encoded NR RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN sends the MN Status transfer.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.1-2 depicts the case where a bearer context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Volume Report message to the MN and includes the data volumes delivered to the UE over the NR radio for the E-RABs to be released.

NOTE 3:	The order the SN sends the Secondary RAT Data Volume Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

11.	If applicable, a path update is performed.

END OF CHANGES

START OF CHANGES

### 10.3.2	MR-DC with 5GC

<< skipped parts >>

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the modification/release of PDU session/QoS flows and to trigger PSCell changes (when MN involvement is needed for this). The MN cannot reject the release request of PDU session/QoS flows. Figure 10.3.2-2 shows an example signalling flow for SN initiated SN Modification procedure.

1.	The SN sends the SN Modification Required message including a SN RRC configuration message, which may contain PDU session/QoS Flow context related, other UE context related information and the new radio resource configuration of SCG. For the release or modification of PDU session/QoS flow, a corresponding PDU session/QoS Flows list is included in the SN Modification Required message.

The SN can decide whether the change of security key is required.

2/3.	If data forwarding and/or SN security key change needs to be applied, or/and measurement gap information needs to be provided to the SN, the MN triggers the MN initiated SN Modification procedure  and provides forwarding address and/or a new SN security key information and/or the measurement gap within the SgNB Modification Request message.

4.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC configuration message the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the MN RRC reconfiguration complete message, including an encoded SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Modification Confirm message containing the encoded SN RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell configured by the SN as described in SN Addition procedure. Otherwise, the UE may perform UL transmission directly after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN sends the MN Status transfer.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-2 depicts the case where a PDU session/QoS Flow context is transferred from the SN to the MN).

10.	If applicable, a PDU Session path update procedure is performed.

END OF CHANGES