3GPP TSG-RAN2 104                                                 R2-1818066

Spokane, USA, 12th - 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Ambiguous UE behaviour with respect to Cell Ranking_Solution 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the latest version of 38.304, the UE behaviour with respect to cell ranking for Intra-frequency and equal priority inter-frequency cell reselection is defind in 5.2.4.6. as follows: In all cases, the UE shall reselect the new cell, only if the following conditions are met: - the new cell is better ranked than the serving cell during a time interval TreselectionRAT; - more than 1 second has elapsed since the UE camped on the current serving cell.. With the above current cell reselection behaviour, NR UE will perform cell reselection to the better ranked cell, whether the ‘rangeToBestCell’ is configured or not. However, according to the current description, if rangeToBestCell is configured, the UE will perform cell reselection to the cell with the highest number of beams (above threshold) among the cells whose R value is within rangeToBestCells of the R value of the highest ranked cell. Considering one case below, it is not clear whether the UE will perform cell reselection for the selected cell or not when: - If the rangeToBestCell is configured; and - the new cell is with the highest number of beams above the threshold; and - the cell’s ranking is below the serving cell. According to the current description, the UE shall perform cell reselection to the new cell. However, according to the yellow highlighted UE behaviour, since the cell ranking is not better than the serving cell, the UE cannot perform sell reselection. In order to clearly specify the UE behaviour on the above case, the TS 38.304 needs to be updated |
| Summary of change: |  | Add UE beahviour in 5.2.4.6 Impact analysis Impacted functionality: Intra-frequency and equal priority inter-frequency Cell Reselection criteria Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - In case explained above, different UEs may perform cell reselection on different cells. - With different rangeToBestCell and absThreshSS-Consolidation values of different cells, UE may perform unnecessary cell reselection after one cell reselection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

<Start of modification 1>

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	if rangeToBestCell is not configured in the current serving cell:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

-	else if rangeToBestCell is configured in the current serving cell:

-	the new cell has higher number of beams above absThreshSS-Consolidation than the serving cell during a time interval TreselectionRAT; and

-	the new cell has R value within rangeToBestCell of the highest R value during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

<End of modification 1>