3GPP TSG-RAN WG2 Meeting #104	R2-1818308

Spokane, US, 12-16 November 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0750 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on indication for user plane resource release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc., MediaTek Inc., Ericsson, Intel Corporation, Nokia, Nokia Shanghai Bell, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Accoring to TS 23.502, a PDU session can be deactivated after the PDU session is established, and then the PDU session can be activated again. SA2 agreed that - UE’s NAS layer is aware that the established PDU session has been deactivated when receiving the indication from AS layer that the user plane resource for the PDU session is released, and - UE’s NAS layer knows that the PDU session has been activated when receiving the indication from AS layer that the user plane resource for the PDU session is established. At RAN2 #103bis meeting, indication for user plane resource establishment was captured in TS 38.331 as below: 2> if an sdap-Config is included: 3> if an SDAP entity with the received pdu-Session does not exist: 4> establish an SDAP entity as specified in TS 37.324 [xx] section 5.1.1; 4> indicate the establishment of the user plane resources for the pdu-Session to upper layers; By using this indication, NAS layer can be aware that PDU session has been activated. However, NAS layer still doesn’t know if PDU session is deactivated. Therefore, it is necessary to provide indication for user plane resource release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An indication of user plane resource release is provided to upper layer when all user plane resource for the PDU session is released. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NAS layer doesn’t know if PDU session is deactivated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.3.5.6	Radio Bearer configuration

5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb3-ToRelease and set to true:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5.

1>	release all SDAP entities, if any, that have no associated DRB as specified in TS 37.324 [xx] section 5.1.2, and indicate the release of the user plane resources for PDU Sessions associated with the released SDAP entities to upper layers.

End of change