3GPP TSG-RAN WG2#104	R2-1818394

Spokane, USA, 12th – 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG failure handling in case of NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SCGFailureInformation message is missing in the ASN.1 for the NE-DC option. Also, the procedural text needs to be updated since it covers only EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SCGFailureInformation message added in the ASN.1 for NE-DC and the procedural text has been updated accordingly. Impact analysis If the network implements the CR and the UE does not, the UE cannot trigger the SCG failure procedure and inform the network. If the UE implements the CR and the network does not, the network will not be able to decode some of the IEs within sent in the SCG failure procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented the UE can not perform the SCG failure procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TR 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.6.13	SCG failure information

#### 5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.6.13.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a; or

1>	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.17.2 of TS 36.133 [29].

In case of DC, upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	if the UE is operating in NE-DC:

2>	initiate transmission of the SCGFailureInformationEUTRA message as specified in TS 38.331 [X, 5.7.xa];

1>	else

2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

END OF CHANGES

START OF CHANGES

#### 5.6.13.x	Failure type determination in NE-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationEUTRA message due to T307 expiry:

2>	set the failureType as t307-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

#### 5.6.13.x	Setting the contents of MeasResultSCG-FailureNR

The UE shall set the contents of the MeasResultSCG-FailureNR as follows:

1>	for each MeasOjectEUTRA for which a measId is configured and measurement results are available;

2>	include rlf-ReportEUTRA in measResultsPerMOList;

END OF CHANGES

START OF CHANGES

### 6.3.5	Measurement information elements

#### –	MeasResultSCG-FailureNR

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of NE-DC.

MeasResultSCG-FailureNR information element

-- ASN1START

-- TAG-MEAS-RESULT-SCG-FAILURE-NR-START

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                     MeasResultList2EUTRA,

...

}

MeasResultList2EUTRA ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA

MeasResult2EUTRA ::=  SEQUENCE {

rlf-ReportEUTRA				RLF-ReportEUTRA  	OPTIONAL,

...

}

-- TAG-MEAS-RESULT-SCG-FAILURE-NR-STOP

-- ASN1STOP

END OF CHANGES