3GPP TSG-RAN WG2 Meeting #104	R2-1818465

# Spokane, USA, 12th - 16th November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0091 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer 2 handling for bearer type change in MR-DC. |
| --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Annex A of TS 37.340 provides an overview on L2 handling for bearer type change in EN-DC. For other MR-DC types and NR-DC, similar description for the layer 2 handling for bearer type change is necessary. In table A-1, the following principles are applied: - If an LTE RLC bearer is released due to bearer type change, it is re-established and released. - If an NR RLC bearer is released due to bearer type change, it is just released. - If the security key is changed, PDCP re-establishment is performed. - If the security key is not changed, PDCP recovery is performed. - If MAC and RLC bearer are continously used by the changed bearer, MAC/RLC behaviour depends on the solution selected by the network, as shown in Note 1 and Note 2 of table A-1. Note 1 is for LTE RLC and Note is for NR RLC. For other MR-DC and NR-DC, the EN-DC principle above can be reused. For NGEN-DC, the table should be the same as A-1. For NE-DC, the difference comes from the RLC version, i.e. MCG is NR and SCG is LTE. For NR-DC, only NR MAC/RLC is used. Modifying existing table for NE-DC and NR-DC becomes complicated to handle so separate tables are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - Description of Annex 1 to cover MR-DC - Title to cover NGEN-DC - Table A-2 for NE-DC - Table A-3 for MR-DC Impact analysis Impacted functionality: NE-DC, NGEN-DC, NR-DC Inter-operability: 1. If the UE is implemented according to this CR but the network is not, there is no backward compatibility issue. 2. If the network is implemented according to this CR but the UE is not, there is no backward compatibility issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The informative behaviour for the bearer type change in MR-DC and NR-DC is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A (informative) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of modification >

Annex A (informative):
Layer 2 handling for bearer type change

This subclause provides for information an overview on L2 handling for bearer type change in MR-DC, with and without security key change (from KeNB/KgNB to S-KgNB/S-KeNB and from S-KgNB/S-KeNB to KeNB/KgNB), i.e. with and without a change of the termination point.

Table A-1: L2 handling for bearer type change with and without security key change in (NG)EN-DC

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 2 SCG MAC: See Note 2 | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 2SCG MAC: See Note 2 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 2SCG MAC: See Note 2 |

NOTE 1:	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be handover, which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

NOTE 2:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

Table A-2: L2 handling for bearer type change with and without security key change in NE-DC

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KgNB <-> S-KeNB) | no key change | with key change (KgNB <-> S-KeNB) | no key change | with key change (KgNB <-> S-KeNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: Re-est+Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: See Note 4 SCG MAC: See Note 4 | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: See Note 4 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: See Note 4 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 4 SCG MAC: See Note 4 |

NOTE 3:	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

NOTE 4:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be SCG change, which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

Table A-3: L2 handling for bearer type change with and without security key change in NR-DC

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KgNB <-> S-KgNB) | no key change | with key change (KgNB <-> S-KgNB) | no key change | with key change (KgNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: See Note 5 SCG MAC: See Note 5 | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: See Note 5 SCG MAC: See Note 5 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 5 SCG MAC: See Note 5 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 5 SCG MAC: See Note 5 |

NOTE 5:	The MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

<End of modification >

Annex: Difference between Tables (Just for information, to be removed in the final version)

The differences of Table A2 from Table A-1 are colored as follows:

- yellow: changed to Note 3, due to NR RLC

- cyan: NR RLC is released.

- green: LTE RLC is re-established and released

- magenta: changed to Note 4, due to LTE RLC

- between the new tables compared to original could somehow be indicated

Table A-2: L2 handling for bearer type change with and without security key change in NE-DC

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KgNB <-> S-KgNB) | no key change | with key change (KgNB <-> S-KeNB) | no key change | with key change (KgNB <-> S-KeNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: Re-est+Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: See Note 3 SCG RLC: See Note 4 SCG MAC: See Note 4 | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: See Note 4 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-est+Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: See Note 4 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 4 SCG MAC: See Note 4 |

NOTE 3:	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

NOTE 4:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be SCG change, which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

The differences of Table A3 from Table A-1 are colored as follows:

- red: changed to Note 5, due to NR RLC for MCG and SCG

- blue: NR RLC is released.

Table A-3: L2 handling for bearer type change with and without security key change in NR-DC

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KgNB <-> S-KgNB) | no key change | with key change (KgNB <-> S-KgNB) | no key change | with key change (KgNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 5 MCG MAC: See Note 5 SCG RLC: See Note 5 SCG MAC: See Note 5 | PDCP: Recovery MCG RLC: release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: release MCG MAC: Reconfigure SCG RLC: See Note 5 SCG MAC: See Note 5 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 5 SCG MAC: See Note 5 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 5 SCG MAC: See Note 5 |

NOTE 5:	The MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.