3GPP TSG-RAN WG2 Meeting #104	R2-1818572

Spokane, USA, 12th - 16th November 2018                                Revision of R2-1818407

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0458 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add p-Max for SUL in SIB2 and SIB4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The description in actions upon reception of SIB2 and SIB4 does not consider SUL. - Only one p-Max can be configured in SIB2 and SIB4. p-Max for SUL carrier in SIB2 and SIB4 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explian the changes - Add decription of SUL in actions upon reception of SIB2 and SIB4. - Add p-Max for SUL carrier in SIB2 and SIB4. Impact analysis Impacted functionality: The changes only impact cell reselection function for SUL capable UEs (standalone NR only) Inter-operability: 1) If the network is implemented according to the CR and the UE is not, there may be different understanding on the p-Max that should be used for cell reselection between network and UE if UE supports SUL. 2) If the UE is implenented according to the CR and the network is not, there may be different understanding on the p-Max that should be used for cell reselection between UE and network if UE supports SUL. |  |  |  |  |  |  |  |  |
| u |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The decription in actions upon reception of SIB2 and SIB4 does not consider SUL. - No p-Max for SUL carrier in SIB2 and SIB4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.3, 5.2.2.4.5, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

=========================Start of 1st change============================

##### 5.2.2.4.3	Actions upon reception of SIB2

Upon receiving SIB2, the UE shall:

1> if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2> if, for the frequency band selected by the UE (from the procedure in Section 5.2.2.4.2) to represent the serving cell’s carrier frequency, the frequencyBandList/ frequencyBandListSUL is present in SIB2 and the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList/ frequencyBandListSUL:

3> apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList/ frequencyBandListSUL;

3> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

4> apply the additionalPmax;

3> else:

4> if the UE supports SUL frequency for this cell:

5> apply the p-MaxSUL;

4> else:

5> apply the p-Max;

2> else:

3> if the UE supports SUL frequency for this cell:

4> apply the p-MaxSUL;

3> else:

4> apply the p-Max;

<Partially omitted>

##### 5.2.2.4.5	Actions upon reception of SIB4

Upon receiving SIB4 the UE shall:

1> if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2> if, for the frequency band selected by the UE to represent a non-serving NR carrier frequency is not a downlink only band:

3> if, for the selected frequency band, the frequencyBandList/ frequencyBandListSUL is present in SIB4 and the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList/ frequencyBandListSUL:

4> apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList/ frequencyBandListSUL;

4> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

5> apply the additionalPmax;

4> else:

5> if the UE supports SUL frequency for this cell:

6> apply the p-MaxSUL;

5> else:

6> apply the p-Max;

3> else:

4> if the UE supports SUL frequency for this cell:

5> apply the p-MaxSUL;

4> else:

5> apply the p-Max;

### =========================End of 1st change============================

### =========================Start of 2nd change============================

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=		SEQUENCE {

cellReselectionInfoCommon			SEQUENCE {

nrofSS-BlocksToAverage					INTEGER (2..maxNrofSS-BlocksToAverage)	OPTIONAL,		-- Need R

absThreshSS-BlocksConsolidation			ThresholdNR									OPTIONAL, 	-- Need R

rangeToBestCell						RangeToBestCell							OPTIONAL,	-- Need R

q-Hyst								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars			SEQUENCE {

mobilityStateParameters				MobilityStateParameters,

q-HystSF						SEQUENCE {

sf-Medium						ENUMERATED {

dB-6, dB-4, dB-2, dB0},

sf-High							ENUMERATED {

dB-6, dB-4, dB-2, dB0}

}

}																						OPTIONAL,		-- Need R

...

},

cellReselectionServingFreqInfo		SEQUENCE {

s-NonIntraSearchP						ReselectionThreshold		OPTIONAL,		-- Need R

s-NonIntraSearchQ						ReselectionThresholdQ		OPTIONAL,		-- Need R

threshServingLowP						ReselectionThreshold,

threshServingLowQ						ReselectionThresholdQ		OPTIONAL, 		-- Need R

cellReselectionPriority					CellReselectionPriority,

cellReselectionSubPriority				CellReselectionSubPriority	OPTIONAL, 		-- Need R

...

},

intraFreqCellReselectionInfo		SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinSUL						Q-RxLevMin					OPTIONAL, 		-- Need R

q-QualMin							Q-QualMin					OPTIONAL,		-- Need S

s-IntraSearchP						ReselectionThreshold,				-- Need S

s-IntraSearchQ						ReselectionThresholdQ		OPTIONAL,	-- Cond RSRQ

t-ReselectionNR						T-Reselection,

frequencyBandList					MultiFrequencyBandListNR-SIB			OPTIONAL,	-- Need R

frequencyBandListSUL				MultiFrequencyBandListNR-SIB			OPTIONAL,	-- Need R

p-Max								P-Max						OPTIONAL,		-- Need R

smtc								SSB-MTC						OPTIONAL,		-- Need R

ss-RSSI-Measurement				SS-RSSI-Measurement		OPTIONAL,		-- Need R

ssb-ToMeasure						SSB-ToMeasure				OPTIONAL,		-- Need R

deriveSSB-IndexFromCell			BOOLEAN,

...,

[[

p-MaxSUL							P-Max						OPTIONAL,		-- Need R

]]

},

...

}

ORangeToBestCell	::= Q-OffsetRange

-- TAG-SIB2-STOP

-- ASN1STOP

SIB2 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells.

intraFreqcellReselectionInfo Cell re-selection information common for intra-frequency cells.

deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to TRUE, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in 38.133 [14].

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

orangeToBestCell Parameter “rangeToBestCell” in TS 38.304 [4].

p-Max Value applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101 [15].

p-MaxSUL Value applicable for the intra-frequency neighbouring NR cells with supplementary uplink (SUL). If absent the UE applies the maximum power according to TS 38.101 [15].

q-Hyst Parameter Qhyst in TS 38.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-QualMin Parameter “Qqualmin” in TS 38.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin Parameter “Qrxlevmin” in TS 38.304 [4], applicable for intra-frequency neighbour cells.

q-RxLevMinSUL Parameter “QrxlevminSUL” in TS 38.304 [4], applicable for intra-frequency neighbour cells.

s-IntraSearchP Parameter “SIntraSearchP” in TS 38.304 [4]. If this field is not present, the UE applies the (default) value of infinity for SIntraSearchP.

s-IntraSearchQ Parameter “SIntraSearchQ” in TS 38.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SIntraSearchQ.

s-NonIntraSearchP Parameter “SnonIntraSearchP” in TS 38.304 [4]. If this field is not present, the UE applies the (default) value of infinity for SnonIntraSearchP.

s-NonIntraSearchQ Parameter “SnonIntraSearchQ” in TS 38.304 [4]. If the field is not present, the UE applies the (default) value of 0 dB for SnonIntraSearchQ.

threshServingLowP Parameter “ThreshServing, LowP” in TS 38.304 [4].

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see 38.215). When the field is absent the UE measures on all SS-blocks.

smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequnecy cells.

threshServingLowQ Parameter “ThreshServing, LowQ” in TS 38.304 [4].

t-ReselectionNR Parameter “TreselectionNR” in TS 38.304 [4].

<Partially omitted>

#### SIB4

SIB4 contains information relevant only for inter-frequency cell re-selection i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=		SEQUENCE {

interFreqCarrierFreqList			InterFreqCarrierFreqList,

lateNonCriticalExtension			OCTET STRING		OPTIONAL,

...

}

InterFreqCarrierFreqList ::=             SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqInfo ::=	 SEQUENCE {

dl-CarrierFreq					ARFCN-ValueNR,	multiFrequencyBandListNR 		MultiFrequencyBandListNR,

frequencyBandList					MultiFrequencyBandListNR-SIB			OPTIONAL,	-- Need R

frequencyBandListSUL				MultiFrequencyBandListNR-SIB			OPTIONAL,	-- Need R

nrofSS-BlocksToAverage			INTEGER (2..maxNrofSS-BlocksToAverage)		OPTIONAL,

absThreshSS-BlocksConsolidation	ThresholdNR									OPTIONAL,

smtc								SSB-MTC										OPTIONAL,	-- Need R

ssbSubcarrierSpacing				SubcarrierSpacing,

ssb-ToMeasure					SSB-ToMeasure 								OPTIONAL,	-- Need R

deriveSSB-IndexFromCell			BOOLEAN,

ss-RSSI-Measurement				SS-RSSI-Measurement	}								OPTIONAL,

q-RxLevMin							Q-RxLevMin,

q-RxLevMinSUL						Q-RxLevMin			OPTIONAL, 		-- Need R

q-QualMin							Q-QualMin			OPTIONAL,		-- Need R,

p-Max								P-Max	 						OPTIONAL,		-- Need R

t-ReselectionNR					T-Reselection,

t-ReselectionNR-SF					SpeedStateScaleFactors OPTIONAL,		-- Need N

threshX-HighP						ReselectionThreshold,

threshX-LowP						ReselectionThreshold,

threshX-Q						SEQUENCE {

threshX-HighQ						ReselectionThresholdQ,

threshX-LowQ						ReselectionThresholdQ

}																	OPTIONAL, -- Cond RSRQ

cellReselectionPriority			CellReselectionPriority		OPTIONAL,		-- Need R

cellReselectionSubPriority		CellReselectionSubPriority	OPTIONAL,		-- Need R

q-OffsetFreq						Q-OffsetRange					DEFAULT dB0,

interFreqNeighCellList			InterFreqNeighCellList		OPTIONAL, 		-- Need R

interFreqBlackCellList			InterFreqBlackCellList		OPTIONAL, 		-- Need R

...,

[[

p-MaxSUL						P-Max						OPTIONAL,		-- Need R

]]

}

InterFreqNeighCellList ::=			SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellInfo ::=			SEQUENCE {

physCellId								PhysCellId,

q-OffsetCell							Q-OffsetRange,

q-RxLevMinOffsetCell				INTEGER (1..8)				OPTIONAL,	-- Need R

q-RxLevMinOffsetCellSUL				INTEGER (1..8)				OPTIONAL,	-- Need R

q-QualMinOffsetCell						INTEGER (1..8)				OPTIONAL,	-- Need R

...

}

InterFreqBlackCellList ::=			SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

-- TAG-SIB4-STOP

-- ASN1STOP

SIB4 field descriptions

absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index.

deriveSSB-IndexFromCell This field indicates whether the UE may use the timing of any detected cell on that frequency to derive the SSB index of all neighbour cells on that frequency. If this field is set to TRUE, the UE assumes SFN and frame boundary alignment across cells on the neighbor frequency as specified in 38.133 [14].

interFreqBlackCellList List of blacklisted inter-frequency neighbouring cells.

interFreqCarrierFreqList List of neighbouring carrier frequencies and frequency specific cell re-selection information.

interFreqNeighCellList List of inter-frequency neighbouring cells with specific cell re-selection parameters.

nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation.

p-Max Value applicable for the neighbouring NR cells on this carrier frequency. If absent the UE applies the maximum power according to TS 38.101 [15].

p-MaxSUL Value applicable for the intra-frequency neighbouring NR cells with supplementary uplink (SUL). If absent the UE applies the maximum power according to TS 38.101 [15].

q-OffsetCell Parameter “Qoffsets,n” in TS 38.304 [4].

q-OffsetFreq Parameter “Qoffsetfrequency” in TS 38.304 [4].

q-QualMin Parameter “Qqualmin” in TS 38.304 [4].

q-QualMinOffsetCell Parameter “Qqualminoffsetcell” in TS 38.304 [4]. Actual value Qqualminoffsetcell = field value [dB].

q-RxLevMinOffsetCell Parameter “Qrxlevminoffsetcell” in TS 38.304 [4]. Actual value Qrxlevminoffsetcell = field value * 2 [dB].

q-RxLevMinOffsetCellSUL Parameter “QrxlevminoffsetcellSUL” in TS 38.304 [4]. Actual value QrxlevminoffsetcellSUL = field value * 2 [dB].

smtc Measurement timing configuration for inter-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms in this frequency.

ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see 38.215). When the field is absent the UE measures on all SS-blocks.

ssbSubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.

threshX-HighP Parameter “ThreshX, HighP” in TS 38.304 [4].

threshX-HighQ Parameter “ThreshX, HighQ” in TS 38.304 [4].

threshX-LowP Parameter “ThreshX, LowP” in TS 38.304 [4].

threshX-LowQ Parameter “ThreshX, LowQ” in TS 38.304 [4].

t-ReselectionNR Parameter “TreselectionNR” in TS 38.304 [4].

## t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [4]. If the field is not present, the UE behaviour is specified in TS 38.304 [4].

========================End of 2nd change============================