3GPP TSG-RAN2 Meeting #104	R2-1818628

# Spokane, United States, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3619 | rev | 3 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to additional SIB1 in eFeNB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, BlackBerry UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2018-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In order to keep aligned understanding for the validity of subframe #3 after additionalTransmissionSIB1 in MIB-NB is configured, RAN2 has introduced the related UE capability. However, the scenario of initial connection establishment hasn’t been covered by the current specifications. For a R15 UE that supports additional SIB1, the UE would always assume there exists dedicated transmission for it in subframe #3 not containing additional SIB1 after reception of Msg4. But for eNB, after transmission of Msg4 and before end of UE capability acquisition, the eNB would not schedule dedicated transmission on the subframe #3. The UE may incorrectly monitor NPDCCH and decode NPDSCH transmission, and finally failure to receive the downlink messages. Then for example, the UE capability acquisition procedure or RRC reconfiguration procedure may fail. Therefore, the UE that supports additional SIB1 cannot assume by default the subframe #3 not containing additional SIB1 transmission as valid DL subframe. An explicit indication to UE would be needed. The UE can assume the subframe #3 not containing additional SIB1 transmission as valid DL subframe only after it obtains the indication from the eNB. For the eNB, it can set such indication for indicating subframe #3 as valid DL subframe in RRC message, e.g, Msg4 only after it acquires the UE capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add an indication in PhysicalConfigDedicated-NB to indicate whether the subframe #3 not containing additional SystemInformationBlockType1-NB transmission can be assumed as a valid NB-IoT DL subframe. Impact Analysis Impacted functionality NPDCCH monitoring and NPDSCH decoding. Inter-operability If the network is implemented according to the CR and the UE is not, or vice versa, the NPDCCH tranmitted in UE-specific search space and NPDSCH scheduled by NPDCCH in the UE-specific search space may be decoded incorrectly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPDCCH tranmitted in UE-specific search space and NPDSCH scheduled by NPDCCH in the UE-specific search space may be decoded incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.213 CRxxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of the modified section>

The IE PhysicalConfigDedicated-NB is used to specify the UE specific physical channel configuration.

PhysicalConfigDedicated-NB information element

-- ASN1START

PhysicalConfigDedicated-NB-r13 ::=	SEQUENCE {

carrierConfigDedicated-r13			CarrierConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npdcch-ConfigDedicated-r13			NPDCCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

npusch-ConfigDedicated-r13			NPUSCH-ConfigDedicated-NB-r13		OPTIONAL,	-- Need ON

uplinkPowerControlDedicated-r13		UplinkPowerControlDedicated-NB-r13	OPTIONAL,	-- Need ON

...,

[[	twoHARQ-ProcessesConfig-r14		ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	interferenceRandomisationConfig-r14	ENUMERATED {true}	OPTIONAL	-- Need OR

]],

[[	npdcch-ConfigDedicated-v1530	NPDCCH-ConfigDedicated-NB-v1530		OPTIONAL	-- Cond TDD

]],

[[	additionalTxSIB1-Config-v1540   ENUMERATED {true}	OPTIONAL -- Cond additionalSIB1

]]

}

-- ASN1STOP

PhysicalConfigDedicated-NB field descriptions

carrierConfigDedicated Anchor/ non-anchor carrier used for all unicast transmissions.

interferenceRandomisationConfig For FDD: Interference randomisation enabled in connected mode, except for random access procedure in connected mode, see TS 36.211 [21]. For random access in connected mode interference randomisation on non-anchor is used and is not used on anchor carrier, see TS 36.211 [21]. For TDD: the parameter is not present.

npdcch-ConfigDedicated NPDCCH configuration.

npusch-ConfigDedicated UL unicast configuration.

twoHARQ-ProcessesConfig Activation of two HARQ processes, see TS 36.212 [22] and TS 36.213 [23].

uplink-PowerControlDedicated UL power control parameter.

additionalTxSIB1-Config Indicates if subframe #3 not containing additional SIB1 transmission is a NB-IoT DL subframe, as specified in 36.213 [23], subclause 16.4.

| Conditional presence | Explanation |
| --- | --- |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| additionalSIB1 | This field is optionally present, Need OR, if additionalTransmissionSIB1 is set to TRUE in MasterInformationBlock-NB; otherwise it is not present. |

<End of the modified section>