3GPP TSG-RAN WG2 Meeting #104	R2-1818804

Spokane, USA, 12 - 16 November 2018

Title:	[DRAFT] LS on PHR timing

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Chunli Wu

E-mail Address:	Chunli.wu@nokia-sbell.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 noticed current PHR timing only covers dynamic grant with DCI, however it should be possible to send PHR via configured grant as well.

RAN2 would like to inform RAN1 that RAN2 made the following agreement on PHR timing:

The MAC entity determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the configured grant(s) and downlink control information which has been received until and including the PDCCH occasion in which the first UL grant for a new transmission is received since a PHR has been triggered if the PHR MAC CE is reported on an uplink grant received on the PDCCH or until the first uplink symbol of PUSCH transmission minus PUSCH preparation time as defined in subclause 6.4 of TS 38.214 [7] if the PHR MAC CE is reported on a configured grant.

RAN2 understood the timing for determining real or virtual PH is also captured in physical layer specification, thus we would like to ask RAN1 to update the corresponding text accordingly.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the RAN2 agreement into account and update the text on real and virtual PH determination accordingly if needed.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#105		25 Feb - 01 Mar 2018	Athens

3GPP RAN2#105bis		08 - 12 Apr 2018		China