3GPP TSG-RAN WG2 Meeting #104 	R2-1818235

# Spokane, USA, 12 – 16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0743 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC processing delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements for RRC processing delay are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements for RRC processing delay are defined, based on contribution R2-1816762. Impact analysis Impacted functionality: RRC procedures Inter-operability: There is no inter-operability issue since the requirements have not been defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC processing delay are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

# 12		Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC connection establishment | RRCSetup | RRCSetupComplete | 15 |  |
| RRC connection resume | RRCResume | RRCResumeComplete | 15 or 12 | N = 12 applies for the case of reception of RRCResume if reducedCP-LatencyEnabled is configured, the UE supports reduced CP latency, and the RRC message only includes MAC and PHY (re-)configurations and does not include (re-)configurations of DRX, Configured scheduling, SCells, and MIMO. For other cases N = 15 applies. |
| RRC reconfiguration (radio resource configuration) | RRCReconfiguration | RRCReconfigurationComplete | 15 |  |
| RRC reconfiguration (measurement configuration) | RRCReconfiguration | RRCReconfigurationComplete | 15 |  |
| RRC reconfiguration (re-configuration with sync) | RRCReconfiguration | RRCReconfigurationComplete | 20 |  |
| RRC reconfiguration (SCell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 20 |  |
| RRC connection re-establishment | RRCReestablishment | RRCReestablishmentComplete | 15 |  |
| UE assistance information |  | UEAssistanceInformation | NA |  |

CHANGE END