3GPP TSG-RAN WG2 Meeting #104	R2-1818249

Spokane, US, 12 – 16 November 2018

Agenda item:	11.6.4.2

Source: 	Huawei, HiSilicon

Title: 	Considerations on NTN impact on cell selection and MSE

Document for:	Discussion and Decision

Introduction

In the RAN plenary #80 meeting, a new RAN3 SI on “solutions evaluation for NR to support Non Terrestrial Network” was approved [1]. The objectives of the SI for Layer 2 and above, and RAN architecture are reported as follows:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2]

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

- Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3]

  - Paging: procedure adaptations in case of moving satellite foot prints or cells

In this contribution, we provide our consideration on NTN impact on cell selection and MSE.

Discussion

  - Cell selection

In [2], the minimum RF characteristic of the terminals operating respectively in Ka band (e.g. very small aperture terminals) and in S band (e.g. handheld terminals) is provided in the following table.

|  | Very Small Aperture Terminal (fixed or mounted on moving platforms) | Handheld or IoT devices (3GPP class 3, see [2]) |
| --- | --- | --- |
| Transmit Power | 2 W (33 dBm) | 200 mW (23 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) | Omnidirectional antenna (linear polarisation) |
| Antenna gain | Tx: 43.2 dBi Rx: 39.7 dB | Tx and Rx: 0 dBi |
| Noise figure | 1.2 dB | 9 dB |
| EIRP | 45.75 dBW | -7 dBW |
| G/T (NOTE 1) | 18.5 dB/K | -33.6 dB/K |
| Polarisation (NOTE 2) | Circular | Linear |

In terrestrial networks, since the cell size is limited, the BS receiver can receive the signal from UEs in cell edge generally, so UE performs cell selection based on the received downlink reference signal without considering uplink signal. However, in NTN, especially GEO based NTN, the path loss is very large due to the long transmission distance. For GEO operating in Ka band, the free space path loss is about -190dBm. For GEO operating in S band, the free space path loss is about -175dBm. It will be very challenging for BS’ receiver to receive the signal from 23dBm successfully.

Observation 1: the transmission power of 3GPP UE may be not enough to support transmitting signal to some satellites with high altitude.

In this case, UE cannot access the network although the UE has camped in a cell, which is similar to the Chiba issue. Considering that the RTT is very long for GEO based NTN, it will be very time-consuming to change the camping cell by adding temporary offset in S criterion after many times of RACH failure. Hence, RAN2 should consider to enhance cell selection mechanism to avoid UEs with low transmission camping the cells with high altitude.

Proposal 1: RAN2 should consider to enhance cell selection mechanism to avoid UEs with low transmission power camping the cells with high altitude.

Mobility state estimation

The MSE mechanism has been introduced in NR and take LTE as a baseline. According to [3], the mobility sates of UE include normal-mobility sate, medium-mobility sate, and high-mobility sate. The UE estimate its mobility state via counting the number of cell reselections during a period of time TCRmax:

State detection criteria:

Normal-mobility state criteria:

-	If number of cell reselections during time period TCRmax is less than NCR_M.

Medium-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than NCR_H.

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than NCR_H.

In terrestrial networks, the mechanism assumes that the cell is static and the UE moves. However, in NTN, both the cell and UE move (except GEO scenario), which causes that it is difficult to distinguish the cell reselection resulting from UE moving or cell moving. If a high speed moving UE moves from terrestrial networks to GEO-based NTN during TCRmax, its mobility state will be overestimated for the current NTN because the coverage of GEO cell is very large and the reselection occurs infrequently. On the contrary, if a low speed moving UE moves from terrestrial networks to LEO-based NTN during TCRmax, its mobility state will be underestimated for the current NTN due to the high speed motion of LEO. If using the MSE directly in NTN, the result of MSE may be not accurate and this leads to apply inaccurate speed dependent scaling factor in other procedures. Hence, RAN2 should consider to study the MSE issue in NTN.

Proposal 2: RAN2 should consider to study the MSE issue in NTN.

Conclusion

In this contribution, we considered NTN impact on cell selection and MSE, and we get the following observation and proposals:

Observation 1: the transmission power of 3GPP UE may be not enough to support transmitting signal to some satellites with high altitude.

Proposal 1: RAN2 should consider to enhance cell selection mechanism to avoid UEs with low transmission power camping the cells with high altitude.

Proposal 2: RAN2 should consider to study the MSE issue in NTN.

References

[1] RP-171450, Study on NR to support Non-Terrestrial Networks.

[2] TR 38.811, Study on New Radio (NR) to support non terrestrial networks

[3] TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state