3GPP TSG-RAN WG2 Meeting #104	R2-1818333

Spokane, USA, 12 – 16 Nov, 2018

Agenda Item	: 	11.6.4.1

Source	: 	LG Electronics Inc.

Title	:	Signalling Delay in NTN

Document for	:	Discussion and Decision

- Introduction

In the last RAN Plenary meeting, for Non-Terrestrial Network (NTN), a study item was agreed [1].

Layer 2 and above, and RAN architecture - Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2] - Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

And, in the last RAN2#103Bis meeting, RAN2 agreed below [2].

Impacts to study for CP - Mobility - TA management and update

In this document, we discuss potential problem which may be caused by signalling delay in NTN.

- Discussion

Following the TR 38.811 [3], the research is saying that the satellite systems feature much larger propagation delays than terrestrial systems. The one-way delay between the UE and the RAN (whether on-board the satellite/HAPS or on the ground) may reach up to 272.4 ms for GSO (Geostationary Synchronous Orbit) systems, and is greater than 14.2 ms for NGSO (Non-Geosynchronous Orbit) systems. It means that round trip time between the UE and the gNB (satellite) could be over 540 ms as the worst scenario and this is significant issue to consider.

Observation 1: In NTN, one-way delay between the UE and the RAN may reach up to 272.4 ms as the worst case.

We think that this propagation delay may cause providing invalid configuration if the network applies user’s input such as measurement report, UE (assistant) information to the RRC (Re-) configuration. The most important scenario is mobility. Because, in NTN, the UE with maximum 1000km velocity is considered including aerial service, radio quality can be drifty deviated especially in the case of moving LOS area to NLOS area or vice versa. In this kind of scenarios, even though the network provide a handover command upon receiving the UE’s measurement report e.g. event A3 entering, the cell which was satisfied with the measurement event may leave the measurement event when the handover command is received by the UE. Following cases could be possible to become invalid handover command to the UE:

- Upon reporting measurement, source cell becomes good

This case is that the UE report measurement event e.g. A3 and the network provide RRC Reconfiguration including handover command after over 500ms. During the delay time of 500ms, the source cell becomes better than threshold so that measurement event is leaving.

- Upon reporting measurement, target cell becomes bad

This case is that the UE report measurement event e.g. A3 and the network provide RRC Reconfiguration including handover command after over 500ms. During the delay time of 500ms, the target cell becomes worse than threshold so that measurement event is leaving.

- Upon reporting measurement, another cell becomes better

This case is that the UE report measurement event e.g. A3 and the network provide RRC Reconfiguration including handover command after over 500ms. During the delay time of 500ms, UE detect another cell which is better than the target cell to move.

Figure 1: An Example of Propagation Delay Problem

Figure 1 is describing one of example about propagation delay problem. In our view, above scenarios may come together. Especially in option (2), considering that the target cell is going worse, the handover may be failed with high possibility. This is totally different scenarios comparing with the legacy terrestrial deployment so that we propose to consider that the UE should always comply the network configuration in NTN if possible.

Observation 2: Especially in case of mobility, invalid configuration may be provided due to large propagation delay.

In our view, there may be 3 options to resolve this kind of invalid mobility situation due to large propagation delay. Upon detecting propagation delay problem, following solutions could be applicable to resolve the problem:

- UE triggers Re-establishment with indication to inform the problem

Since moving towards invalid target cell would cause handover failure with high possibility, triggering re-establishment could be a simple solution to resolve the problem. While performing re-establishment, the UE may indicate the propagation delay problem via MSG3 or MSG messages. However additional propagation delay will be happen while the UE performs RRC Connection Re-establishment.

- After mobility towards target cell, UE sends Reconfiguration Complete with indication to inform the problem

Since the basic principle of UE behaviour is that UE is always supposed to comply the network’s configuration, this solution may keep the principle as is. However, in some cases of the above, target cell becomes no longer to be proper cell to move so that the mobility cause handover failure with high possibility since the target cell regards invalid already.

- Without mobility, to source cell, UE sends Reconfiguration Complete with indication to inform the problem

The solution (1) may cause another propagation delay during re-establishment procedure. Then, the solution (2) may cause handover failure with high possibility. Unless the source cell still below the threshold of event reporting which is given by gNB, the solution (3) can be the efficient to support reliability of mobility scenarios.

Proposal: RAN2 is kindly asked to discuss below options to handle invalid mobility due to large propagation delay.

- RRC Connection Re-establishment with indication to inform problem

- Sending indication to inform problem after handover

- Sending indication to inform problem w/o handover

- Conclusion

In conclusion, we propose the followings:

Observation 1: In NTN, one-way delay between the UE and the RAN may reach up to 272.4 ms as the worst case.

Observation 2: Especially in case of mobility, invalid configuration may be provided due to large propagation delay.

Proposal: RAN2 is kindly asked to discuss below options to handle invalid mobility due to large propagation delay.

- RRC Connection Re-establishment with indication to inform problem

- Sending indication to inform problem after handover

- Sending indication to inform problem w/o handover

- References

- RP-181370	Study on solutions evaluation for NR to support Non Terrestrial Network, Thales

- R2-1815605	Report from Break-Out Session, Session Chair, InterDigital

- 3GPP TR 38.811 Study on New Radio (NR) to support non terrestrial networks