3GPP TSG-RAN WG2 Meeting #104	R2-1818831

# Spokane, USA, 12 - 16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0082 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UE capability coordination for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE capability coordination section lacks Stage 2 text on how the capabilities for E-UTRA, NR and MR-DC are retrieved and how they are kept consistent relative to each other. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 7.3 it is clarified how the MN requests the above mentioned capabilities and how they are kept consistent with each other. Impact analysis Impacted functionality: MR-DC capability coordination. Inter-operability: None. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification text does not provide a clear explanation on how the capabilities for E-UTRA, NR and MR-DC are retrieved and how they are kept consistent relative to each other. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 7.3	UE capability coordination

For a UE supporting MR-DC, the MR-DC specific capabilities (e.g. the MR-DC band combinations, listing the supported E-UTRA and NR band combinations) are contained in a MR-DC capability container. MR-DC specific capabilities as well as NR PDCP capabilities (included in both E-UTRA Capability and NR capability) are visible to both the MN and SN.

Other RAT specific capabilities need not be visible to the node of the other RAT.

The MN shall, at least, provide the UE with a filter, comprising of NR and E-UTRA frequency bands, that is used to retrieve the MR-DC specific capabilities and the E-UTRA and NR capabilities relevant for MR-DC. In addition, for MR-DC, the MN ensures that the same filter is provided to the UE across multiple capability enquiries (i.e. E-UTRA, NR and MR-DC) to guarantee that the retrieved capabilities across all the requests are consistent with each other. Furthermore, the MN stores the retrieved capabilities and the corresponding filter, used to retrieve those capabilities, in the core network for later use.

For the UE capabilities requiring coordination between E-UTRA and NR (i.e. band combinations, baseband processing capabilities and the maximum power for FR1 the UE can use in SCG), it is up to the MN to decide on how to resolve the dependency between MN and SN configurations. The MN then provides the resulting UE capabilities usable for SCG configuration to the SN. As part of an SN initiated SN modification, the SN may indicate the desired UE capabilities usable for SCG configuration, and it is up to the MN to make the final decision whether to accept or reject the request.

Next Modified Subclause