3GPP TSG RAN WG2#105	R2-1900015

Athens, Greece, 25 February – 1 March, 2019

3GPP TSG RAN WG1 Meeting #95			R1-1814339

Spokane, USA, November 12th – 16th, 2018

Title:	Reply LS on the search space of cross-carrier scheduling

Response to:	Reply LS (R1-1812115/R2-1816064) on the search space of cross-carrier scheduling from RAN WG2

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

To:	RAN WG2

CC:	-

Contact Person:

#### Name:	Fred TAKEDA

E-mail Address:	kazuki.takeda.zd@nttdocomo.com

1. Overall Description:

RAN1 would like to thank RAN2 regarding the reply LS on higher layer configuration for search space monitoring. RAN1 further discussed and achieved following agreements:

Agreements: For the agreements made in RAN1#94bis as below: - UE uses the same ID for linkage of the search spaces configured in scheduled and scheduling cell, and ignores the CORESET ID configured in the scheduled cell RAN1 re-confirms the agreements, with further clarification as below: - Number of candidates configured in search-space in scheduled cell apply to the search-space of the same index in scheduling cell. - Note: If only one BWP is configured on scheduled cell, then number of cross-carrier search-spaces is limited to 10 instead of 40 - noOfCandidates in SS set(s) of active BWP of the scheduled cell(s) is applied to the corresponding SS set(s) of active BWP of the scheduling cell(s) - Note: this is the original RAN1 solution and concerns of RAN2 has been addressed Agreement - For cross carrier scheduling, Tci-PresentInDCI has to be always enabled - For cross-carrier scheduling with tci-PresentInDCI being enabled for a DCI format 1_1, the scheduling timing offset cannot be smaller than the threshold - Above two bullet applies in case the scheduled carrier is in FR2 - The first bullet applies for both FR1 and FR2 - The first two bullets apply only for CORESET associated for cross carrier scheduling

Therefore,

- No further consideration on solution 2 is necessary.

- For the issues listed in R1-1812115,

  - Only the parameter nrofCandidates and searchSpaceId in the IE SearchSpace configured in scheduled cell are applicable for search space monitoring for cross-carrier scheduling. Other parameters in the IE SearchSpace configured in scheduled cell are ignored by the UE.

  - As agreed, nrofCandidates from the IE SearchSpace of a DL BWP of the scheduled cell is applicable to the IE SearchSpace of active DL BWP of scheduling cell with the same searchSpaceId only if the DL BWP of scheduled cell is active. In other words, nrofCandidates in the IE SearchSpace of the non-active DL BWP of the scheduled cell is not valid in the active DL BWP of scheduling cell.

  - For cross-carrier scheduling, tci-PresentInDCI is always enabled for CORESET associated for cross carrier scheduling.

  - For the IE PDCCH-Config in the scheduled cell, parameters other than searchSpacesToAddModList and searchSpacesToReleaseList are not necessary.

  - For the IE controlResourceSet in the scheduled cell, any parameters are not necessary.

2. Actions:

To RAN WG2

ACTION: RAN1 would like RAN2 to take the above into consideration for the specification work.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #96				21st – 25th Feb. 2019	Athens		Greece.

TSG-RAN WG1 Meeting #96bis				8th – 12th April 2019			China.