3GPP TSG-RAN WG2 Meeting #105                                                        R2-1900146

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Resource Allocation Mode Control

Agenda Item:	11.4.2.2

Document for:	Discussion and Decision

Introduction

For LTE V2X, only one resource allocation mode can be configured to a UE, either mode-3 or mode-4. But on the last RAN2#104 meeting, it was agreed that:

RAN2 will support the case a UE can be configured to perform both mode-1 and mode-2 at the same

time assuming RAN1 does not have concern on it. FFS on the scenario which it is applicable.

In this contribution, we will discuss further on this issue.

Discussion

Resource allocation mode control granularity

From the perspective of requirement, the main motivation of introducing two resource allocation modes configured simultaneously is that NR V2X supports multiple services with different QoS requirements. Different V2X services may need different resource allocation mode in order to meet their different QoS requirements. Hence it is obvious that the resource allocation mode should be decided based on SLRB or V2X QoS parameters.

Proposal 1: Resource allocation mode should be configured based on SLRB or V2X QoS parameters.

Furthermore, it needs to be decided whether one SLRB or one set of V2X QoS parameters can be configured with more than one resource allocation mode simultaneously. If it is allowed, it should depend on UE implementation to determine which resource allocation mode should be used at each transmission occasion. If the resource allocation mode is determined by UE and UE is allowed to change the resource allocation mode frequently, the SR/BSR procedure will be very complex. Hence, one SLRB or one set of V2X QoS parameters should be configured with only one resource allocation mode.

Proposal 2:  One SLRB or one set of V2X QoS parameters should be configured with only one resource allocation mode.

For resource allocation mode configured based on V2X QoS parameters, UE assistant information is not mandatory. For example, if the network can configure the resource allocation mode for all possible V2X QoS parameters combinations, UE assistant information is unnecessary; otherwise, UE assistant information is necessary. For resource allocation mode configured based on SLRB, the UE assist information should at least contain the LCID of the SLRB, the QoS parameters of the SLRB.

Proposal 3: If resource allocation mode is decided based on SLRB, UE should report SLRB related assistant information to the network.

Applicable for only inter-frequency or both inter and intra-frequency

Before deciding whether UE configured with two resource allocation modes simultaneously is only applicable for inter-frequency or applicable for both inter-frequency and intra-frequency, we need to analyze how it works for only inter-frequency case and both intra-frequency and inter-frequency case separately:

Case 1: Only inter-frequency

If simultaneously configuration of mode-1 and mode-2 resource allocation is only applicable for inter-frequency, it means that one BWP can only be configured with only one resource allocation mode and there must be some association between BWP and resource allocation mode. The association between BWP and resource allocation mode can be explicitly or implicitly indicated:

- Explicitly method

The explicitly method means network explicitly configures the association between resource allocation mode and BWP, as shown in (A) of Figure 1.

- Implicitly method

The implicitly method means the network should at least indicate the following two kinds of information to UE:

- The mapping between resource allocation mode and SLRB/QoS parameters;

- The mapping between SLRB/QoS parameters and BWP, e.g., for example, through sidelink LCH restrictions.

The UE can deduce the resource allocation mode for each BWP based on the above two mapping relations, as shown in (B) of Figure 1.

Figure1.  Simultaneously configuration of mode-1 and mode-2 only applicable for inter-frequency

Based on the above analysis, for Case 1, it is clearly that the mapping amongst SLRB, BWP and resource allocation mode is very clear. No much specification effort is needed.

Case 2: Both intra-frequency and inter-frequency

If simultaneously configuration of mode-1 and mode-2 resource allocation can be applicable for intra-frequency, the mapping between resource allocation mode and BWP is not needed. It only needs to configure the resource allocation mode for each SLRB or V2X QoS parameter set. As shown in Figure-2:

Figure2.  Simultaneously configuration of mode-1 and mode-2 applicable for inter/intra-frequency

For this case, as shown in Figure 2, one BWP can support two resource allocation modes simultaneously. But according to UE capability, UE may not be able to support more than one TB on one BWP at the same time. In this case, if both SLRB1 and SLRB2 have available data, which resource allocation mode should be used at a certain time should be discussed, which is shown in Figure 3.

Figure3. Resource allocation mode collision

Some rules should be defined to solve the collision for the resource allocation mode on one BWP, the possible rules can be:

- Option 1: Based on SLRB QoS parameters, e.g., based on priority or reliability;

- Option 2: mode-1 is always prioritized, if there is no mode-1 resource allocation, mode-2 can be used.

Based on the above analysis, Case 2 only needs to address one more issue (resource allocation mode collision issue) compared with Case 1. In order to solve this issue, it only needs to define one rule to determine which resource allocation mode will be used in case of collision. The specification effort is also acceptable.

Proposal 4: Simultaneously configuration of mode-1 and mode-2 resource allocation modes can be supported for both inter-frequency and intra-frequency.

Proposal 5: In order to support simultaneously configuration of mode-1 and mode-2 resource allocation modes for intra-frequency, one rule needs to be defined in order to determine only one resource allocation mode for one BWP at a certain time.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Resource allocation mode should be configured based on SLRB or V2X QoS parameters.

Proposal 2:  One SLRB or one set of V2X QoS parameters should be configured with only one resource allocation mode.

Proposal 3: If resource allocation mode is decided based on SLRB, UE should report SLRB related assistant information to the network.

Proposal 4: Simultaneously configuration of mode-1 and mode-2 resource allocation modes can be supported for both inter-frequency and intra-frequency.

Proposal 5: In order to support simultaneously configuration of mode-1 and mode-2 resource allocation modes for intra-frequency, one rule needs to be defined in order to determine only one resource allocation mode for one BWP at a certain time.

The related TP is attached in section 4.

Text proposal to TR 38.885

TEXT PROPOSAL START

### 5.1.4	L2/L3 protocols

In NR sidelink, V2X communication is supported for RRC_CONNECTED UEs, RRC_IDLE UEs and RRC_INACTIVE UEs (if supported).

Both UEs in RRC_INACTIVE and those in RRC_IDLE perform V2X communication in NR sidelink by using the cell-specific configurations included in SIB(s) specific to V2X.

To support V2X communication in NR sidelink, NR RRC layer provides at least the following functionalities in Uu:

- Acquisition of V2X-specific SIB(s);

- Establishment of RRC connection for V2X sidelink communication: for the UE configured by the upper layers to transmit V2X communication in NR sidelink with related data available, an RRC connection is established, if at least the following condition(s) are met:

- the frequency on which the UE is configured to transmit V2X communication in NR sidelink is included in the V2X-specific SIB(s), without the inclusion of transmission resource pool for that frequency;

- Configuration of resource allocation modes for V2X communication in NR sidelink: resource allocation Mode 1 and Mode 2 of NR sidelink can be configured at the same time for the UE, which is applicable for both inter-frequency and intra-frequency. In addition, the resource allocation mode can be configured based on SLRB or V2X QoS parameters, but one SLRB or one set of QoS parameters can be configured with only one resource allocation mode. ;

- Mobility management:

- During handover, the transmission and reception of V2X communication in NR sidelink are performed based on at least the following configuration(s)/operation(s):

  - Configuration of the exceptional transmission resource pool and reception resource pool of the target cell, which can be used by the UE during handover, are provided in the handover command;

- Cell selection and reselection for V2X communication in NR sidelink are performed based on at least the following criterion(-a)/configurations

  - The carrier frequencies that may provide V2X sidelink resource configuration or inter-frequency configuration can be (pre)configured;

  - The frequencies providing inter-frequency V2X configurations in NR sidelink are prioritized during cell (re)selection;

  - It is up to UE implementation how to minimize the interruption of V2X transmission and reception in NR sidelink during cell reselection;

- Sidelink UE information reporting;

- Sidelink related measurement and reporting, including:

- Measuring and reporting of CBR;

- Reporting of location information;

Editor's note: Whether to support CBR measurement and reporting, and simultaneous configuration of mode 1 and mode 2 for resource allocation for the UE has RAN1 dependency.

- Reporting of UE assistance information for traffic pattern(s) (periodicity, offset, and packet size), at least for periodic traffic;

Editor's note: The above control plane functionality list is subject to further study and may be updated by adding additional functionalities. Solutions in additional to those listed above are not excluded.

NEXT TEXT PROPOSAL