3GPP TSG-RAN WG2 Meeting #105                                                        R2-1900157

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	PDCP duplication in IIoT

Agenda Item:	11.7.4

Document for:	Discussion and Decision

Introduction

In IIoT SI, “PDCP duplication with more than 2 copies leveraging (combination of) DC and CA” is an important topic. In last meeting, some companies provided their opinions [1][2][3][4].

This contribution discusses this topic from the aspects of: necessity, scenarios, required enhancements.

Discussion

PDCP duplication is a useful L2 tool for reliability enhancement so that we can satisfy the reliability requirement of URLLC with moderate channel condition in which the reliability target of L1 is not very stringent. To satisfy the reliability requirement in IIoT, one important issue is whether to support more than 2 legs, for example, 3-leg duplication.

In [2], the reliability improvement via 3-leg duplication has been simulated. However, as stated in LS [5] to SA2: “if two links operate with 10-4 reliability in PHY/MAC layer, then it is possible to achieve 10-8 reliability for a transmission on PDCP layer”. The most stringent reliability requirement in URLLC is 1-10-9 and several use cases require the reliability of 1-10-8, then the reliability requirement can be achieved by duplication over two legs when each link can reach high enough reliability, such as 1 - (10-4~10-6). And RAN2 always use efficient scheduling tools to differentiate the reliability of different radio bearers in UL over the same radio channel (e.g. LCP restrictions) to reach the anticipated reliability. In a word, even if 3-leg has some reliability enhancement, it is not essential for IIoT.

Observation 1: The requirement of more than 2 legs is not essential.

Meanwhile, one more leg will introduce more resource waste and design complexity. At least below aspects shall be studied:

- Applicable use cases considering the combinations of: CA, DC/MR-DC, SRB/DRB, RLC mode;

- How to configure the multiple legs;

- Activation/deactivation design;

- Leg selection and switching among 1, 2, 3-leg;

- Resource efficiency.

Observation 2: 3-leg duplication would have major design impact and would require studying at least all applicable use cases (CA, DC, SRB/DRB, …), configuration aspects, activation/deactivation design, leg selection and switching, and resource efficiency.

Proposal 1: Activation of more than 2 legs for duplication is not needed.

In last meeting, some companies provided another direction: more than 2 legs are configured but at most 2 legs activated especially considering the combination of DC and CA [1][4].

CA duplication configured with more than 2 legs is shown in Figure 1. In CA duplication, gNB needs to configure different legs on different CC sets. If gNB configures more than 2 legs and activates at most 2 legs, part of CCs cannot be used at all while CA duplication is activated (Figure 1-right). Alternately, the CCs belonging to the deactivated leg could be re-allocated to the activated legs. However, it will increase the complexity and introduce unexpected influence, such as CC switching time.

Meanwhile, configuration duplication over N>2 CC sets does not really help gNB and UE to select better CCs because even with only 2 CC sets, gNB scheduler, based on UE feedback (e.g. SRS) will always select the best CC among the available CCs in each CC set.

Therefore, more than 2-leg configuration for CA duplication ends-up fragmenting the CC resources thus resulting in a less efficient resource usage of the CCs within CA configuration.

Finally, Rel-15 only allows re-configuring the CC set of a duplicated logical channel with RRC, via the LCP channel mapping restriction parameter. It is therefore quite slow which also contributes to the inefficiency of the scheme.

Figure 1 CA duplication (3-leg configuration and 2-leg activation)

Observation 3: Configuring more than 2 legs for CA duplication is useless and less efficient than the current scheme.

In NR Rel-15, at most 2 legs can be configured and either DC duplication or CA duplication could be configured in the DC architecture. DC duplication configured with more than 2 legs (at most 4 legs) is shown in Figure 2. It is a combined configuration of DC and CA duplication actually. MCG and SCG have completely independent resources.  After duplication activation, either DC duplication or CA duplication is used similar to NR Rel-15 (Figure 2-right).

Figure 2 DC duplication (4-leg configuration and 2-leg activation)

With the method of Figure 2, PDCP duplication can be used more efficiently. UE can be activated as CA duplication quickly when one CG is not suitable for duplication due to some reasons such as channel condition deterioration, resource overload, etc. On the other hand, DC duplication can be activated quickly when there is no redundant resource for CA duplication in one CG, for example, channel condition of several CCs in one CG is unreliable, or either CG has not enough resource for CA duplication but the resource for DC duplication can be provided.

To verify whether the method can be used, we need to consider the feasibility and complexity further.

The method of Figure 2 can be implemented as follows:

- Configuration:

  - One RB corresponding to one PDCP can be mapped to 1 or 2 RLC entities in one CG, so at most 4 legs could be configured. When only 1 RLC entity is configured per CG, it is the same as NR Rel-15.

  - In one CG, a default RLC entity can be configured for the case that only one leg is activated in the CG so that the activated leg in one CG does not need to be explicitly indicated in the activation/deactivation MAC CE. Note that this configuration is the same as CA duplication in one CG in NR Rel-15.

  - gNB can configure whether DC duplication or CA duplication is used as default when duplication is activated.

- Activation:

  - When only 1 RLC entity is configured per CG (same as NR Rel-15), duplication activation/deactivation MAC CE in NR Rel-15 is used.

  - When 2 RLC entities are configured in one or both CGs, duplication activation/deactivation MAC CE in NR Rel-15 is used to activate the configured default RLC entities.

  - After duplication activation, if gNB wants to change the duplication mode (from DC duplication to CA duplication, or from CA duplication to DC duplication), another MAC CE (e.g. DC/CA duplication switching MAC CE) can be used. The MAC CE can adopt the same MAC CE format as duplication activation/deactivation MAC CE and use a separate LCID value to indicate.

Above solution options are too detailed though in SI stage. It is mainly used to prove that the method of Figure 2 can be implemented in a simple and backward compatible way.

Observation 4: Configuring more than 2 legs for DC duplication is useful and can be implemented in a simple and backward compatible way.

Proposal 2: In DC duplication, the method of configuring 1 or 2 legs per Cell Group and activating at most 2 legs in the DC architecture should be considered.

Conclusion

This contribution discusses the PDCP duplication support in IIoT and provides below proposals and observations.

Activation:

Observation 1: The requirement of more than 2 legs is not essential.

Observation 2: 3-leg duplication would have major design impact and would require studying at least all applicable use cases (CA, DC, SRB/DRB, …), configuration aspects, activation/deactivation design, leg selection and switching, and resource efficiency.

Proposal 1: Activation of more than 2 legs for duplication is not needed.

Configuration:

Observation 3: Configuring more than 2 legs for CA duplication is useless and harmful.

Observation 4: Configuring more than 2 legs for DC duplication is useful and can be implemented in a simple and backward compatible way.

Proposal 2: In DC duplication, the method of configuring 1 or 2 legs per CG and activating at most 2 legs in the DC architecture can be considered.

Reference

- R2-1817583	, On PDCP Duplication Enhancements with Combination of DC and CA, Nokia, Nokia Shanghai Bell

- R2-1818466	, Analysis of Duplication with More Than 2 Legs, Samsung

- R2-1817071, Considerations on PDCP duplication with multiple copies, ZTE, Sanechips

- R2-1817180	, Multiple duplicate handling, Ericsson

- R2-1816043, LS on TSN requirements evaluation