3GPP TSG-RAN WG2 Meeting #105                                                       R2-1900186

Athens, GR, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Carrier/Resource (Re-) selection Function

Agenda Item:	11.4.2.2

Document for:	Discussion and Decision

Introduction

In the RAN2#104 meeting, the following agreement was made for sidelink carrier/resource (re-)selection function.

Agreements on MAC:

2:	Sidelink carrier/resource (re-)selection function is supported in NR MAC at least for NR Sidelink broadcast. RAN2 should further study whether LTE operation can be reused for Sidelink carrier/resource (re-)selection function in NR, considering RAN1 progress.

In this contribution, we will firstly review the LTE sidelink carrier/resource (re-)selection function and then discuss the sidelink carrier/resource (re-)selection function for NR.

Discussion

LTE sidelink carrier/resource (re-)selection function

For the sidelink resource (re-)selection, there are seven trigger conditions are listed in TS36.321, as below:

- if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by upper layers in probResourceKeep;

- if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the configured sidelink grant during the last second;

- if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the configured sidelink grant is equal to sl-ReselectAfter;

- if there is no configured sidelink grant;

- if the configured sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by upper layers in maxMCS-PSSCH and the MAC entity selects not to segment the RLC SDU;

- if transmission(s) with the configured sidelink grant cannot fulfil the latency requirement of the data in a sidelink logical channel according to the associated PPPP, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU;

- if a pool of resources is configured or reconfigured by upper layers for the selected carrier

For the sidelink carrier (re-)selection function in 36.321, a V2X UE shall consider the CBR of a carrier. If the CBR of the carrier is below threshCBR-FreqReselection associated with priority of sidelink logical channel, the UE can select the carrier and the associated pool of resources. If the CBR of the carrier is below threshCBR-FreqKeeping associated with priority of sidelink logical channel, the UE can keep selecting the carrier and the associated pool of resources; otherwise, the UE shall re-select to other carrier(s) whose CBR is below threshCBR-FreqReselection associated with priority of sidelink logical channel.

NR sidelink carrier/resource (re-)selection function

As agreed in RAN2#104, sidelink carrier/resource (re-)selection function shall be supported in NR MAC for NR sidelink broadcast. For sidelink resource (re-)selection function, the senvern trigger conditions of LTE sidelink can be re-used as a baseline for NR sidelink. The trigger conditions of LTE sidelink may need to be enhanced according to RAN1 and RAN2 progress. For example, in the 6th trigger condition of LTE sidelink, the trigger condition may change from PPPP to VQI according to the discussion in QoS.

In RAN2#104, the CBR measurement/reporting mechanism for LTE V2X sidelink communication was agreed to be as the baseline for NR V2X sidelink. In RAN1#AdHoc 1901, it was agreed that at least one congestion metric will be introduced for NR sidelink and details will be discussed in WI phase. Therefore, for NR sidelink carrier (re-)selection function in NR MAC for NR sidelink broadcast, the CBR is also one of trigger conditions as LTE sidelink carrier (re-)selection mechanism. If there are any enhancements or new measurments for sidelink related measurement and reporting mechanism according to RAN1 progress, the NR sidelink carrier (re-)selection function for NR sidelink broadcast needs to be further studied.

Proposal 1: At least for NR sidelink broadcast, the LTE sidelink carrier/resource (re-)selection function can be reused as baseline.

For NR sidelink unicast and groupcast, the sidelink carrier/resource (re-)selection function also needs to be supported. The sidelink resource (re-)selection function can avoid resource collision and impove the reliability of sidelink unicast and groupcast communications. If a carrier is busy that means the CBR of the carrier is higher than a threshold, UE may not successfully select a resoure on the carrier for sidelink unicast and groupcast communications. UE also needs to re-select to other carrier(s) whose CBR can satisfy the criterion. Therefore, the sidelink carrier/resource (re-)selection function is also important for NR sidelink unicast and groupcast.

Proposal 2: For NR sidelink unicast and groupcast, the sidelink carrier/resource (re-)selection function is necessary to be supported.

For NR sidelink unicast and groupcast, the LTE sidelink carrier/resource (re-)selection function can be reused as a startpoint.

For the resource (re-)selection, new enhancements should be considered since new features are introduced in NR sidelink unicast and groupcast. In RAN1#94bis, the HARQ and CSI feedback was agreed to be supported for NR sidelink unicast and groupcast. The V2X UE can determine the sidelink physical channel quality according to the HARQ and CSI feedback. For example, when the sidelink physical channel quality decreases, the sidelink resource shall be re-selected for NR sidelink unicast and groupcast.

Proposal 3: For NR sidelink unicast and groupcast, the trigger conditions of resource (re-)selection function need consider the sidelink physical channel quality.

For the sidelink carrier (re-)selection, the congestion metric of a carrier is also a trigger condition for NR sidelink unicast and groupcast. Some enhancements and new features are specified in NR compared with LTE carrier aggregation, e.g., multiple numerologies. In RAN1#95 and RAN1#Adhoc 1901, it was agreed that only one SL BWP is configured and active in a carrier for a NR V2X UE and the numerology is a part of SL BWP configuration. Therefore, when a UE needs select/change a numerology according to the UE traffic, the UE needs (re-)select a sidelink carrier with a suitable numerology.

Proposal 4: For NR sidelink unicast and groupcast, the trigger conditions of carrier (re-)selection function need consider the numerology of the NR sidelink carrier.

Conclusion

According to the above analysis, the proposals for sidelink carrier/resource (re-)selection function are as follows:

Proposal 1: At least for NR sidelink broadcast, the LTE sidelink carrier/resource (re-)selection function can be reused as baseline.

Proposal 2: For NR sidelink unicast and groupcast, the sidelink carrier/resource (re-)selection function is necessary to be supported.

Proposal 3: For NR sidelink unicast and groupcast, the trigger conditions of resource (re-)selection function need consider the sidelink physical channel quality.

Proposal 4: For NR sidelink unicast and groupcast, the trigger conditions of carrier (re-)selection function need consider the numerology of the NR sidelink carrier.