3GPP TSG-RAN WG2 Meeting #105	R2-1900202

Athens, Greece, 25 February– 01 March 2019

Source:	CATT

Title:	Power Saving Signal/Channel and Related Procedures

Agenda Item:	11.11.4

Document for:	Discussion and Decision

- Introduction

It was agreed in RAN#80 meeting that power saving techniques should be studied in a new SI [1]. One of the objectives is listed below:

- Identify techniques for UE power saving study with focus in RRC_CONNECTED mode [RAN1, RAN2]

  - Study UE adaptation to the traffic and UE power consumption characteristics in frequency, time, antenna domains, DRX configuration, and UE processing timeline for UE power saving

(Note: existing UE capabilities are assumed for UE processing timeline)

    - Network and/or UE assistance information

    - Include mechanism in reducing PDCCH monitoring, taking into account current DRX scheme

  - Study the power saving signal/procedure for triggering adaptation of UE  power consumption characteristics

It can be seen that power saving signal/procedure is one of the study point. And it has been agreed that power saving signal can be used in Rel-16 for power saving in RAN1, as captured in TR 38.840 [2]. In this document, we briefly describe the main power saving signal/channel options under discussion in RAN1 and analyze the related procedures for triggering adaptation of UE power consumption.

- Discussion

The UE power saving in time domain currently relies on the periodic configuration of DRX to allow UE to sleep during the DRX inactive state in RRC connected mode. UE will wake up from sleep mode when entering DRX ON duration and perform PDCCH monitoring during DRX active state. Even if no data is scheduled by gNB, UE monitors PDCCH during the whole ON-duration period of DRX cycle. On the other hand, if data has been scheduled, UE will continue to monitor PDCCH for the remaining (potentially extended) DRX ON period after the data transmission.

In order to further reduce the power consumption, a power saving signal can be used to wake-up the UE for PDCCH monitoring either independently or in complement to DRX operation. In this contribution we present some possible procedures addressing both cases.

2.1 Power saving signal/channel options

The current RAN1 progress on this design is captured in TR 38.840 [2] as follows:

- Power saving signal/channel # The power saving signal/channel for UE adaptation includes the following signals/channels for further study - Existing signal/channel based power saving signal/channel - PDCCH channel - TRS, CSI-RS type RS, SSS-like and DMRS - PDSCH channel carried MAC CE and/or RRC signaling - New power saving signal/channel – sequence based The aspects of the power saving signal/channel used for the UE adaptation to the traffic used for further evaluation of power saving signal design in addition to its triggering to the power saving gain. - Network resource overhead - Resource and/or periodicity of power saving signal/channel - Multiplexing capability - Usage of resource - Coexistence/multiplexing with existed signal/channel of Rel-15 - UE-specific, group-specific, cell-specific power saving signal/channel - Detection performance - Complexity - Power consumption of the power saving signal/channel

From the above and as further elaborated in [3], two main power saving signal options can be envisioned:

- Option 1: RF-based low complexity receiver with a periodic and low-power signal. This option falls in the above category “New power saving signal/channel – sequence based”.

- Option 2: Baseband receiver with periodic signal/channel. This is typically the case where existing signals/channels (e.g. PDCCH) could be reused.

In both options, the power saving signal indicates whether to wake-up or to remain asleep at some pre-configured occasions. UE monitors the power saving signal at these occasions and wakes-up or goes to sleep accordingly. The wake-up / go-to-sleep indication can be in the form of an ON/OFF signal e.g. with option 1, or with binary values (e.g. as ACK/NACK) e.g. with option 2. In the following we elaborate on the impact of such signals on PDCCH monitoring and DRX procedures.

2.1.1 RF-based Power Saving Signal

In option 1, the power saving signal is based on RF-based receiver with low complexity. The low complexity RF receiver is used to detect the power saving signal with limited capability of interference resistance, but very low power consumption. This enables monitoring the signal with short periods, e.g. every 1 or 2 slots while in sleep state [3]. Procedure-wise, the UE performs quasi-continuous monitoring of the power saving signal and remains in sleep state as long as the signal indicates to do so. Upon detecting a power saving signal “positive” indication, UE wakes up and monitors the PDCCH. Given the signal is almost continuous, there is no need for any timer to go back to sleep: the signal itself triggers the UE back to sleep. The basic principle is illustrated in Figure 1 where such signal is used as standalone mean to control PDCCH monitoring.

Figure 1 Controlling PDCCH monitoring with quasi-continuous power saving signal

It should be further noted that, given the power saving signal is received at an independent front end device, it does not require UE to turn ON the NR receiver and to perform RRM/CSI measurements and the UE can go in deep sleep in a similar way as during DRX Inactive period, even while monitoring the power saving signal. Such power saving signal approach is therefore well suited for a standalone PDCCH monitoring reduction procedure (independently of DRX).

Observation 1: RF-based, short-periodicity power saving signal is well suited to control the PDCCH monitoring activity for any type of traffic, independently of the DRX procedure and without a need for any timer.

2.1.2 Baseband-based Power Saving Signal

With a baseband receiver, the higher power consumption required for monitoring the power saving signal imposes a larger periodicity to keep a low UE power consumption while in sleep state. Therefore, the procedure is similar to that described in Section 2.1.1, except, that the power saving signal would not be used for triggering the UE back to sleep. Instead, this could be based upon timer expiry. For example the timer can be designed similar to the drx-onDurationTimer. Alternately, a MAC CE could indicate the UE to go back to sleep. The procedure is illustrated in Figure 2, and can be viewed as some aperiodic DRX scheme.

Figure 2 “Aperiodic DRX” with power saving signal triggering PDCCH monitoring

Proposal 1: RAN2 shall study the MAC procedure associated with the RAN1-designed power saving signal(s)/channel(s) for efficient PDCCH monitoring reduction in absence of DRX configuration.

It can be noted though that, as mentioned in Section 2.1.1, a baseband-based (e.g. PDCCH based) power saving signal would require UE to stay tune for next power saving signal detection.  This would not allow UE to go to light/deep sleep [2], and, for such type of signal/channel, DRX is still useful to enable UE to light/deep sleep for a long duration of power saving.

Observation 2: Baseband-based, power saving signal/channel (e.g. PDCCH based) is not well-suited for efficient PDCCH monitoring reduction in absence of DRX configuration.

As a result, a more efficient use of baseband-based power saving signal is in complement to DRX, as explained below.

Using the power saving signal in complement to DRX can further reduce the UE power consumption based on the following principles:

- Just before or at the beginning of the Active Time, UE monitors the power saving signal to check whether it needs to wake-up or not.

- When not in Active Time (DRX-OFF period), UE neither monitors the wake-up signal nor the PDCCH.

As illustrated in Figures 3 and 4, at the beginning of (or just before) the Active Time, if the UE does not receive from the power saving signal an indication to wake-up, it assumes there is no data and stays in the sleep state (although still monitoring the power saving signal, if any, during the Active Time period). Otherwise, UE wakes-up and performs PDCCH monitoring, as in normal DRX operation. Such operation allows further reducing the power consumption because:

- Power saving signal is only monitored during or before Active Time

- The power saving signal allows skipping some Active Time periods.

Note that a “preparation period” always precedes the DRX Active Time in preparation for the UE processing, such as channel/beam tracking and RRM/CSI measurements. Such preparation period has been captured in the TR [2] based on UE implementation.  Given the start of DRX Active Time is deterministic, in Rel-15 the preparation period is done by UE implementation without any specification.  The power saving signal triggering UE adaptation to the DRX operation not only saves power for not performing PDCCH decoding but also on not performing channel measurements in preparation period.

A first example of the usage of this procedure is provided in Figure 3, where the DRX is configured to match with a periodic but non deterministic traffic where data may or may not be present at periodic intervals. In such case, only one power saving signal occasion is needed, before or at the beginning of the Active Time. Indeed, if the UE only serves this traffic, or if other traffic data can afford being delayed to the next DRX cycle, the full Active Time period can be skipped.

Figure 3 Power saving signal controlling DRX Active Time skipping

Another example of PDCCH monitoring reduction with power saving signal during DRX is shown in Figure 4. This time, we take the case of a non-continuous, non-periodic traffic, such as gaming, where intermittent data arrival not exactly matching the DRX cycle will likely extend the active periods of DRX. It is therefore difficult for the UE to go to sleep in-between such data bursts. In such case, a power saving signal can allow reducing significantly the PDCCH monitoring by micro/light sleep occasions within the original DRX Active Time. Another proposal to reduce PDCCH monitoring is the dynamic indication of skipping PDCCH decoding through DCI [2].  A new DCI field would need to be specified for gNB to indicate skipping a number of PDCCH monitoring occasions.   The UE behavior during Active Time is basically the same as described for PDCCH monitoring reduction without DRX in Section 2.1.1: it wakes-up if it detects the positive power saving signal; otherwise, it stays in the micro sleep or further goes to light sleep state. Therefore, similarly, the RF-based power saving signal is also best-suited for this procedure.

Figure 4 Triggering reduction of PDCCH monitoring with DRX for aperiodic traffic

Proposal 2: RAN2 shall study DRX enhancements leveraging the RAN1-designed power saving signal(s)/channel(s)

Proposal 3: As a baseline, the following power-saving principle is used in DRX:

- Power saving signal/channel is only monitored just before or during Active Time

- Conclusion

In this document, we present different power saving signal options from [2] and analyze their usage in various possible associated procedures. The resulting observations and proposals are as follows, and we propose a TP to TR 38.840 in Section 8:

Observation 1: RF-based, short-periodicity power saving signal is well suited to control the PDCCH monitoring activity for any type of traffic, independently of the DRX procedure and without a need for any timer.

Proposal 1: RAN2 shall study the MAC procedure associated with the RAN1-designed power saving signal(s)/channel(s) for efficient PDCCH monitoring reduction in absence of DRX configuration.

Observation 2: Baseband-based, power saving signal/channel (e.g. PDCCH based) is not well-suited for efficient PDCCH monitoring reduction in absence of DRX configuration.

Proposal 2: RAN2 shall study DRX enhancements leveraging the RAN1-designed power saving signal(s)/channel(s)

Proposal 3: As a baseline, the following power-saving principle is used in DRX:

- Power saving signal/channel is only monitored just before or during Active Time

- Reference

- RP-181463, New SID: Study on UE Power Saving in NR, 3GPP TSG RAN Meetings #80, La Jolla, USA, 11th  – 14th,  June 2018;

- TR 38.840 v0.1.0, Study on UE Power Saving (Release 16);

- R1-1900345, UE Power saving schemes with power saving signal/channel/procedures, CATT, RAN1 Ad-Hoc Meeting 1901, Taipei, 21st – 25th, January, 2019

- Text Proposal to TR 38.840

    - Power saving procedure

The power saving signal/channel in triggering UE adaptation to DRX operation for further study is that the power saving signal/channel can be configured along the DRX configuration as the indication for UE to wake up from the sleep state.  RS resources can be considered to assist UE in performing RRM/CSI measurement and channel time/frequency and/or beam tracking.

Using the power saving signal in complement to DRX can further reduce the UE power consumption based on the following principles:

- Just before or at the beginning of the Active Time, UE monitors the power saving signal to check whether it needs to wake-up or not.

- When not in Active Time (DRX-OFF period), UE neither monitors the wake-up signal nor the PDCCH.

As illustrated in Figurec1, at the beginning of (or just before) the Active Time, if the UE does not receive from the power saving signal an indication to wake-up, it assumes there is no data and stays in the sleep state. Otherwise, UE wakes-up and performs PDCCH monitoring, as in normal DRX operation. Such operation allows further reducing the power consumption because:

- Power saving signal is only monitored during or before Active Time

- The power saving signal allows skipping some Active Time periods, including preparation time.

Figure 1 Power saving signal controlling DRX Active Time skipping

The power saving signal/channel candidate in triggering UE to achieve reducing PDCCH monitoring for further study  is that the power saving signal/channel can be used to trigger UE to skip the PDCCH monitoring and/or to go to sleep for a period of time.

This procedure assumes UE monitors the power saving signal on periodic occasions, with very low power consumption. Procedure-wise, the UE performs quasi-continuous monitoring of the power saving signal and remains in sleep state as long as the signal indicates to do so. Upon detecting a power saving signal “positive” indication, UE wakes up and monitors the PDCCH. If the signal can be monitored with short periodicity e.g. 1 or 2 slots, there is no need for any timer to go back to sleep: the signal itself triggers the UE back to sleep. The basic principle is illustrated in Figure 2 where such signal is used as standalone mean to control PDCCH monitoring.

Figure 2 Controlling PDCCH monitoring with quasi-continuous power saving signal

In case the power saving signal periodicity is configured large to enable sufficient low power consumption, the power saving signal would not be used for triggering the UE back to sleep. Instead, this could be based upon timer expiry. For example the timer can be designed similar to the drx-onDurationTimer. Alternately, a MAC CE could indicate the UE to go back to sleep. The procedure is illustrated in Figure 3, and can be viewed as some aperiodic DRX scheme.

Figure 3 “Aperiodic DRX” with power saving signal triggering PDCCH monitoring

The power saving signal/channel in triggering UE frequency domain processing adaptation for further study is the power saving signal/channel can be used to trigger the indication of RS configuration for channel tracking, CSI measurements, and beam management for the additional assistance of dynamic switching of BWP or activation of SCell in achieving the power saving gain.  The power saving signal/channel can be used for BWP switching, activation/deactivation of SCell or adaptation of PDCCH monitoring and/or CORESET/search space of PCell/SCell.