3GPP TSG-RAN WG2 Meeting #105												R2-1900234

Athens, Greece, 25th Feb. - 01st Mar. 2019

Source: 	vivo

Title:  	Discussion on SR procedure for NR-U

Agenda Item:	11.2.1.2

Document for:	Discussion and Decision

Introduction

According to the discussion on the SR procedure in the NR-U SI phase, RAN2 made the following agreement:

If SR is not transmitted due to LBT failure, the UE should not be prohibited from trying again by the prohibit timer.

In this contribution, we provide more details on the SR procedures on how to mitigate the LBT impacts of NR-U.

Discussion

As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR: 2> initiate a Random Access procedure (see subclause 5.1) on the SpCell and cancel the pending SR. 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a UL-SCH resource: 3> if SR_COUNTER < sr-TransMax: 4> increment SR_COUNTER by 1; 4> instruct the physical layer to signal the SR on one valid PUCCH resource for SR; 4> start the sr-ProhibitTimer.

More than one opportunities for SR

According to the SR procedure quoted above [1] and highlighted in the purple color, when the MAC instructs the PHY to initiate the SR procedure, the MAC only indicates one selected valid PUCCH resource for SR transmission. Then the UE cannot access different frequency channels for the SR transmission at the same time.

Observation 1: According to the current MAC specification, the MAC indicates only one valid PUCCH resource for SR transmission for each SR transmission attempt.

Figure 1: Multiple SR resource(s) for one SR transmission attempt

To resolve the first issue as given in Observation 1, more than one SR resources can be indicated from the MAC to the PHY for each SR transmission attempt to have multiple LBTs at least on different frequency domains. The PHY can access the channel for each SR resource as indicated by the MAC, and only one SR is transmitted by the PHY if the channel access succeeds.

Proposal 1: More than one SR resources can be indicated from the MAC to the PHY.

Proposal 2: The PHY selects one SR resource for transmission if the channel access succeeds.

If multiple opportunities of SR transmission is supported, RAN2 needs to decide if more than one opportunities of SR resources indicated from the MAC to the PHY can be in different frequency domains or different time domains.

Proposal 3: If Proposal 1 is agreed, RAN2 is kindly requested to discuss if more than one SR resources indicated from the MAC to the PHY are in different frequency domains or different time domains.

SR prohibit timer

Figure 2: Legacy SR prohibit timer

According to the SR procedure text (which is highlighted in the blue and yellow color) of the 38.321 quoted above [1] and the Figure 2 illustrated above, the MAC starts the sr-ProhibitTimer immediately after the MAC indicates the SR transmission (e.g. SR resource 1) to the PHY, regardless whether the SR is transmitted or not. Then the subsequent SR transmission (e.g. SR resource 2) could be blocked due to the running sr-ProhibitTimer, even though the SR is not transmitted/blocked by the LBT failure.

Observation 2: According to the current MAC specification, the sr-ProhibtTimer which starts immediately after the MAC indicates the SR to the PHY blocks the subsequent SR transmission attempt.

Figure 3: NR-U SR prohibit timer

To resolve the second issue as given in Observation 2, the sr-ProhibitTimer is stopped when the LBT of the SR transmission attempt (e.g. SR resource 1) does not succeed in PHY, so as to allow an immediate SR re-transmission attempt (e.g. SR resource 2 as illustrated in Figure 3). If the LBT of any SR transmission succeeds, the sr-ProhibitTimer can be running as specified in the current MAC specification.

Proposal 4: RAN2 is kindly requested to confirm that the sr-ProhibitTimer of NR-U is started immediately when the MAC indicates the SR transmission opportunity to the PHY, same as the current NR MAC specification.

Proposal 5: The PHY indicates the SR transmission status (i.e. LBT failure indication) due to the LBT failure to stop the sr-ProhibitTimer.

Proposal 6: When the sr-ProhibitTimer is stopped, the pending SR in MAC can trigger another SR transmission, same as the current NR MAC specification.

SR counter

Regarding the SR_COUNTER, we consider that if the MAC does not increment SR_COUNTER when the SR is dropped due to the channel access failure, the SR procedure could be extended for a long time, and the UE may not be able to fall back to the RACH procedure. To allow the UE fall back to the RACH procedure when several SR transmission attempts fail due to the LBT failure, the SR_COUNTER increases for each SR transmission attempt regardless of whether the SR transmission succeeds or not.

Proposal 7: The SR_COUNTER increases when the MAC indicates the SR transmission to the PHY, same as the current NR MAC specification.

Conclusions

Based on the analysis given above, we have the following Observations and Proposals：

Observation 1: According to the current MAC specification, the MAC indicates only one valid PUCCH resource for SR transmission for each SR transmission attempt.

Observation 2: According to the current MAC specification, the sr-ProhibtTimer which starts immediately after the MAC indicates the SR to the PHY blocks the subsequent SR transmission attempt.

Proposal 1: More than one SR resources can be indicated from the MAC to the PHY.

Proposal 2: The PHY selects one SR resource for transmission if the channel access succeeds.

Proposal 3: If Proposal 1 is agreed, RAN2 is kindly requested to discuss if more than one SR resources indicated from the MAC to the PHY are in different frequency domains or different time domains.

Proposal 4: RAN2 is kindly requested to confirm that the sr-ProhibitTimer of NR-U is started immediately when the MAC indicates the SR transmission opportunity to the PHY, same as the current NR MAC specification.

Proposal 5: The PHY indicates the SR transmission status (i.e. LBT failure indication) due to the LBT failure to stop the sr-ProhibitTimer.

Proposal 6: When the sr-ProhibitTimer is stopped, the pending SR in MAC can trigger another SR transmission, same as the current NR MAC specification.

Proposal 7: The SR_COUNTER increases when the MAC indicates the SR transmission to the PHY, same as the current NR MAC specification.

Reference

[1] 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”.