3GPP TSG-RAN WG2 Meeting #105                                                        R2-1900243

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Common design and QoS Handling for IAB

Agenda Item:	11.1.2

Document for:	Discussion and Decision

Introduction

In the last RAN2 meetings, some consensus about IAB bearer mapping and unified design had been reached and captured in TR [1].

The IAB architecture should support many-to-one and one-to-one bearer mappings in a common design since both mapping options provide benefits in different deployment and traffic scenarios.

This design should allow many-to-one and one-to-one bearer mappings to be used at the same time.

The design supports hop-by-hop ARQ. End-to-end ARQ is not excluded for one-to-one mapping.

The design addresses LCID-space and LCG-space limitations to support fine-granular QoS for a sufficiently large number of bearers.

In this paper, we continue to discuss detailed common design handling procedure and QoS parameter issues and give our preference.

Bearer mapping

In the last RAN2 meetings, both one-to-one mapping and many-to-one mapping between UE bearers and BH RLC-channels were agreed to be supported on bearer mapping in IAB node. The following table provides a comparison of these two mapping from several aspects:

Table 1 Comparison Table

|  | One-to-one mapping | Many-to-one mapping |
| --- | --- | --- |
| Number of BH RLC-channels | Extension required. | No impact. |
| Number of logical channels in BH link | Extension required. | No impact. |
| MAC data PDU format in BH link | Re-designed for extended LCH number Maximum number is difficult to be decided because of various IAB situations. | Reusing legacy MAC PDU format |
| BSR in BH link | Depends on whether the maximum value of LCG ID is extended or not. | No impact. |
| LCP in BH link | LCP in BH is per UE bearer level. | No impact |
| QoS guarantee and fairness | Finer granularity in BH link bearers. But algorithms are complex and reference information is local. | Base on global information to semi-statically adjust mapping relationship and parameters for QoS guarantee and fairness. |

The common architecture is that IAB interface includes following layers from high to low:

- Adaptation layer: maintain mapping between UE bearers and RLC channels. Many-to-one and one-to-one bearer mappings may have common mapping configuration scheme as much as possible;

- RLC layer: no impact;

- MAC layer: LCID and potential LCG ID space will be extended when one-to-one bearer mapping. Many-to-one and one-to-one bearer mappings will have different MAC configuration, e.g. MAC PDU format;

In our understanding, many-to-one and one-to-one bearer mappings can be configured based on each child IAB node because one child IAB node connected to a parent node has one MAC entity in upstream direction.

Proposal1: common architecture and configuration scheme except for different MAC configuration can be used for many-to-one and one-to-one bearer mappings.

QoS and mapping configuration

When support many-to-one mapping between UE DRBs and BH RLC channels, two solutions are possible for bearer mapping decisions and configuration.

Solution1: Centralized bearer mapping:

In this solution, the IAB donor decides all the bearer mappings. The IAB donor sends the bearer mappings and QoS related parameters to all intermediate IAB nodes. DL bearer mappings and DL QoS parameters (e.g. scheduling parameters etc.) can be sent via F1-like interface to DU part of parent IAB nodes and UL bearer mappings and UL QoS parameter (e.g. LCP parameters etc.) can be send via RRC signaling to MT part of child IAB nodes. Legacy procedures can be reused as much as possible.

When a new UE bearer is setup or released, the CU can add or modify the mapping between UE bearer and IAB RLC-channel and also update the related QoS parameters (e.g. at least the GFBR or PBR for non-GBR services should be taken into account) for the related IAB RLC-Channels.

There are two sub-branches:

- Alt1: Donor informs mapping between UE bearers and each IAB RLC-channels;

Figure 1 Centralized bearer mapping Alt1

- Alt2: Donor informs mapping between UE bearers and access IAB RLC-channels or mapping between child IAB RLC-channels and parent IAB RLC-channels.

Figure 2 Centralized bearer mapping Alt2

Alt1 is more flexible and uniform for all bearer mapping, which does not distinguish access IAB and other intermediate IAB. And if each packet carries information of UE bearer in its adaptation header, there is no problem to map this packet to the right IAB RLC-channel. The advantage of Alt2 is to map between RLC-channels directly and just saves processing overhead of header inspection in some UL cases.

Solution2: Distributed bearer mapping:

In this solution, the bearer mapping decisions are made by intermediate IAB nodes separately. IAB donor sends QoS information of UE DRBs and IAB RLC-channels to the related intermediate IAB nodes. And each intermediate IAB node decides the mapping based on these QoS information.

When a new UE bearer is setup or released, the donor needs to update the related QoS parameters (e.g. at least the GFBR or PBR for non-GBR services should be taken into account) for the related IAB RLC-channels.  Then the donor sends the updated QoS parameters to all the related IAB nodes.

Figure 3 Distributed bearer mapping

From the perspective of function, mapping configuration and QoS parameter management are all L3 function. IAB node is a L2 entity. Hence it is more reasonable that mapping and QoS management is controlled by the donor L3. Furthermore, legacy F1 procedure and RRC signaling can be reused as much as possible. Secondly, from point of signaling overhead, centralized bearer mapping will not introduce obvious signaling overhead increasing. Distributed bearer mapping is also based on the QoS parameter update. When a new UE is added or released, QoS parameter update signaling is also needed. Compared to distributed bearer mapping, centralized mapping just needs some mapping update information accompanied with the above QoS parameter update signaling. The overheads are similar.

Based on the above analysis, the centralized per-UE bearer mapping is more feasible, which is also helpful for the fast re-routing after BH RLF.

Proposal2: The IAB donor decides and informs the bearer mapping between UE DRBs and IAB RLC-channels to all related IAB nodes.

QoS and Delay budget handling

Based on the above sections, it is the IAB donor to be responsible for bearer mapping decision and QoS parameters management. In NR, QoS related parameters include the followings:

- UE AMBR: DL AMBR can be controlled in the donor and UL AMBR can be controlled in the access IAB;

- Allocation and Retention Priority: Configuration of ARP parameter to IAB nodes depends on whether IAB nodes need to perform bearer level handling or not in the case of resource limitations, e.g. admission control, congestion control, etc;

- 5QI: Including resource type (GBR or Non-GBR), priority level, packet delay budget, packet error rate, averaging window, maximum data burst volume. These parameters can be carried in F1* signaling and LCP parameters similar with legacy procedure;

- Reflective QoS Attribute: RQA is not needed in IAB nodes because it is just used in SDAP layer;

- GBR Information: can be carried in F1* signaling and LCP parameters similar with legacy procedure;

Among them, packet delay budget (PDB) is a parameter related to maximum tolerable transmission delay. The value of PDCP discard timer is usually set based on PDB minus the delay of wired network, which means that if the packet is scheduled longer than the expected PDB requirement, expired packets will be discarded. In an IAB network especially a multi-hop case, transmission delay is usually longer than legacy one-hop network. It is worth discussing whether discard timer information needs to be delivered to intermediate IAB nodes for some discarding decision. In our understanding, transferring rest delay budget hop-by-hop may have some gains in buffer management and resource efficiency improvement. But the cost is to carry time information in every packet, e.g. timestamp of arriving PDCP or the rest of delay budget etc. Timestamp needs to carry a precise absolute time in header of packet. The rest of delay budget can reduce header overhead but introduce complexity of calculating the rest value. These solutions will introduce big overhead and complexity compared to the limited benefits.

Observation: Delivering delay budget information will introduce big overhead and complexity compared with its benefits.

Conclusion

According to the analysis in the above sections, we have following observations and proposals:

Observation: Delivering delay budget information will introduce big overhead and complexity compared with its benefits.

Proposal1: common architecture and configuration scheme except for different MAC configuration can be used for many-to-one and one-to-one bearer mappings.

Proposal2: The IAB donor decides and informs the bearer mapping between UE DRBs and IAB RLC-channels to all related IAB nodes.

References

- TR38.874  Study on Integrated Access and Backhaul, v1.0.0