3GPP TSG-RAN WG2 Meeting #105                                                          	R2-1900272

Athens, Greece, Feb. 25th – Mar.1st, 2019 		Resubmission of R2-1816953

Source:	CATT

Title:	TP on Combination Solutions for Capability Signalling Optimization

Agenda Item:	11.5.3

Document for:	Discussion and Decision

Introduction

In [1], we primarily analyzed the existing solution’s pros and cons, and provided an optimization solution which combines the existing three solutions. In this contribution, we provide the corresponding TP.

Reference

[1] R2-1900271	Consideration on Combination Solutions for Capability Signalling Optimization	CATT

Text proposal

6.1.2	UE capability Identity combined with other means

Figure 6.1.2-1: Procedure of combination solution

UE capability ID based solution can work together with compression and segmentation solutions. If the UE reported capability ID is not known by network, the network may trigger the capability enquiry procedure and may provide the compression configuration to the UE at the same time. The UE will generate the whole UE capability information message and compress it according to the compression configuration. If the compressed message is larger than a threshold, e.g. 9000 bytes, it can be segmented into several messages and sent to network. The network will reassemble these segmented messages into one compressed message and then perform the decompression to get the original UE capability information message.